3GPP TSG-RAN WG4 Meeting #115                                                        R4-2505941

# Malta, MT, 19th – 23rd May, 2025

Source:	Apple

Title:	RRM performance requirements for beam management and positioning accuracy

Agenda Item:	7.21.6

Release:                Rel-19

Document for:	Discussion

Introduction

Rel-18 Study Item was approved on the Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface with the target to study the 3GPP framework for AI/ML for air-interface corresponding to each targeted use cases (i.e., CSI feedback enhancement, beam management, and positioning accuracy enhancements for different scenarios) regarding aspects such as performance, complexity, and potential specification impact [1] [2].

According to latest WID in [2], RAN4 is required to study the interoperability and testability aspects for each use case: Specifically, RAN4 is expected to study the requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable.

The current agreements on how to perform the RAN4 study on general issues for AI/ML, and issues related to interoperability/testing have been captured in the latest TR [3]

In this contribution, we provide our viewpoints on some of the interoperability and testability aspects for beam management.

Discussion

## 2.1 Framework for Defining Requirements in Tests for AI/ML

From RAN1 evaluation results there are many implementation factors that could influence the AI/ML model performance. Diverse company-specific assumptions and implementations regarding model structure and parameters often lead to significant performance variations. To ensure consistency across companies' results, incorporating a reference AI/ML decoder model becomes critical in defining performance benchmarks for the UE side encoder.

From a UE processing capability limitation perspective, RAN4 will need to find a measure to access and evaluate complexity. There should be some limitations and complexity considerations when RAN4 decides on the requirements.

As an example, from RAN1 performance evaluations for the single sided BM use case, the complexity of AI/ML models reported across the companies can vary drastically as it is shown in the figure below from TR:

Fig 1: Complexity of AI/ML models from evaluation results 
in terms of FLOPs and number of parameters for BM cases

Accordingly, the performance variations across companies as outlined in Section 6.3.2.1.1 from the TR can vary from 60% accuracy to 90% accuracy where most of the performance variation could be attributed to model complexity and other differences in simulation assumptions (training procedures, training data, etc)

One question that arises is: What should be the granularity for defining the parameters to specify the reference AI/ML model for BM use case? In addition to model parameters, other factors that could affect performance include the model output type and the associated types of training data. For example, significant performance variations across companies are reported for the BM use case in section 6.3.2.3 of the TR. These variations depend on the output type, such as labels (classifier network) or L1-RSRP (CNN or MLP). For training input types for BM we could also differentiate between wide beams (SSB based, wide Tx beams) vs narrow beams (CSI-RS based, narrow Tx beams) types of set B of beams.

Similarly to the list of parameters specified for the two-sided model in CSI use case, RAN4 should investigate the set of parameters and granularity for the other use cases. (BM and positioning)

For example, for the BM case we could start with the following parameters:

- General Complexity

  - Number of parameters

  - Memory Size

  - FLOPS

- Training output type (labels vs L1-RSRP, Classifier vs MLP)

- Training Input type (narrow beams (CSI-RS) vs wide beams (SSB) Tx beams)

We provide a table with the proposed parameters for specifying reference model for BM use case. (table 1)

Proposal 1: RAN4 should investigate the specification of reference AI/ML models for BM, the associated training procedures, and the training datasets to ensure alignment of simulation results. This will help pave the way for defining performance requirements for the BM use case, with careful consideration of constraints on model complexity. To initiate the discussion, we have provided a reference table as a starting point.

| Category | Parameter | Description/Examples |
| --- | --- | --- |
| Model architecture parameters | Model type | CNN(case 1), MLP (case 1_, RNN (Case 2), LSTM (Case 2) |
|  | Fixed point representation | Int8, int16, floating point etc |
| Model Training/Inference related parameters | Training procedure | FFS (e.g Initialization method, training duration, training completion criteria, etc) |
|  | Loss function | NMSE, Cross Entropy etc. |
|  | Training datasets | Channel model (CDL) number of set -B/ set-A beams, specify TX codebooks. Best Rx beam selection (best UE beam) SSB beams (setA and setB) And /or CSI RS beams |
|  | Impairments (can be used for inference too) | RSRP measurements with impairments: Impairment types affecting set B measurements and set A measurements. |
|  | Hyperparameters | Learning rate, batch size, regularization techniques and strength, optimization algorithm, max epochs,etc. |
|  | Cross-validation details | Dataset splits for training/testing/validation |
| Complexity | Model Parameter Size |  |
|  | Model Memory Size |  |
|  | FLOPS |  |
| Scalability | Supported set A | 64 beams |
|  | Supported set B pattern | 1/4, => 16 beams |
|  | Time Window configuration (BM case 2) |  |
|  | Number of future predicted instances (BM case 2) |  |
| Performance Metric | Beam ID, RSRP accuracy | Top-K beams, values of K? |

Table 1: Parameters for Specifying Reference Model for BM use cases

## 2.2  KPIs/Test Metrics for BM Use case

In TR 38.843 the following agreements are captured  for KPIs/Test metrices for BM use case.

Both spatial-domain DL beam prediction and temporal DL beam prediction are considered.

For metrics for beam management requirements/tests, the following test metrics are identified and could be considered

-	Option 1: RSRP accuracy

-	Option 2: Beam prediction accuracy

-	Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam"

-	Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

-	Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams"

-	Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB,

-	Related measurement accuracy can be considered to determine x

-	Option 4: combinations of above options

The overhead/latency reduction should be considered for the requirements as the side condition.

For option 1, RSRP prediction accuracy focus is about the RSRP difference for the same beam. RSRP prediction accuracy is more similar as legacy RSRP measurement accuracy definition:

Absolute L1-RSRP accuracy = measured L1-RSRP of beam index n - ideal absolute L1-RSRP of beam index n

RSRP prediction accuracy metric is defined in clause 6.3.1 in TR 38.843:

For AI/ML models, which provide L1-RSRP as the model output, the accuracy of predicted L1-RSRP is to be evaluated. Companies optionally report average (absolute value)/CDF of the predicted L1-RSRP difference, where the predicted L1-RSRP difference is defined as the difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the same beam.

From RAN4#110-bis the following agreement has been reached:

Companies are invited to provide inputs/proposals to refine the definition of RSRP accuracy. Hold on the discussions for concrete test metrics until RAN1 had conclusions on the schemes.

From the network operator's perspective, a number of optimal beams need to be known in order to optimize load balancing and make trade-offs in performance and complexity. Following this perspective and in accordance with legacy schemes, a minimum RSRP accuracy test should be applied to all reported/predicted beams. Additionally, in legacy systems, there is no testing to confirm if the reported beam powers correspond to the best genie beam indices. Ensuring that RSRP accuracy implies beam prediction accuracy is challenging and depends on absolute RSRP accuracy and the RSRP difference between the best beams.

Observation 1 : From the network operator's perspective, a number of optimal beams need to be known in order to optimize load balancing and make trade-offs in performance and complexity

Observation 2 : Ensuring that RSRP accuracy implies beam prediction accuracy is challenging and depends on absolute RSRP accuracy and the RSRP difference between the best beams

In line with the legacy approach, we aim to ensure RSRP accuracy for all predicted beams. When using neural networks for regression, we can rely on the Normalized Mean Squared Error (NMSE) criterion for training. It's crucial to ensure that the accuracy of weaker beams is not compromised by the accuracy of the strongest beams

Observation 3 : When using neural networks for regression, we rely on the Normalized Mean Squared Error (NMSE) criterion for training. It's crucial to ensure that the accuracy of weaker beams is not compromised by the accuracy of the strongest beams

Proposal 2: We propose that RSRP accuracy be defined as the difference between the predicted RSRP and the measured (genie) RSRP associated with the same Tx beam. Accuracy requirements should apply to all predicted beams that satisfy the predefined side conditions, including SNR.

When considering the necessity of testing for beam prediction accuracy, we propose to study the additional information and significance that this test will provide, especially in light of our definition of RSRP accuracy.

During RAN4#112bis the following agreements were reached:

Issue 2-3: RSRP Absolute accuracy

Agreement:

- If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – groundtruth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on groundtruth L1-RSRP].

Issue 2-5: Requirements for beam predictions

Agreement:

  - Requirements to be defined:

    - Measurement period for prediction (how long will set B be measured before making a prediction)

      - FFS on whether the legacy measurement period can be reused or not.

    - Prediction accuracy

      - FFS how/what to define depending on the chosen KPI (e.g. predicted RSRP, beam ID, Top-K/1 or Top 1/K, etc)

    - TCI state known/unknown

  - FFS on whether the other requirements are needed

During RAN4#114bis the following agreements were reached:

Issue 2-1: Metrics/KPIs for beam ID prediction

Agreement:

- Metrics/KPIs for beam ID prediction, at least for the case if only beam ID is reported:

The successful rate for the correct prediction,

  - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB,

  - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation

    - FFS on N values

  - FFS on x values

Note: if x=0 and N=1, it means Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

Issue 2-2: Absolute RSRP accuracy

Agreement:

- Absolute RSRP accuracy requirement applies to Top-1 of predicted beams at least

  - FFS whether to apply to other beams

Issue 2-3: Relative RSRP accuracy

Agreement:

- RAN4 to use the follow as baseline for further discussions, if the relative RSRP accuracy requirement is defined

  - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) -  (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value].

    - Reported L1-RSRP can be predicted,

    - FFS whether reported L1-RSRP can be measured RSRP

  - Relative RSRP accuracy requirements apply for Case 1, and FFS on whether it can apply for Case 2

Measurement Period for prediction

The legacy L1-RSRP measurement period and accuracy requirement is based on 1 shot measurement. The measurement period has resource sharing factor ‘P’, number of sample ‘M’ configured by measurement configuration, RX beam sweeping factor ‘N’ in FR2. The measurement period for Set-B would highly depend on the accuracy needed for prediction. We propose to use the legacy measurement period as a starting point and further evaluate if it is sufficient to achieve good prediction accuracy.

Observation 4 : The measurement period for Set-B would depend on the accuracy needed for prediction.

Proposal 3: Use legacy measurement period as a starting point for Set-B for evaluation if sufficient to achieve good prediction accuracy.

During RAN4#114bis the following agreements were reached:

Issue 2-4: Measurement period for inference

Agreement:

- For measurement delay component of the prediction delay:

  - Use “legacy” L1-RSRP measurement as the baseline for Case 1

    - FFS whether same values for M,N,P should be reused

  - FFS for case 2

For the FFS part on RSRP accuracy whether to apply to other beams we would like to list the following options:

Option 1: The index i may be any beam index in top-K beams based on ground-truth L1-RSRP

Option 2:  The index i may be any beam index in top-K beams based on predicted L1-RSRP

For option 1, if the predicted beams selected to meet accuracy requirement are based on ground-truth L1-RSRP, there may be a case to be noticeable as follows.

Case: The Top-K beam based on predicted L1-RSRP (e.g., beam X) is not any of the Top-K beams based on ground-truth L1-RSRP

To explore the probability of this case occurring in actual predictions, we have some simulations, and the results provided below.  We can see that due to mis-selection of top-K predicted beams the genie RSRP (at the predicted indices) is less than the Genie RSRP of the genie beam indices, indicating that there is certain probability that the Top-K predicted beam is not any of the Top-K beams based on ground-truth RSRP

Fig 1: Top-K predicted beam is not any of the top-K beams

Observation 5: There is a probability that the Top-K predicted beams may not align with the Top-K beams determined by ground-truth RSRP measurements

Under Option 1, the predicted L1-RSRP of beam X is not required to meet the accuracy requirement because beam X is not among the Top-K beams based on ground-truth L1-RSRP. However, the predicted L1-RSRP of beam X must still be reported if necessary. For DL Tx beam prediction, the UE is responsible for predicting and reporting the strongest beams along with their corresponding predicted RSRP values. This is because any reported beam could potentially be selected by the network as the Tx beam for transmission, meaning the predicted RSRP for each reported beam must maintain a certain level of accuracy. Consequently, if Option 1 is adopted, the accuracy of the predicted RSRP cannot be guaranteed in cases where the Top-K beams based on predicted L1-RSRP do not match the Top-K beams based on ground-truth L1-RSRP.

Observation 6 : For RSRP absolute accuracy, if the index i can represent any beam within the Top-K beams based on ground-truth L1-RSRP, the RSRP accuracy for beams that are part of the Top-K predicted beams but not included in the Top-K beams based on ground-truth RSRP cannot be guaranteed.

Based on the above analysis, we propose Option 2, where the index i refers to any beam within the Top-K beams based on predicted L1-RSRP. This definition ensures that the accuracy of all predicted RSRP values for the reported beams is maintained

Proposal 4: The absolute RSRP accuracy for AI/ML-based beam prediction is defined as the difference between the predicted L1-RSRP of beam index i and the ground-truth L1-RSRP of beam index i. Here, the index i can represent any beam within the Top-K beams based on predicted L1-RSRP.

RAN1 has already agreed that the beam index must be included as a mandatory element in BM reporting. From a key performance indicators (KPIs) standpoint, the accuracy of predicting the beam index is a critical factor. RAN1 is also actively defining the details for performance monitoring, where beam prediction accuracy has been established as a primary metric. Taking these aspects into account, beam prediction accuracy serves as one of the core metrics/KPIs for establishing BM requirements

Regarding the FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation, we will propose two methods:

Top-N Method A: Modified Option 3

Let the best top-K genie beams be bk  and the top-K predicted beam as bk≈k=1,⋯,K  .  Let the genie aided sorted beam index be bgk=sortRSRPbk.  Also denote the measured genie RSRPs for genie predicted and AI/ML predicted indices as RSRPbk and RSRPbgk respectively. Introduce the channel realization ’m’ (channel realization) and the corresponding RSRPbk  and  RSRPbgk.  Introduce the variable Cm=n=1Nwn1RSRPbgnn>RSRPbnn-Xn,  where wn  is a normalization weighting function, and 1(x) returns 1 if condition is satisfied, X is a specified margin in dB. The final metric is Beam_Acc=m=1MCm/N, for top-N beams with N<=K.

Proposal 5: Employ Modified Option 3 for beam prediction accuracy for top-N beams predicted beams (prediction order is not penalized)

Top-N Method B: Composite Beam Ranking Score (CBRS)

Define the following quantities CBRSgenie=k=1KRelevancekgenielog2k+1   and CBRSpredicted=k=1KRelevancekpredictedlog2k+1.   The composite beam ranking score is defined as CBRS=BRSpredictedBRSgenie.

The following characteristics can be observed:

Composite: Blends signal strength (RSRP), ranking order, and normalization.

Relevance Scores (e.g., RSRP-based signal strength).

Logarithmic Discounting:  Discounts errors in higher ranks more severely (other weighting functions can be included)

Normalization: Compares predictions to the ideal ranking (IDCG), ensuring scores range between 0 and 1.

Example: CBRS= 1.0: Perfect alignment with ideal beam ranking.

Example: CBRS = 0.9: High quality, but degraded by suboptimal predictions

Normalization can take place in the following way: Relevancekgenie=RSRPbkgenie-MinkRSRPbkgenieMaxkRSRPbkgenie-MinkRSRPbkgenie

Relevance scores quantify the utility or importance of each beam index (e.g. RSRP) in a ranked list (e.g., ).

These scores directly determine the "gain" in the beam ranking score calculation, which aggregates the usefulness of beams based on their positions. (logarithmic discounting)

Higher relevance scores at higher positions (lower i) contribute more to the cumulative gain due to the logarithmic discounting factor.

The maximum possible score is achieved when beams are ordered by descending relevance. Relevance scores (RSRP) define the "ideal" ranking.

The final matric can be expressed as: CBRS=k=1KRSRPbkpredicted-MinkRSRPbkpredictedMaxkRSRPbkpredicted-MinkRSRPbkpredictedlog2k+1k=1KRSRPbkgenie-MinkRSRPbkgenieMaxkRSRPbkgenie-MinkRSRPbkgenielog2k+1

Proposal 6: Employ the composite beam ranking score (CBRS)  for beam prediction accuracy for top-N beams predicted beams (prediction order is penalized)

In Fig 2 and Fig 3 we provide an example with proposed Modified Option 3 metric that considers the overall beam predictions (N=5) and composite beam ranking score.

Fig 2: Modified Option 3 metric that considers the overall beam predictions (N=5) and their relative significance based on RSRP powers

Fig 3: Composite Beam Ranking Score metric that considers the overall beam predictions (N=5) and their relative significance based on RSRP powers

Known condition for TCI State

The known condition for TCI state is defined based on the period between TCI state switch and the last transmission of RS or RS QCLed with target TCI state. If the target TCI state is a predicted beam, and the RS was not transmitted to, measured and reported by UE, then we need to introduce a new definition for known condition. We also need to differentiate between UE side prediction and NW side prediction. For the case when target TCI state is a RS based on beam prediction -

UE side model: The UE predicts and reports the top K beams based on measurements on set B. Since the UE is predicting the beam(s) reported, it is known to the UE if a TCI state is received within a certain period. The known condition can be determined based on last transmission of set-B.

NW side model: The UE reports the measurements for Set B and target TCI state is based on NW side prediction, the TCI state is unknown to the UE. In case the RS of target TCI state is transmitted to the UE, measured and reported prior to the TCI state switch, then it is known to the UE.

Figure 3: Known condition for UE side prediction

Observation 7 : The legacy definition for known TCI state cannot be used when target TCI state is based on predicted beam and not transmitted to and measured & reported by UE.

Observation 8 : RAN4 needs to discuss known TCI for UE side and NW side prediction.

Proposal 7: For UE side model known TCI state is determined based on Set B transmission and UE report of top-K.

Proposal 8: For NW side model - 
(1) TCI state is unknown if RS of target beam is not transmitted to UE prior to TCI state switch
(2) If RS of target beam is transmitted to the UE and reported, legacy definition of known TCI state is applicable

For UE side Rx beam assumption, there are also different assumptions:

Alt 1: UE always uses the best Rx beam to measure set B beams and set A beams and the corresponding L1-RSRP

This imply beam sweeping in the data collaboration procedure at the UE side for set A and set B measurement.

In training data collection for NW side model based on MDT framework, the UE report the gNB beam with corresponding L1-RSRP with best UE Rx beam.

During inference, UE report the L1-RSRP and corresponding NW beam index(es) (CRI/SSBRI) assuming the best UE Rx beam.

How best UE Rx beam is determined requires advanced UE side Rx beam management implementation. For example,

UE record the best Rx beam in global coordinates during training,

During inference, based on sensor information and UE orientation, to figure out the best Rx beam to use for the set A.

During performance monitoring, best Rx beam for set A and set B respectively is used.

Alt 2: UE always use the best Rx beam for set B measurement, and UE used the best Rx beam of set B for set A measurement during training and inferencing.

This imply that UE beam sweeping for set B measurement during training and inference.

In addition, using data collection for training procedure, the RS configuration in gNB needs to indicate the root RS of set A configuration with respect to set B RS.

There can be proprietary information disclosures concern at the NW vendor. There is no agreement in RAN1 (no related proposals either in this case).

In training data collection for NW side model based on MDT framework, the UE report the gNB beam with corresponding L1-RSRP with best UE Rx beam for set B, and set A L1-RSRP with best UE Rx beam QCLed to set B beam.

During inference, UE report the set B L1-RSRP and corresponding NW beam index (CRI/SSBRI, can be more than 4) assuming the best UE Rx beam for set B.

The same QCL type D root RS to set B should be signaled by the NW, as the training phase.

Proposal 9: NW side data collection, QCL type D is signaled between set A and set B RS.

Proposal 10: For NW side model, TCI state is known if QCL source is signaled in both data collection and inference (to ensure UE Rx consistency), it is known. Otherwise, it is unknown.

Proposal 11: (for Alt 1):  Define UE capability for Alt 1 of UE implementation. When UE does not support this capability, the TCI state is unknown. Unknown TCI state switching delay is applied.

Proposal 12: (for Alt 2): When gNB signal the QCL relationship between set A and set B, in training RS configuration, inference RS configuration and performance monitoring RS configuration (example of Alt 2 NW beam implementation of set A and set B),

The TCI state is known when the following is met:

Target TCI state is based on RS from set A beams with QCL relationship configured to a set B beam

The time between TCI state switch command and transmission of the QCLed RS from set B shall not exceed X ms

X = 1280 ms (legacy)

UE shall send measurement report for the QCLed RS from Set B between transmission and TCI state switch

TCI state and SSB of TCI state remain detectable during TCI state switching period

Otherwise, it is unknown.

## 2.3  Datasets for training/testing

For training the AI/ML models for beam management we need to agree on what the source of the training data is.  In RAN4#111 the following agreement was reached:

Issue 2-6:	Datasets for training/testing

- Proposals

  - Option 1: Training Data set to be specified in RAN4(directly or through some algorithm )

  - Option 2: Training data set to be left to implementation (companies can generate it based on knowledge of the test environment)

  - Option 3: others

- Recommended WF

  - To be discussed

Agreement:

- Vendors may take into account the test environment/conditions defined by RAN4 when training the UE

  - sufficient test environment/conditions should be defined to enable vendors to create the data needed for training

- FFS on proposals to augment training data to avoid overfitting of UE models to the test environment

- RAN4 to strive to make the test conditions similar to field deployment conditions

We need to agree if the goal of the training data is to train the model for testing purposes only or for training the model for real deployment as well.

If the training data will be used to train one model (used in testing and deployment) it would be hard to gurantee similar performance (testing environment and in the field) since Tx codebooks could be different from vendor to vendor.

Proposal 13: RAN4 should agree that the training dataset used in testing is strictly for testing purposes. In real-world applications, it will be the responsibility of UE implementations to determine the appropriate training dataset. This could involve using a vendor-specific dataset or creating a mixed dataset sourced from multiple vendors for training their models.

RAN4's goal of aligning test conditions with real-world deployment scenarios is closely related to ensuring generalization performance. Since field deployment conditions can vary significantly across different environments, the test conditions should account for this variability

Observation 9 : RAN4's goal of aligning test conditions with real-world deployment scenarios is closely related to ensuring generalization performance. Since field deployment conditions can vary significantly across different environments, the test conditions should account for this variability

From RAN4 #112 the following agreements were reached:

Issue 1-2: Channel modeling

Agreement:

- Use synthetic channels as the default assumption for conformance requirements/test.

  - RAN4 may conduct an analysis for each use case to determine the reliability of using synthetic channels for test data in evaluating models trained on real data.

    - The field data can be considered for the analysis.

Issue 1-3: Testing goals

Agreement:

- The testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved

  - FFS on whether and how many different test configurations/parameters are used for tests.

  - LCM would be tested

For better clarity and understanding, it is beneficial to distinguish between three specific sets of scenarios/conditions:

- All possible Deployment Scenarios/Conditions: These include the entirety of scenarios in which the AI functionality is intended for deployment. These conditions encapsulate the comprehensive real-world scenarios in which the AI system will operate, with the aim of meeting the specified RAN4 requirements within this context.  The requirements could in general be met with either AI/ML or legacy procedures.

- RAN4 Specified Test Conditions: These conditions encompass the specific scenarios for which RAN4 tests will be defined, ensuring that the requirements for these tests are met. These conditions should include the identified scenario as well as the other scenarios for generalization tests.

- Non-Tested Conditions that are part of possible Deployment scenarios/conditions: This category includes additional conditions that are part of the possible deployment conditions but are not explicitly addressed during RAN4 testing. These additional conditions could be met with either AI/ML or legacy operation through LCM procedures and post deployment  (Set 1 = Set 2 union Set 3)

For RAN4 specified test conditions, the testing data could be generated across a variety of different conditions/congiguations:

- Details of the channel fading characteristics (CDL, LOS/NLOS, etc)

- Different paterns of set B/set A

- Tx codebooks, Tx antenna architecture layout , antenna spacing

- SNR conditions

- Carrier Frequency

- Doppler conditions, measurement window configurations and window of predicted time instances (BM case 2)

We have the following proposals:

Proposal 14: To define the testing data for BM and provide sufficient test environments/conditions for vendors to generate the necessary training data, RAN4 should select a set of standardized test conditions. UEs will be tested across these identified conditions, and it will be up to the UE implementation to either switch between models or employ a supermodel capable of accommodating all these conditions during. Example of test conditions/configurations are:

- Details of the channel fading characteristics (CDL, LOS/NLOS, etc)

- Different paterns of set B/ set A

- Tx codebooks,Tx antenna architecture layout, antenna spacing

- SNR conditions

- Carrier Frequency

- Doppler conditions, measurement window configurations and window of predicted time instances (BM case 2)

Proposal 15: For all potential non-tested conditions that are part of possible deployment scenarios, including additional conditions not explicitly addressed during RAN4 testing, we propose to employ Post-Deployment Procedures. These procedures will augment RAN4 conformance testing to effectively manage and assess performance across all possible deployment conditions

## 2.4 Measurement error impact and the ground truth

During RAN4#112 and RAN4112bis meetings, RAN4 agreed to study the impact of measurement errors to AI/ML BM functionality on the performance of prediction of AI/ML BM. The agreements are copied below:

Agreement: RAN4 will study the impact of measurement error for the input to inference and dataset for training(set B measurement error) on prediction accuracy, companies should bring proposals on how to proceed with such a study. - impact on different prediction accuracy metrics should be taken into account - studies already performance by other groups(RAN1) could also be taken into account Note: intention of the study is just to understand the impact of measurement error on prediction accuracy.

Agreement: ​ RAN4 to perform study to assess the impact of measurement errors on prediction accuracy​ Perform LLS to evaluate the RSRP prediction accuracy and other KPIs​ - UE Model trained using ideal measurements​ Simulation assumptions in R4-2417211 can be used as starting point.

In the following section, we simulated the impact of measurement errors for an AI/ML model in inference operation. We employed spatial domain prediction with 32 predicted beams in Set A, and 8 measured beams in Set B which is subset of Set A. We trained a regression AI/ML MLP model to predict the RSRPs in set A from set B.

## 2.4.1 Methodology

We followed the procedure described in R4-2417211, where we trained an AI/ML model with a subset of samples from an ideal L1-RSRP dataset generated from the system level simulation (SLS) following the assumptions defined in TR 38.843 Table 6.3.1-1. We considered Approach A, and we used the LLS simulation assumptions defined in Table 6 of Annex I to generate the Baseband (BB) errors. We also implemented the RF errors, and we added both BB and RF errors to the subset of ideal L1-RSRP data to derive the dataset with measurement errors as follows

The results are compared to the performance metrics without considering measurement errors, for which we used another subset of samples from the ideal L1-RSRP dataset generated from the SLS and without measurement errors.

For RF error modeling, we consider following options:

•	No error: This is the baseline case where RF errors are not considered as input of the AI/ML model.

•	Gaussian (Independent for each TX beam): 95% of users are distributed within ±4.5 dB error margin with mean zero. The RF error is generated independently for each RSRP measurement of TX beam in a single inference instance so that different TX beams are measured with different RF errors.

•	Uniform distribution (Independent for each TX beam): All users are distributed within ±4.5 dB error margin with mean zero. The RF error is added independently for each RSRP measurement of TX beam in a single inference instance so that different TX beams are measured with different RF errors.

For evaluating the performance of the AI/ML prediction we employed the following metrics:

- Top-K/1 and Top-1/K prediction accuracy, where K = 1, 2, 3, 4, 5

- 90%-tile L1-RSRP difference between the maximum RSRP of the Top-1/Top-5 predicted beam(s) and the RSRP of the genie aided strongest beam

- 90%-tile L1-RSRP difference between the predicted L1-RSRP of the Top-1/Top-5 predicted beam(s) and the ideal L1-RSRP of the same beams

The set A and set B of angles are shown in Fig 4.

Figure 4: set A and set B of angles

## 2.4.2 Evaluation Results with no Errors

In this section we present results for inference with no measurement errors.  In Fig 5 we show the TopK/1 and Top1/K beam prediction with no measurement errors.

Figure 5:  TopK/1 and Top1/K beam prediction

In Fig 6 we present results for the metric:  90%-tile L1-RSRP difference between the maximum RSRP of the Top-1/Top-5 predicted beam(s) and the RSRP of the genie aided strongest beam.

Figure 6: 90%-tile L1-RSRP difference of genie and top predicted beams

- In Fig 7 we present results for the metric:  90%-tile L1-RSRP difference between the predicted L1-RSRP of the Top-1/Top-5 predicted beam(s) and the ideal L1-RSRP of the same beams

Figure 7: 90%-tile L1-RSRP difference of genie and top predicted beams with same beam index

## 2.4.3 Evaluation Results with RF Gaussian errors

In this section we present results with independent (per Tx beam) Gaussian RF errors.

In Fig 8 we show results for TopK/1 and Top1/K beam prediction accuracy and degradation with RF errors. We can see that with RF Gaussian errors the TopK/1 prediction accuracy will be reduced by 11.7% for Top1 and 8.3 % for Top2.

Figure 8:  TopK/1 and Top1/K beam prediction accuracy and degradation with RF errors

In Fig 9 we present results for the metric:  90%-tile L1-RSRP difference between the maximum RSRP of the Top-1/Top-5 predicted beam(s) and the RSRP of the genie aided strongest beam with RF Gaussian errors.

Figure 9:  90%-tile L1-RSRP difference of genie and top predicted beams with RF Gaussian Errors

In Fig 10 we present results for the metric:  90%-tile L1-RSRP difference between the predicted L1-RSRP of the Top-1/Top-5 predicted beam(s) and the ideal L1-RSRP of the same beams with RF Gaussian errors.

Figure 10:  90%-tile L1-RSRP difference of genie and top predicted beams with same beam index with RF errors (Degradation)

## 2.4.4 Evaluation Results with RF Gaussian errors + LLS errors

In  figure 11, we present the CDF of our simulated BB errors generated at SNR=-3dB. In figure 12, we present results for topK/1 accuracy in the presence of BB and RF errors (Gaussian). In figure 13, we present results for RSRP prediction accuracy with BB and RF (Gaussian) errors for both the absolute RSRP prediction and relative degradation due to measurement errors. In figure 14, we present results for topK/1 accuracy in the presence of BB and RF errors (Uniform). In figure 15, we present results for RSRP prediction accuracy with BB and RF (Uniform) errors for both the absolute RSRP prediction and relative degradation due to measurement errors.

Figure 11:  CDF of LLS BB errors

Figure 12:  TopK/1 and Top1/K beam prediction accuracy and degradation with RF plus LLS errors

Figure 13: 90%-tile L1-RSRP difference of genie and top predicted beams with same beam index with RF plus LLS errors

Figure 14: 90%-tile L1-RSRP difference of genie and top predicted beams with same beam index with RF plus LLS errors

Figure 15: 90%-tile L1-RSRP difference of genie and top predicted beams with same beam index with RF plus LLS errors

Proposal 16: RAN4 should consider adopting a realistic approach to model RF errors, as the prediction accuracy difference of the AI/ML model varies significantly between the worst-case scenario (independent RF errors) and the best-case scenario (same RF error for all Tx beams). To study the impact of measurement errors, the method of RF error modeling should be included in RAN4 discussions. Potential approaches include:

Using a truncated uniform distribution;

Using a truncated Gaussian distribution.

## 2.5 Simulation Results with Updated simulation assumptions for beam prediction

In this section we present simulation results with the updated simulation assumptions as discussed in R4-2505152. The following KPIs were agreed:

At least the following performance characteristics are to be provided

KPI 1: Beam prediction accuracy

- The successful rate for the correct prediction which is considered as maximum ground truth RSRP among top-K predicted beams is larger than the ground truth RSRP of the strongest genie-aided beam – x dB, where K=1,2,3,4,5, X= 1, 2, 3dB

KPI 2: Absolute RSRP accuracy

- 90%-tile L1-RSRP difference between the predicted L1-RSRP of the Top-1/ Top-3/ Top-5 predicted beam(s) and the ground truth L1-RSRP of the same beams

Note 1: the ‘ground truth’ underlined refers to:

- Ideal value without consideration of measurement error (for Case 1 and Case 2a defined in clause 6)

- Measurement result with consideration of measurement error (for Case 2b and Case 3 defined in clause 6)

Note 2:

90%-tile L1-RSRP difference = max (abs (95%-tile L1-RSRP), abs (5%-tile L1-RSRP))

We also agreed on the following simulation procedures:  We provide the following procedures for companies to perform simulations to evaluate the impact of measurement error in BM prediction Sceanario-1: spatial domain prediction with 32 beams in Set A, Set B is subset of Set A and contains 8 beams for measurement.

The following cases are treated as the candidate Options:

- Case 1: No error will be considered in training dataset, model input during inference and ground-truth.

- Case 2:

  - Case 2a No error will be considered in training dataset and ground-truth. Error will be considered in model input during inference

  - Case 2b No error will be considered in training dataset and ground truth for training. Error will be considered in model input during inference and ground-truth for inference

- Case 3: Error will be considered in training dataset, model input during inference and ground-truth.

For Case 1, the simulation procedure includes:

- Companies to generate ideal L1-RSRP dataset from the SLS assumption defined in Table 6 and TR 38.843 Table 6.3.1-1.

- Use subset samples of the ideal dataset generated from the 1st step for training models.

- Filter UEs in the other subset samples of the ideal dataset (none overlap samples with the subset dataset in 2nd step) which SNR> -3dB (use this SNR range as the starting point) to derive the ideal dataset for inference

Note: UE distribution is 100% outdoor

- Use the ideal dataset for inference and to evaluate the performance metric assuming that the ground truth is without the measurement error

## 2.5.1 New Simulation Results

We provide updated simulation results in the following tables:

Table 1:  KPI-1 Case 1

Table 2:  KPI-2 Case 1

Table 3:  KPI-1 Case 2a

Table 4:  KPI-2 Case 2a

Table 5:  KPI-1 Case 2b

Table 6:  KPI-2 Case 2b

Table 7:  KPI-1 Case 3

Table 8:  KPI-2 Case 3

Let Xgenie  denote the ground truth value ("genie" X) and Xmeasured  denote the measured value of X. Let Δ=Xgenie-Xmeasured denote the measurement error (requirement accuracy).  Xpred=fAI/MLXsubsetmeasured the AI/ML predicted value of X using a subset of the measured X.  The prediction error of the model under clean inputs ϵpred=Xgenie-Xpred(clean). The input measurement error: ϵinput=Xgeniesubset-Xmeasuredsubset

Observation 10: Assuming the ML model is Lipschitz continuous with constant L, then the total error between the predicted X and the genie X is bounded by: Xgenie-Xpred≤ϵpred+L⋅ϵinput. This represents the total inference-time error between Xpred and Xgenie accounting for both: the model's prediction error when trained on clean inputs ϵpred  and the additional error due to noisy inputs at inference L ϵinput

For Case 2b when we compare the difference between the predicted values and the genie values containing measurements errors, we provide the following analysis: We want to bound the difference Xpred-Xmeasured. We can write: Xpred-Xmeasured≤Xgenie-Xmeasured+Xpred-Xgenie. That is, Xpred-Xmeasured≤ϵpred+L⋅ϵinput+∆. Where Δ=Xgenie-Xmeasured  is the full measurement error across all dimensions of X,  and ϵinput=Xgeniesubset-Xmeasuredsubset measurement error in just the subset of X used as input to the model

Observation 11: Assuming the ML model is Lipschitz continuous with constant L, then the total error between the predicted X and the measured X is bounded by: Xpred-Xmeasured≤ϵpred+L⋅ϵinput+∆. Where Δ=Xgenie-Xmeasured  is the full measurement error across all dimensions of X,  and ϵinput=Xtruesubset-Xmeasuredsubset measurement error in just the subset of X used as input to the model

Comparing predicted RSRPs to their corresponding measured values using an absolute accuracy metric mostly reflects relative prediction error. However, this relative error can exceed the acceptable RSRP margin defined in legacy systems. Therefore, while the absolute accuracy requirement for AI/ML predictions may appear relaxed relative to legacy measurements, the resulting performance gap underscores the need to either enhance input measurement quality or enforce stricter prediction accuracy.

Observation 12: To avoid performance degradation, the absolute RSRP accuracy of AI/ML-based beam management (BM) predictions should meet the existing legacy requirements.

Proposal 17: To ensure that AI/ML-predicted RSRP values meet legacy performance expectations, one of the following should be pursued:

- Improve the accuracy of the measured RSRP values used as input to the AI/ML model, so that the resulting predictions satisfy the absolute RSRP accuracy requirement. (by considering side conditions on SNR, or restricting model’s applicability to inner cell UEs)

- Instead of relying on AI/ML to directly predict RSRP values, the AI/ML model is used solely as a top-beam classifier that predicts the optimal beam indices. The network then transmits the corresponding beams, and the UE performs actual RSRP measurements on these beams. In this approach, only the beam index classification accuracy needs to be defined as a performance requirement

- Alternatively, tighten the acceptable prediction accuracy requirement for AI/ML-based RSRP prediction so that it aligns more closely with the legacy RSRP margin.

- Investigate feasibility of Noise aware training: training the model with noisy inputs and genie outputs (or close to genie, high SNR).  Clean-input-trained models suffer from distribution shift at inference since the model never saw noisy data and may amplify input errors. Noisy-input-trained models internalize the mapping from corrupted inputs to correct outputs.

## 2.6 Generalization Issues/Associated ID

For the UE-side beam prediction models (BM-Case 1 and 2), an AI/ML-based prediction model is used to identify the optimal beam from the base station, leveraging a codebook that was included in the training data for model development.

However, if the codebook used by the base station differs from the one (or ones) used during training, the model may not be effective in predicting the best beam.

To address this challenge, RAN1 has proposed a solution involving the introduction of an associated ID approach, as outlined in the following agreement:

With the associated ID approach, the UE must store or download multiple models when codebooks vary across different vendors or environments.

This requirement could expose proprietary algorithms used by network vendors for beam management, as the indication of codebook switches may inadvertently reveal such vendor-specific details

To avoid the extra complexity of managing associated IDs across infra-vendors, a NW can equalize the differences between its own proprietary codebook and a RAN4 specified codebook for training the model. The network employs an AI/ML-based RSRP Equalizer to align UE-based predictions with those that account for the network's specific codebook (CB) implementations. This approach utilizes a one-sided AI/ML model implemented at the UE, which sends additional RSRP measurements to the network. The network then uses these measurements/predictions to equalize and adjust the UE's predictions, ensuring compatibility with its own CB design.

Proposal 18: To avoid the extra complexity of managing associated IDs across infra-vendors, develop a framework where the NW can equalize the differences between its own proprietary codebook and a RAN4 specified codebook for training the model. The network could employ a proprietary RSRP Equalizer to align UE-based predictions with those that account for the network's specific codebook (CB) implementations.

2.8 Generalization issues for BM

For AI/ML based BM solution, generalization poses one of the main challenges for RAN4 testing. Generalization issue includes the following main aspects:

Changing radio conditions

If an AI/ML functionality/model has been trained primarily on a dataset representing specific radio conditions, it may encounter diminished performance when faced with varying channel conditions in real-world environments. For example different propagation environments characterized by different angular/delay spread (LOS/NLOS) can alter the spatial mapping from set B to set A of beams learnt by AI/ML. Similarly the spatial mapping between set B and set A can be impacted by the carrier frequency/band of the measurements.

Changing configurations/parameters settings (conditions and additional conditions)

- The performance of different AI/ML applications is greatly influenced by the level of generalization achieved  which depends on the configuration and parameter settings utilized for training dataset creation. For instance, in the context of beam management in AI/ML, configurations (conditions and additional conditions) need to consider various beam sets and codebooks, along with variations in the number of wide and narrow beams (Fig 9), different beam widths, Tx angles, a diverse grid of set B beam configurations,  and measurements across multiple frequencies or bands.

Fig. 9:  Various beam sets and codebooks across different cells

Observation 13 : To guarantee that the UE operates within acceptable margins, it's essential to subject it to various radio conditions and additional conditions for testing and generalization validation in RAN4

For the generalization verification aspects, the following contents are agreed to be captured in TR 38.843 [1]:

The goals of generalization test are to verify whether the minimum level of performance of AI/ML functionality/model can be achieved/maintain under the identified scenarios and/or configurations, while the performance won’t be significantly degraded in other scenarios and/or configurations. The following aspects should be considered for generalization/scalability related testing: - details about the scenarios and/or configurations for test and the corresponding AI/ML models/functionality - what the minimum level performance for each identified scenario and/or configuration is - what the significant degradation for other scenarios and/or configurations is It should also be considered that generalization and/or scalability related requirements for different scenarios/ configurations can be implicitly handled in the test case definition. As for the handling of generalization tests, the following option is considered as baseline: Signalling based LCM procedures and performance monitoring are considered in dedicated test cases and are excluded in tests verifying generalization. RAN4 may define multiple tests with different conditions. In each of the test, TE configures the same specified UE configuration, and therefore the same specified UE configuration is tested under different conditions to verify its generalizability. (environment differs in each test but not changing dynamically during the test) - Specified UE configuration includes functionality and/or model ID if defined

The discussion concerning "details about the scenarios and/or configurations for testing and the corresponding AI/ML models/functionality" closely aligns with the RAN1 agreement regarding AI/ML functionality and models. These are proposed for the identification of AI/ML functionality/model, as outlined in the following excerpt from the agreed RAN1 section of TR 38.843:

<RAN1 Agreement, captured from TR38.843 v1.3.0> For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models, functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG. ... For AI/ML model identification and model-ID-based LCM of UE-side models and/or UE-part of two-sided models, model-ID-based LCM operates based on identified models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature/FG and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side.

A model associated with configuration/condition associated with UE capability and additional conditions is shown in Fig 10.

Fig. 10:  A model associated with configurations/conditions associated with UE capability and additional condition

Our view on the "details about the scenarios and/or configurations for testing and the corresponding AI/ML models/functionality" is shown in Fig 11.  For the AI/ML functionality the UE capability report involves grouping of AI/ML models into buckets corresponding to non-overlapping sets of supporting configurations/conditions (conditions associated with Model A, Model B, etc). These conditions are associated with the UE capability for functionality. For each of these generic model there are sub-models that can be associated with additional conditions for model identification. There are two types of additional conditions. The first type is the additional conditions that can be specified (and they can be signaled as assistance information to the UE to select the proper model). These NW side additional conditions should be aligned between training and inference for the UE sided model for performance improvement. According to this information, UE can select the most suitable model to improve performance and pass the test.

Fig. 11:  Capability report and additional conditions

RAN4 requirements should define tests for the AI/ML model such that the testing data align with the training data. That is, some of the conditions and additional conditions for functionalities and model identification, such as the beam pattern in Set B, the number of beams in setA/B and their associated mapping (wide SSB, versus CSI-RS narrow beams), time window configurations for BM case2, etc should align between training and RAN4 test. In order for the UE to select the appropriate model, some indication should also be part of the test definition. This assistance information will be part of the additional conditons that could be shared from the NW (NW side additional condition). Other more proprietary information (3dB beam width, antenna spacing, Tx beam angles) will be part of the additional conditions that cannot be shared (not part of UE capability) and could be considered to be  part of generalization tests

As mentioned, UE sided model would require some information on the configuration of set B codebook pattern for inference. This could be indicated as additional assistance information. Based on RAN1 discussion, there is no consensus to support the assistance information from NW on NW-side beam shape information. Simulation results confirm that the consistency / association of Set B beams and Set A beams across training and inference is “beneficial” from performance perspective from :

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, consistency / association of Set B beams and Set A beams across training and inference is beneficial from performance perspective. Note: Whether specification impact is needed is a separate discussion.

Thus, concerning the testing implications, ensuring the consistency or association of Set B beams and Set A beams throughout training and inference should be guranteed in the RAN4 core requirements. This consistency serves as additional assistance information for inference at the UE from the network side.

Proposal 19:  In the RAN4 core requirement, it is mandated that the consistency or association between Set B beams and Set A beams during both training and inference must be guaranteed. This serves as additional assistance information from the network side for testing UE-sided models during inference and monitoring

From the RAN1 agreement captured above, it can be seen that a UE will report the supported configurations as part of capability signaling for a certain model/functionality. The other scenarios and/or configurations for generalization can be decided based on the supported configuration reported by UE.

For defining tests we can identify a set of reference conditions/configurations based on the capability signaling report from the UE. Example of reference radio conditions that UE could support can be an AWGN propagation condition or a particular CDL channel (link level). For example UE capability could indicate that the output type of the AI/ML model is label based (classifier), therefore the model should be tested under option 2 for BM KPI test metric.

For the other scenarions/configurations for defining generalization tests, we capture the following paragraph from the TR: “. In each of the test, TE configures the same specified UE configuration, and therefore the same specified UE configuration is tested under different conditions to verify its generalizability. (environment differs in each test but not changing dynamically during the test)

Specified UE configuration includes functionality and/or model ID if defined ”

For functionality based LCM, functionality is enabled by configurations supported by conditions indicated by UE capability. Given that configuration, and as part of generalization test we can vary some condition/additional conditions.  For example, tests can be defined where the patterns of beam set A and set B vary,  or their associated mapping changes (e.g. SSB to CSI-RS wide to narrow beam mapping), or different antenna configirations/layout like 4x16 antenna versous 8x8 antenna layout, different antenna spacings (e.g spacing = 0.8 lamda).   Another example of additional condition could be the  frequency/band of the sweeping beams, number of history measurements time measurements for BM case 2 (Set B pattern in temporal domain), system level propagation channels (Uma, Umi), different UE speeds, different beam shapes, etc Some of these conditions can be signalled explicitly or implicitly to the UE during the test and the UE can select the appropriate model. This is shown in Fig 12 where UE2 switches models to improve generalization performance based on assistance signaling of additional conditions.

Fig. 12:  Generalization performance improvement with model switch based on assistance information or autonomous model switch

For generalization testing, there is a type of addtional conditions which are the sensitive proprietary information that NW vendors may not be willing to share. Examples of these conditions are:

- NW side beam shape information

  - 3dB bandwidth, physical antenna spacing, antenna array size, Tx beam angles, boressight directions

For defining generalization tests, the identified scenarios and configurations shall be associated with the UE capability of an AI/ML-enabled Feature FG. For example, we could define a generalization test associated with model B as it is shown in Fig 13. Model B is associated with some specific conditions/configurations, for functionality identification as indicated by UE capability. Under that configuration, UE could have other models associated with additional conditions (these conditions cannot be specified). These additional conditions will be the different scenations that will be part of the generalization tests.

Some of the additional conditions can be shared during the test in the form of assistance information to improve the generalization ability of the UE.

For the other additional conditions that the NW cannot share due to proprietary information, we can employ a virtual ID in the training dataset that indicates the additional conditions that the model was trained with. By signaling this ID, the UE can select the model that was trained with this dataset.

Observation 14 : For BM use case the identified scenarios and configurations can be initially understood as those reported by UE through capability signaling as part of functionality identification.

Observation 15 : The additional conditions for the AI/ML model training (which do not constitute part of UE capability) for the AI/ML-enabled feature/FG can serve as the different scenarios/configuration for defining generalization

Proposal 20: RAN4 should define identified scenarios/configurations associated with the UE capability report of an AI/ML-enabled Feature FG.  For defining generalization tests, the additional conditions can serve as the other identified scenarios/configurations for the BM use case

Proposal 21: RAN4 should investigate the feasibility of providing assistance information for the additional conditions to aid generalization and consistency across training and testing when defining requirements. Other additional conditions that are not part of UE capability can be used to define generalization tests

Proposal 22: For additional conditions that cannot be shared due to proprietary concerns, RAN4 can explore the feasibility of using a virtual ID to indicate the specific conditions under which a model was trained. This approach would assist in the proper selection of UE models to support generalization. Additionally, RAN4 should identify which additional conditions should be exclusively reserved for generalization tests.

Other or additional parameters, such as propagation conditions/channel models, UE-side configurations, or other external conditions, cannot be defined by the feature functionality/configuration or included in the Associated ID.

Proposal 23: RAN4 should analyze how changes in test conditions, such as propagation conditions, affect the performance of AI/ML-enabled features. This analysis will help define the appropriate number of requirements and tests needed for each AI/ML-enabled feature and its corresponding ID combination.

2.9 Consistency between Training and Inference

In RAN4#110 bis  multiple companies brought up the need to discuss the consistency between set A and set B, otherwise it is not expected that beam prediction would work. In RAN4#111 the following options were provided:

Issue 2-7:	Consistency

- Proposals

  - Option 1: consistency between set A and set B is reflected by below aspects:

    - Same NW antenna/beam configurations for set A and set B, and that its configurations don’t change during training and inference.

    - Same channel model for set A and set B, and that its configurations don’t change during training and inference. (Question: if consistency is valid for a non-static(time-varying) channel for set A and set B?)

  - Option 2: Consistency should be defined in a different way

  - Option 3: no need for any consistency definition

- Recommended WF

  - To be discussed

Proposal 24: RAN4 to consider option 1 as a baseline for achieving consistency between training and inference for both set A and set B for beam spatial and temporal prediction

As it is shown in the figure below, in a non-static condition the set B configuration could change (let’s say B’). In this case the consistency is violated since set B’ is a different configuration from set B that the model was trained with. Consistency should be considered only for static conditions and not-static conditions should test the UE ability to adapt and employ diverse AI/ML models to match the conditions.

Table 1. Examples of Static versus Non-static Conditions

Proposal 25: Consistency should be considered in RAN4 only for static conditions and not-static conditions should test the UE ability to adapt and employ diverse AI/ML models to match the conditions (generalization)

The existence of generalization cases highlights the necessity of tackling performance issues that arise when the assumptions made during training, such as using a machine learning model trained with a specific set of network/user equipment (NW/UE)-side additional conditions, do not align with the assumptions made during inference. This misalignment can lead to certain performance degradations.

As agreed in RAN#102, ensuring consistency between training and inference regarding any identified additional conditions on the network side for inference at user equipment (UE) is a key objective for the beam prediction use case within the scope of Release 19 work item.

As highlighted in TR 38.843 (Clause 4.2.3), additional conditions refer to any aspects that are assumed for the training of the model but are not a part of UE capability for the AI/ML-enabled feature/FG. Moreover, additional conditions of a model can be divided into two categories: NW-side additional conditions and UE-side additional conditions

The following options can be taken as potential approaches to improve generalization and maintain performance by ensuring consistency between training and inference, in accordance with clause 4.2.3 of TR 38.843:

·  Model identification to achieve alignment on the NW-side additional condition between NW-side and UE-side.

·  Model training at NW and transfer to UE, where the model has been trained under the additional condition.

·  Information and/or indication on NW-side additional conditions is provided to UE.

·  Consistency assisted by monitoring (by UE and/or NW, the performance of UE-side candidate models/functionalities to select a model/functionality)

The concept of additional conditions was introduced during the latter stages of the study item when the necessity for these conditions became apparent due to the presence of AI/ML training variables that may lack standardization. For example, in AI/ML-based beam management use cases, elements of network topology such as cell layout or antenna configuration, or internal variables of user equipment (UE) such as UE speed, can influence the selection of the AI/ML model to be utilized during AI/ML operations.

Nonetheless, while these variables may be essential for training AI/ML models and thus required as input for AI/ML model operation, they may not be standardized due to proprietary information concerns. Therefore, categorizing these variables as additional conditions can facilitate the AI/ML operation.

Among the methods explored for ensuring consistency between the network and UE for these additional conditions, the "Consistency assisted by monitoring" methodology appears to be the least optimal solution. It is expected to cause significant delays in selecting the optimal AI/ML model for the UE.

Consider a scenario where multiple sets of beam management models are developed for different user equipment (UE) speeds, and the network intends to utilize the monitoring procedure to select the optimal model. In this case, the network must activate multiple models at the UE one by one, each associated with different UE speeds, until it identifies an AI/ML model that yields the best performance. During the period when the network is activating non-optimal model(s) at the UE, UE performance suffers, resulting in a delay in selecting the most optimal model.

Observation 16:  Achieving consistency between training and inference by model monitoring could result in delays and increased complexity in model management for BM use case

If multiple models with varying generalization capabilities and requirements for network-side additional conditions are trained by different UE vendors, it would necessitate substantial standardization efforts.

Observation 17:  If multiple models with varying generalization capabilities and requirements for network-side additional conditions are trained by different UE vendors, it would necessitate substantial standardization efforts for BM use case

In order to ease the burden for testing models with different NW additional conditions, it would beneficial to train the UE-side model with mixed dataset from various gNB settings (additional conditions), thus reducing the number of AI/ML models (selected by NW-side additional conditions) required to guarantee generalization and maintain the system performance.  UE complexity increases if the UE needs to store and manage multiple AI/ML models.

Proposal 26:  In order to ease the burden for testing models with different NW additional conditions, it would beneficial to train the UE-side model with mixed dataset from various gNB settings, thus reducing the number of AI/ML models (selected by NW-side additional conditions) required to guarantee generalization and maintain the system performance for BM use case

Proposal 27: For achieving consistency between training and inference, if the model drifts due to misalignment of network-side additional conditions, the alignment could be achieved through an ID assigned by the network during training data collection. This ID indicates the association of the training data with the additional conditions implied to generate those data

If the UE doesn’t have the ID but the NW has the model ID, then the NW can transfer the model to UE and UE update its list of models:

Proposal 28: For achieving consistency between training and inference, if the UE lacks the ID but the network possesses the model ID, then the network can transfer the model to the UE, and the UE can update its list of models

If the UE has a model that partially supports the NW-sided additional conditions then the NW can trigger data collection for fine tuning of the UE-sided model:

Proposal 29: For achieving consistency between training and inference, if the UE has a model that partially supports the network-sided additional conditions, then the network can trigger data collection for fine-tuning the UE-sided model. Subsequently UE updates its list of model IDs.

If there is no model at the network or UE, then the network configures the UE to fallback to legacy mode.

In case there is no ID available, then NW can configure a model monitoring procedure with UE-side measurements to check if the consistency of the additional conditions can be achieved. If performance criteria are met the consistency is achieved and the model has been verified. Otherwise, fallback to legacy mode is configured.

Proposal 30: The UE updates its model list with a new model derived from either fine-tuning, model transfer from the NW, or monitoring to ensure the consistency of additional conditions. Then, the UE assigns an ID to the new model that supports the NW's additional conditions and shares this information with the NW. If some of the new conditions/configurations are standardized, the UE updates its capability report accordingly.

Proposal 31: If there is no ID available to associate training data with additional conditions, and monitoring procedure fails to guarantee the consistency of the model with the additional conditions then UE should fallback to legacy mode.

For model identification each model is characterized by conditions and additional conditions. An example is shown in Fig 12. The complexity of LCM model management increases since the number of candidate models is large. Deactivations, latency and delays can occur when model monitoring takes place or when an appropriate model is not found (in this case UE fallback to legacy)

Fig. 13:  Model Identification and NW-side additional conditions

The current schemes proposed to ensure consistency between training and inference for sustaining system performance is through assistance information or model identification which is only used for model selection.

Observation 18: Current proposals on assistance information for additional conditions and Model Identification only serve the purpose of selecting the appropriate AI/ML model. However, this approach may not be scalable due to considerations of UE implementation complexities and granualtity of conditions/additional conditions. Complexity can increase substantially, especially if condition granularity is fine.

We propose defining some form of assistance information signal as input to the AI/ML model, which will be incorporated into the model’s inference engine. We propose the employment of an AI/ML supermodel that supplements its core inputs with auxiliary information signals, which will be integrated into its inference engine. The form of auxiliary information signals should optimize the tradeoff between disclosing proprietary information and enhancing the generalizability of the AI/ML model. As a result, the number of required AI/ML models needed to be stored, managed, and tested will be drastically reduced. An example is shown in Fig 13.

Fig. 14:  AI/ML Super Model with auxiliary information signals

Proposal 32:  Investigate the feasibility of enhancing the generalizability of the AI/ML model and reducing the number of AI/ML models and the testing burden for the beam management case by supplementing the core AI/ML input signals with both network (NW) and  UE auxiliary information signals integral to its inference engine

Proposal 33:  Investigate the feasibility of training the models with a mixed dataset associated with both network (NW) and UE auxiliary information signals to further enhance the generalizability of the AI/ML model for the beam management case and reduce the number of generalization tests.

# Conclusion

In this section we summarize our discussion by listing the observations and proposals discussed in this contribution:

Observation 1 : From the network operator's perspective, a number of optimal beams need to be known in order to optimize load balancing and make trade-offs in performance and complexity

Observation 2 : Ensuring that RSRP accuracy implies beam prediction accuracy is challenging and depends on absolute RSRP accuracy and the RSRP difference between the best beams

Observation 3: When using neural networks for regression, we rely on the Normalized Mean Squared Error (NMSE) criterion for training. It's crucial to ensure that the accuracy of weaker beams is not compromised by the accuracy of the strongest beams

Observation 4: The measurement period for Set-B would depend on the accuracy needed for prediction.

Observation 5: There is a probability that the Top-K predicted beams may not align with the Top-K beams determined by ground-truth RSRP measurements

Observation 6 : For RSRP absolute accuracy, if the index i can represent any beam within the Top-K beams based on ground-truth L1-RSRP, the RSRP accuracy for beams that are part of the Top-K predicted beams but not included in the Top-K beams based on ground-truth RSRP cannot be guaranteed.

Observation 7 : The legacy definition for known TCI state cannot be used when target TCI state is based on predicted beam and not transmitted to and measured & reported by UE.

Observation 8 : RAN4 needs to discuss known TCI for UE side and NW side prediction.

Observation 9 : RAN4's goal of aligning test conditions with real-world deployment scenarios is closely related to ensuring generalization performance. Since field deployment conditions can vary significantly across different environments, the test conditions should account for this variability

Observation 10: Assuming the ML model is Lipschitz continuous with constant L, then the total error between the predicted X and the genie X is bounded by: Xgenie-Xpred≤ϵpred+L⋅ϵinput. This represents the total inference-time error between Xpred and Xgenie accounting for both: the model's prediction error when trained on clean inputs ϵpred  and the additional error due to noisy inputs at inference L ϵinput

For Case 2b when we compare the difference between the predicted values and the genie values containing measurements errors, we provide the following analysis: We want to bound the difference Xpred-Xmeasured. We can write: Xpred-Xmeasured≤Xgenie-Xmeasured+Xpred-Xgenie. That is, Xpred-Xmeasured≤ϵpred+L⋅ϵinput+∆. Where Δ=Xgenie-Xmeasured  is the full measurement error across all dimensions of X,  and ϵinput=Xgeniesubset-Xmeasuredsubset measurement error in just the subset of X used as input to the model

Observation 11: Assuming the ML model is Lipschitz continuous with constant L, then the total error between the predicted X and the measured X is bounded by: Xpred-Xmeasured≤ϵpred+L⋅ϵinput+∆. Where Δ=Xgenie-Xmeasured  is the full measurement error across all dimensions of X,  and ϵinput=Xtruesubset-Xmeasuredsubset measurement error in just the subset of X used as input to the model

Observation 12: To avoid performance degradation, the absolute RSRP accuracy of AI/ML-based beam management (BM) predictions should meet the existing legacy requirements

Observation 13 : To guarantee that the UE operates within acceptable margins, it's essential to subject it to various radio conditions and additional conditions for testing and generalization validation in RAN4

Observation 14 : For BM use case the identified scenarios and configurations can be initially understood as those reported by UE through capability signaling as part of functionality identification.

Observation 15 : The additional conditions for the AI/ML model training (which do not constitute part of UE capability) for the AI/ML-enabled feature/FG can serve as the different scenarios/configuration for defining generalization

Observation 16:  Achieving consistency between training and inference by model monitoring could result in delays and increased complexity in model management for BM use case

Observation 17:  If multiple models with varying generalization capabilities and requirements for network-side additional conditions are trained by different UE vendors, it would necessitate substantial standardization efforts for BM use case

Observation 18: Current proposals on assistance information for additional conditions and Model Identification only serve the purpose of selecting the appropriate AI/ML model. However, this approach may not be scalable due to considerations of UE implementation complexities and granualtity of conditions/additional conditions. Complexity can increase substantially, especially if condition granularity is fine.

Proposal 1: RAN4 should investigate the specification of reference AI/ML models for BM, the associated training procedures, and the training datasets to ensure alignment of simulation results. This will help pave the way for defining performance requirements for the BM use case, with careful consideration of constraints on model complexity. To initiate the discussion, we have provided a reference table as a starting point.

Proposal 2: We propose that RSRP accuracy be defined as the difference between the predicted RSRP and the measured (genie) RSRP associated with the same Tx beam. Accuracy requirements should apply to all predicted beams that satisfy the predefined side conditions, including SNR.

Proposal 3: Use legacy measurement period as a starting point for Set-B for evaluation if sufficient to achieve good prediction accuracy.

Proposal 4: The absolute RSRP accuracy for AI/ML-based beam prediction is defined as the difference between the predicted L1-RSRP of beam index i and the ground-truth L1-RSRP of beam index i. Here, the index i can represent any beam within the Top-K beams based on predicted L1-RSRP.

Proposal 5: Employ Modified Option 3 for beam prediction accuracy for top-N beams predicted beams (prediction order is not penalized)

Proposal 6: Employ the composite beam ranking score (CBRS) for beam prediction accuracy for top-N beams predicted beams (prediction order is penalized)

Proposal 7: For UE side model known TCI state is determined based on Set B transmission and UE report of top-K.

Proposal 8: For NW side model - 
(1) TCI state is unknown if RS of target beam is not transmitted to UE prior to TCI state switch
(2) If RS of target beam is transmitted to the UE and reported, legacy definition of known TCI state is applicable

Proposal 9: NW side data collection, QCL type D is signaled between set A and set B RS.

Proposal 10: For NW side model, TCI state is known if QCL source is signaled in both data collection and inference (to ensure UE Rx consistency), it is known. Otherwise, it is unknown.

Proposal 11: (for Alt 1):  Define UE capability for Alt 1 of UE implementation. When UE does not support this capability, the TCI state is unknown. Unknown TCI state switching delay is applied.

Proposal 12: (for Alt 2): When gNB signal the QCL relationship between set A and set B, in training RS configuration, inference RS configuration and performance monitoring RS configuration (example of Alt 2 NW beam implementation of set A and set B),

The TCI state is known when the following is met:

Target TCI state is based on RS from set A beams with QCL relationship configured to a set B beam

The time between TCI state switch command and transmission of the QCLed RS from set B shall not exceed X ms

X = 1280 ms (legacy)

UE shall send measurement report for the QCLed RS from Set B between transmission and TCI state switch

TCI state and SSB of TCI state remain detectable during TCI state switching period

Otherwise, it is unknown.

Proposal 13: RAN4 should agree that the training dataset used in testing is strictly for testing purposes. In real-world applications, it will be the responsibility of UE implementations to determine the appropriate training dataset. This could involve using a vendor-specific dataset or creating a mixed dataset sourced from multiple vendors for training their models.

Proposal 14: To define the testing data for BM and provide sufficient test environments/conditions for vendors to generate the necessary training data, RAN4 should select a set of standardized test conditions. UEs will be tested across these identified conditions, and it will be up to the UE implementation to either switch between models or employ a supermodel capable of accommodating all these conditions during. Example of test conditions/configurations are:

·       Details of the channel fading characteristics (CDL, LOS/NLOS, etc)

·       Different paterns of set B/ set A

·       Tx codebooks,Tx antenna architecture layout, antenna spacing

·       SNR conditions

·       Carrier Frequency

·       Doppler conditions, measurement window configurations and window of predicted time instances (BM case 2)

Proposal 15: For all potential non-tested conditions that are part of possible deployment scenarios, including additional conditions not explicitly addressed during RAN4 testing, we propose to employ Post-Deployment Procedures. These procedures will augment RAN4 conformance testing to effectively manage and assess performance across all possible deployment conditions

Proposal 16: RAN4 should consider adopting a realistic approach to model RF errors, as the prediction accuracy difference of the AI/ML model varies significantly between the worst-case scenario (independent RF errors) and the best-case scenario (same RF error for all Tx beams). To study the impact of measurement errors, the method of RF error modeling should be included in RAN4 discussions. Potential approaches include:

Using a truncated uniform distribution;

Using a truncated Gaussian distribution.

Proposal 17: To ensure that AI/ML-predicted RSRP values meet legacy performance expectations, one of the following should be pursued:

- Improve the accuracy of the measured RSRP values used as input to the AI/ML model, so that the resulting predictions satisfy the absolute RSRP accuracy requirement.

- Instead of relying on AI/ML to directly predict RSRP values, the AI/ML model is used solely as a top-beam classifier that predicts the optimal beam indices. The network then transmits the corresponding beams, and the UE performs actual RSRP measurements on these beams. In this approach, only the beam index classification accuracy needs to be defined as a performance requirement

- Alternatively, tighten the acceptable prediction accuracy requirement for AI/ML-based RSRP prediction so that it aligns more closely with the legacy RSRP margin.

- Noise aware training: training the model with noisy inputs and genie outputs (or close to genie, high SNR).  Clean-input-trained models suffer from distribution shift at inference since the model never saw noisy data and may amplify input errors. Noisy-input-trained models internalize the mapping from corrupted inputs to correct outputs.

Proposal 18: To avoid the extra complexity of managing associated IDs across infra-vendors, develop a framework where the NW can equalize the differences between its own proprietary codebook and a RAN4 specified codebook for training the model. The network could employ a proprietary RSRP Equalizer to align UE-based predictions with those that account for the network's specific codebook (CB) implementations.

Proposal 19:  In the RAN4 core requirement, it is mandated that the consistency or association between Set B beams and Set A beams during both training and inference must be guaranteed. This serves as additional assistance information from the network side for testing UE-sided models during inference and monitoring

Proposal 20: RAN4 should define identified scenarios/configurations associated with the UE capability report of an AI/ML-enabled Feature FG.  For defining generalization tests, the additional conditions can serve as the other identified scenarios/configurations for the BM use case

Proposal 21: RAN4 should investigate the feasibility of providing assistance information for the additional conditions to aid generalization and consistency across training and testing when defining requirements. Other additional conditions that are not part of UE capability can be used to define generalization tests

Proposal 22: For additional conditions that cannot be shared due to proprietary concerns, RAN4 can explore the feasibility of using a virtual ID to indicate the specific conditions under which a model was trained. This approach would assist in the proper selection of UE models to support generalization. Additionally, RAN4 should identify which additional conditions should be exclusively reserved for generalization tests.

Proposal 23: RAN4 should analyze how changes in test conditions, such as propagation conditions, affect the performance of AI/ML-enabled features. This analysis will help define the appropriate number of requirements and tests needed for each AI/ML-enabled feature and its corresponding ID combination.

Proposal 24: RAN4 to consider option 1 as a baseline for achieving consistency between training and inference for both set A and set B for beam spatial and temporal prediction

Proposal 25: Consistency should be considered in RAN4 only for static conditions and not-static conditions should test the UE ability to adapt and employ diverse AI/ML models to match the conditions (generalization)

Proposal 26:  In order to ease the burden for testing models with different NW additional conditions, it would beneficial to train the UE-side model with mixed dataset from various gNB settings, thus reducing the number of AI/ML models (selected by NW-side additional conditions) required to guarantee generalization and maintain the system performance for BM use case

Proposal 27: For achieving consistency between training and inference, if the model drifts due to misalignment of network-side additional conditions, the alignment could be achieved through an ID assigned by the network during training data collection. This ID indicates the association of the training data with the additional conditions implied to generate those data

Proposal 28: For achieving consistency between training and inference, if the UE lacks the ID but the network possesses the model ID, then the network can transfer the model to the UE, and the UE can update its list of models

Proposal 29: For achieving consistency between training and inference, if the UE has a model that partially supports the network-sided additional conditions, then the network can trigger data collection for fine-tuning the UE-sided model. Subsequently UE updates its list of model IDs.

Proposal 30: The UE updates its model list with a new model derived from either fine-tuning, model transfer from the NW, or monitoring to ensure the consistency of additional conditions. Then, the UE assigns an ID to the new model that supports the NW's additional conditions and shares this information with the NW. If some of the new conditions/configurations are standardized, the UE updates its capability report accordingly.

Proposal 31: If there is no ID available to associate training data with additional conditions, and monitoring procedure fails to guarantee the consistency of the model with the additional conditions then UE should fallback to legacy mode.

Proposal 32:  Investigate the feasibility of enhancing the generalizability of the AI/ML model and reducing the number of AI/ML models and the testing burden for the beam management case by supplementing the core AI/ML input signals with both network (NW) and  UE auxiliary information signals integral to its inference engine

Proposal 33:  Investigate the feasibility of training the models with a mixed dataset associated with both network (NW) and UE auxiliary information signals to further enhance the generalizability of the AI/ML model for the beam management case and reduce the number of generalization tests.

# References

[1] RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm, RAN#94e.

[2] RP-242399 - Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface,

[3] 3GPP TR 38.843, “Technical Specification Group Radio Access Network;Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface,

[4] TR 38.810, v16.7.0, “Study on test methods”.

[5] R4-2501726, “ Discussion on test setup for AI/ML based beam management”, R&S Feb 2025

[6] R4-2500167,  “FR2 test setup for AI/ML beam management evaluation”, Anritsu Feb 2025