3GPP TSG-RAN WG4 Meeting #115                                 R4-2505945

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.21.3

Source:	Apple

Title:	Discussion on CSI reporting requirement and testing framework for CSI prediction

Release:       Rel-19

Document for:	Discussion

# Introduction

Rel-19 Work Item (WI) was approved on the Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface (WID in [1]). The application of AI/ML techniques to NR air interface has been studied in FS_NR_AIML_Air.

This work item provides normative support for the general framework of AI/ML concerning air interfaces. It also enables the implementation of recommended use cases outlined in the previous study. Furthermore, several study objectives within this project aim to address outstanding issues identified during the study, with the goal of enhancing understanding in preparation for future normative effort.

The current agreements on how to perform the RAN4 study on general issues for AI/ML, and issues related to interoperability/testing have been captured in the latest TR [2]

In this contribution, we provide our viewpoints on some of the interoperability and testability aspects for AI/ML for NR air interface for CSI compression and prediction.

# Metric for RAN4 Requirements

In the previous meeting RAN4#112, the following agreements was reached for CSI prediction.

Agreement:

Use relative throughput as requirement metric for CSI prediction

- companies are invited to bring contribution on how the requirement could be defined:

  - what is the baseline that throughput would be compared to

using relative throughput as the test metric for inference, the baseline denominator for relative throughput still requires further clarification.

In the current PMI test case, RAN4 uses random PMI as the baseline to evaluate whether the UE-reported PMI is accurate enough. This approach is analogous to a scenario where no UE reporting occurs.

Issue 2-1: Channel model for CSI prediction tests

Agreement:

- For channel model for CSI prediction tests, TDL channel models will be used.

  - FFS on whether CDL channel model is needed for generalization test.

Issue 2-2: Reference throughput

Agreement:

- reference throughput is obtained based on random PMI with Rel-15 Type I single panel codebook

Issue 2-3: Testing conditions

Agreement:

- use static conditions as baseline.

  - FFS on whether to have non-static condition

Issue 2-5: Prediction delay requirements

Agreement:

- Prediction delay includes:

  - Measurement for prediction

    - FFS on whether the legacy measurement delay can be reused

  - Inference delay

  - Other components are FFS

The following issues/agreements were discussed in RAN4#114:

Issue 1-2: Reference model for CSI prediction

Agreement:

- Assume no reference AI model for CSI prediction as baseline

  - If it is difficult to align the results corresponding to test metric, RAN4 will discuss the reference model.

Issue 1-3: Training and test data consistency

Agreement:

- further discuss the following options

  - Option 1: Consistency between training data and test data should be ensured

    - RAN4 to study how to ensure consistency between training data(simulation) and test data (conformance test) and whether anything special should be done during the test

  - Option 2: Consistency will be implicit as test emulated channel will be very similar to the one used in RAN4 simulations

Issue 1-4: Generalization

Agreement:

- Postpone this discussion until the requirement and test definition is clear

During RAN4#112bis it was discussed whether the new requirements on AI/ML-based functionality should be worse or better than the requirements on the legacy CSI feedback:

Issue 1-4: AI/ML Performance Agreement: - For CSI prediction, AI/ML based performance requirement should not be worse than the legacy (non-AI/ML based) if a relevant equivalent legacy requirement/feature exists. - FFS which legacy requirement/feature are relevant - FFS whether AI/ML based performance should be better than the legacy - FFS on target for AI/ML based beam management

- On Rel-19 AIML-based CSI reporting

Since time domain prediction reporting mechanism is already introduced in Rel-18, RAN1 at 116bis meeting agreed to assume R18 eType II doppler codebook for AI/ML-based prediction:

Agreement For Rel-19 study on CSI prediction, companies are encouraged to evaluate throughput performance by comparing performance with non-AI/ML based CSI prediction. - R18 eType II doppler codebook is assumed for CSI report for both AI/ML and Non AI/ML prediction. - Companies to report the assumption for N4, which could be 1, 2, 4, 8. Note: Non-AI/ML based CSI prediction (Benchmark 2) can include statistical model based CSI prediction (e.g., based on Kalman filter, Wiener filter, Auto-regression). Conclusion For Rel-19 study on CSI prediction only, consider UE-sided model only. Agreement - For the AI/ML based CSI prediction, for CSI report, adopt following as a baseline for evaluation purpose - N4 value: 1, 4 - Others can be additionally submitted, e.g., 2, 8 - paramCombination-Doppler-r18: 6,7 or paramCombination -r16 = 5,6 (for Benchmark 1) - Others can be additionally submitted. - Note: The same selected parameter combination shall be applied for benchmarks. - CSI report periodicity: 5ms, 20ms (encouraged) - Others can be additionally submitted, e.g., 10ms

During RAN4#114bis the following agreements were captured:

Issue 1-1: Codebook to be used in the tests

Agreement:

- In Rel-19 only PMI requirements based on AI/ML CSI prediction will be defined

- use Rel-18 eType II Doppler codebook for AI/ML based CSI predictions

Issue 1-2: Requirement baseline

Agreement:

  - Requirement to be introduced for Dopler domain basis N4=1

      - N4>1 can be further discussed in the future if needed

Issue 1-4: Test metric

Requirements to be defined based on the ratio (same metric as for the current PMI reporting requirements in 38.101-4)

      - tue_AI/ML is 90 % of the maximum throughput obtained at SNRue_AI/ML  [a given SNR] using the precoders configured according to the UE reports,

      - trnd is the throughput measured at SNRue_AI/ML  [the same SNR] with random precoding.

        - trnd is obtained with Rel-15 type I single panel codebook

      - Note: SNRue_AI/ML is calculated in the same way as SNRue and SNRfollow1,follow2 in section 6.3 of 38.101-4

- Updated Simulation Results for AI/ML based CSI prediction

During RAN4#114bis the following agrrement was reached for the updated simulation results:

Way forward for next step simulation for CSI prediction

  - For step-1,

    - Interested companies are encouraged to provide the 2nd round of CSI prediction simulation results in RAN4#115 meeting.

  - For step-2,

    - Simulation assumptions for step-2 are updated in RAN4#114bis meeting.

    - Interested companies are also encouraged to provide the 1st round of CSI prediction simulation results (SGCS as KPI, SGCS between the eigenvectors of predicted PMI derived from the ideal DL channel estimation and ground-truth eigenvectors), in RAN4#115 meeting.

  - For step-3,

    - CSI prediction simulation (TPs as KPI, with practical DL channel estimation) is postponed to RAN4#116 meeting.

    - Note: Simulation assumptions for CSI prediction (TPs as KPI, with practical DL channel estimation) will be updated further according to the progress of RAN1&RAN4 in RAN4#115. Companies are encouraged to check the updated assumptions and input suggestions/comments in RAN4#115 meeting.

  - Companies are encouraged to provide the “sample and hold” results for above simulations for steps 1 and 2.

  - Companies can also provide the model complexity information (e.g., flops, backbone) of their AI/ML CSI prediction models.

The updated simulation assumptions for AI/ML based CSI prediction are shown below:

Table 1: Updated simulation assumptions for AIML based CSI prediction

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD, TDD, OFDM |
| Carrier frequency | 2GHz for FDD 4GHz for TDD |
| Subcarrier spacing | 15kHz for FDD, 30kHz for TDD |
| Bandwidth | 10MHz for FDD, 40MHz for TDD |
| Symbol | 1 symbol |
| Number of subbands | 13 subbands, 4RB as a bundle for FDD 14 subbands, 8RB as a bundle for TDD |
| gNB TX antennas | Option1:32(N1,N2)=(4,4) as defined in Rel-15 type1 Option2:16(N1,N2)=(4,2) as defined in Rel-18 CSI prediction requirements |
| UE RX antennas | 4 |
| Channel model | TDLA30 with XP medium correlation as baseline |
| Doppler spread | 20Hz, 50Hz and 100Hz |
| Channel estimation for CSI prediction | Step-1: - Ideal DL channel estimation, for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.). Step-2: - Ideal DL channel estimation, for the purpose of calibration and/or comparing intermediate results (e.g., to check SGCS after CSI feedback). Step-3: - Practical DL channel estimation, use a common CSI-RS configuration as in demodulation test (may need to define/choose a CSI-RS configuration) |
| CSI-RS configuration | FFS: following CSI-RS configuration: - For 32Tx Number of CSI-RS ports (X); 32. CDM Type; CDM4 (FD2, TD2). Density (ρ); 1. First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3); (2, 4, 6, 8). First OFDM symbol in the PRB used for CSI-RS (l0, l1); (5, 12). - For 16Tx Number of CSI-RS ports (X); 16. CDM Type; CDM4 (FD2, TD2). Density (ρ); 1. First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3); (2, 4, 6, 8). First OFDM symbol in the PRB used for CSI-RS (l0); (5). |
| Rank per UE | Step-1: Rank 1 and 2 to do quick check Step-2 and Step-3: Rank 2 as baseline |
| CSI feedback assumption | Step1: check the CSI prediction (outputs Raw channel) performance, do not need to take CSI feedback into account |
|  | Step-2 and Step-3: take CSI feedback into account - Use Rel-18 eTypeII-Doppler, N4 = 1， N4>1 can be further discussed in the future if needed |
| Test setup(timeline) | Only for step-3 FFS: following test setup (timeline): For FDD test setup with Periodic CSI-RS configuration - CSI-RS periodicity: 5 ms - CSI feedback periodicity: 5 ms - Scheduling delay (from CSI feedback to apply in scheduling): 4 slots Note: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation using a CSI-RS resource set in which the last CSI-RS resource is transmitted at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4) - Test setup for FDD test setup with Periodic CSI-RS configuration is captured in Table2 For TDD test setup with Periodic CSI-RS configuration - CSI-RS periodicity: 5 ms - CSI feedback periodicity: 5 ms - Scheduling delay (from CSI feedback to apply in scheduling): 6 slots Note : If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation using a CSI-RS resource set in which the last CSI-RS resource is transmitted at a downlink slot not later than slot#(n-6), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). - Test setup for TDD test setup with Periodic CSI-RS configuration is captured in Table3 FFS test setup for aperiodic CSI-RS configuration |
| SNR assumption | Only for step-3 [FFS, e.g., Option1: claimed by companies for step-2 first round simulation(TP as KPI) Option2: Example: [-6:2:10] dB Option3: other suggestions ] |
| CSI feedback overhead | PC7 as baseline |
| Baseline | FFS: - Random PMI with Rel-15 Type I single panel codebook if needed (e.g., for Step-2 or performance requirements) |
| KPI | Step-1: check SGCS before CSI feedback (between the raw channel matrix and ground-truth at UE) - Average of SGCS for intermediate results over all subbands per layer (e.g. for rank 2), - comparing the SVD of model output with SVD of the ground truth Raw channel Step-2: - check SGCS after CSI feedback Step-3: - check TPs after CSI feedback |
| Model input type | Raw channel matrix |
| Model out type | Raw channel matrix |
| Observation window | Observation window (number/distance): 5/5ms as baseline Optional for 10/5ms |
| Prediction window | Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance): 1/5ms/5ms |
| Training data and test data | Use following combinations: Training dataset (TDLA30); Test on (TDLA30). 20Hz Doppler spreads; 20Hz Doppler spreads. 50Hz Doppler spreads; 50Hz Doppler spreads. 100Hz Doppler spreads; 100Hz Doppler spreads. Mixed Doppler spreads (20Hz, 50Hz, 100Hz); 20Hz Doppler spreads. 50Hz Doppler spreads. 100Hz Doppler spreads. |

Simulation assumptions:

Training Data: We generated TDLA30 channels with Nrx=4, Ntx =16/32, cross-pol antennas, and we computed the eigenvectors rank 1 and rank 2. We generate CSI samples every 5ms, each CSI sample is a tensor of dimension: [52(tones) 2(Re,Im) 4 16/32]. We generated 20 sequences of CSI samples each one with different random seed, each sequence had 5000 time samples for a total of 100,000 samples. We used the first 16 sequences for training and the last 4 sequences for testing.

AI/ML model: We employed a spatiotemporal LSTM model, with a sliding window of observations =5. A fully connected layer was used after LSTM. The output size to the fully connected network was equal to the spatial flattened size and the input size was determined by the hidden size of the LSTM. The LSTM also has a number of layers which determine the complexity. The model is trained with MSE loss and Adam optimizer in Pytorch.

Simulation methodology: We have the following variables in our simulations: the genie CSI sample every 5ms, the predicted sample, and the sample and hold sample (SH) which is the last sample from the observation window. The baseline prediction is the SH scheme and we aim at evaluating the gains of AI/ML prediction compared to SH. The comparisons are done with 2 metrics: NMSE and SGCS. For SGCS, we compute the covariance matrix across tones for each CSI sample (predicted, genie, and SH) , and we compute the principal eigenvector

Fig 1. Sliding window based prediction

Table 2: Updated simulation results for AIML based CSI prediction (Ideal CE, no codebook)

Table 3: Updated simulation results (Ideal CE, no codebook) trained mixed datasets 16 ports

Table 3: Updated simulation results (Ideal CE, no codebook) trained mixed datasets 32 ports

Observation 1: AI/ML-based prediction shows notable performance improvements over the Sample-and-Hold method for Doppler spreads up to 50 Hz. However, these gains tend to diminish as the Doppler spread increases, with little to no improvement observed around 100 Hz.

Proposal 1: RAN4 to perform simulation results alignment among companies. If simulation results don’t match, companies should be encouraged to share CSI prediction datasets and/or agree on AI/ML model architectures

Proposal 2: Simulation results indicate a tradeoff between training and testing losses, highlighting potential overfitting when models are not well matched to the Doppler conditions. RAN4 is invited to discuss the need for selecting or designing AI/ML models optimized for specific Doppler spreads, or alternatively, exploring model fine-tuning or generalization techniques that enable a single model to effectively support a range of Doppler conditions without requiring multiple models.

# Conclusion

In conclusion the following observations and proposals were discussed in this contribution:

Observation 1: AI/ML-based prediction shows notable performance improvements over the Sample-and-Hold method for Doppler spreads up to 50 Hz. However, these gains tend to diminish as the Doppler spread increases, with little to no improvement observed around 100 Hz.

Proposal 1: RAN4 to perform simulation results alignment among companies. If simulation results don’t match, companies should be encouraged to share CSI prediction datasets and/or agree on AI/ML model architectures

Proposal 2: Simulation results indicate a tradeoff between training and testing losses, highlighting potential overfitting when models are not well matched to the Doppler conditions. RAN4 is invited to discuss the need for selecting or designing AI/ML models optimized for specific Doppler spreads, or alternatively, exploring model fine-tuning or generalization techniques that enable a single model to effectively support a range of Doppler conditions without requiring multiple models.

# Reference

[1] RP-242399 – “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”

[2] 3GPP TR 38.843, “Technical Specification Group Radio Access Network;Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”