3GPP TSG-RAN WG4 Meeting #115					 R4-2505962

Malta, MT,19th - 23rd May, 2025

Source:	Qualcomm Incorporated

Title:	Power boosting and MPR reduction

Agenda item:	7.1.2.2.1

Document for:	Approval

# 1.	Introduction

In RAN4#114-bis there was a new WF [1] to further study power domain enhancements for NR single carrier. In this paper we present additional views on this topic.

# 2. 	Discussion

In RAN4#114-bis there was a WF created [1] to further study the open items on power domain enhancements for NR single carrier.

In the last meeting it was agreed to use NRB for BW extensions as noted below [1]:

#### Issue 1-1-1: RB based or CBW based extension

- WF

  - Symmetrical/ Asymmetric for maximum extension at each side are determined by 0.5*NRB

  - If a limited number of additional fixed values are introduced, the extension is determined by X*NRB at each side, where X < 0.5

  - The extended RBs are aligned with common RB grid

Also in the last meeting whether the UE either calculates or is informed of the free RBs on either side of the UE CBW was unresolved. The two proposals discussed for determining the number of free RBs were by indication through NW signalling or by UE calculation based on information received during RRC configuration and via SIB information. We think that it would be easier for the NW to perform the computations as it has all the necessary information more readily than the UE. Therefore the simplist solution is for the NW to signal the UE bandwidth expansion. We have clarified our proposal from the last meeting based on the agreement made for issue 1-1-1 where BW extension is given in terms of RBs [2].

Proposal 1: The symmetric or asymmetric UE CBW extension in terms of RBs is signalled by the NW.

In this contribution, ‘RBs available for ‘extension’ on each side of the UE CBW’ are referred to as ‘free RBs’ for compactness.

In the above mentioned WF it was proposed that companies further discuss how best to define the new inner RB region in the extended UE CBW scenario. As mentioned in our previous paper [2] attempting to formulate the appropriate wording to describe the inner RB region when the UE CBW is extended leads to confusing language which is open to misinterpretation. We think it is best to describe the new inner region based on the equations in section 6.2 of the standard [3] but with NRB modified to account for BW extension on one or both sides of the original UE BW as indicated below.

RBStart,Low = max(1,floor(LCRB/2))

NRB’=NRB+p+q

And ‘p’ and ‘q’ are the RBs available for BW extension on the left and right edge of the UE CBW respectively

RBStart,High = NRB’-RBStart,Low-LCRB=NRB+p+q-RBStart,Low-LCRB

RBStart,Low ≤ RBStart ≤RBStart,High     & LCRB ≤ Ceil(NRB/2)

An RB allocation belongs to the inner region if the following equations are satisfied

RBStart,Low ≤ RBStart ≤RBStart,High     & LCRB ≤ Ceil(NRB/2)

RBStart,Low = max(1,floor(LCRB/2)

RBStart,High = NRB’-RBStart,Low-LCRB

Where:

NRB’=NRB+p+q

And ‘p’ and ‘q’ are the RBs available for BW extension on the left and right edge of the UE CBW respectively

Proposal 2: Use equation-based method to determine the new inner RB region in the extended UE CBW scenario.

Extra virtual RBs are added starting from the left of RB(1) and the right of RB(NRB) and there is no need for a guard band between the extra RBs and NRB of the original UE CBW.

Proposal 3: There is no guard band between the UE CBW and the extended UE CBW

To reduce the validation effort, it is proposed to set the bandwidth extension to a maximum of NRB/2 based on the number of free RBs and spurious emissions requirements as detailed later in this paper.

ITU in SM.329 define SE based on the ‘necessary bandwidth’ (NB), which in turn is defined as:

For a given class of emission, the width of the frequency band which is just sufficient to ensure the transmission of information at the rate and with the quality required under specified conditions.

Since NB is unchanged despite the CBW extension, the frequency ranges of applicability of the ITU requirements also remain unchanged, at 2*NB away from channel edge. UE CBW can be used as proxy for NB.

Proposal 4: SE starts no further than 2x UE CBW from channel edge when network enables UE to assume BW extension

Based on the agreements from RAN4#114 [4] the SE limits apply for frequency ranges that are more than UE CBW+5MHz from the edge of the extended UE CBW. Based on this the UE CBW extension, X on either side should be configured such that the frequency offset from where the ITU emissions start should be greater than the frequency offset from where 3GPP SE limits apply. This means that:

(2*UE CBW) ≥ (X+UE CBW + 5MHz).

i.e. X ≤ (UE CBW - 5MHz)

From the above it can be seens that the ITU SE specification limits the UE CBW extension.

Observation 1: The maximum allowed extension on each side is limited to (UE CBW - 5MHz)

Proposal 5: If the number of free RBs is greater than NRB/2 then the UE CBW extension is limited to NRB/2 if the number of RBs in the BW = (UE CBW - 5MHz) is greater than or equal to NRB/2. If the number of free RBs is less than NRB/2 or the number of RBs in the BW = (UE CBW - 5MHz) is less than NRB/2 then no UE CBW extension is allowed.

In our opinion it makes sense to use IBE from the edges of the original UE transmission bandwidth configuration to the inner edges of the extended UE guardband, which means that the IBE should cover the original UE CBW guardband as well. Also, the IBE in this region should be an extension of the IBE in the original UE CBW region as indicated in figure 3.There has been much debate regarding using a flat IBE profile which we think may not provide sufficient protection for the adjacent users given that due to BW extension there is no SEM, spurs and ACLR requirements in the extended BW. Also, by inspecting the IBE equations in the standard there does not seem to be an issue with extending the evaluation BW to include the extended RB [3]. An inspection of the IBE equations in the standard reveals that the IBE requirement in the original UE BW remains unchanged even with BW extension [3].

Proposal 6: IBE should be used from the edges of the original UE transmission bandwidth configuration to the inner edges of the extended UE guardband but not include it.

Figure 3: Extension of IBE mask

Proposal 7: The IBE from the edges of original UE transmission bandwidth configuration to the inner edges of the extended guardband region should be an extension of the IBE for the signal within the UE CBW.

We think that the guard band at the edge of the extended UE CBW can be the same as the guard band of the original UE CBW as there is no signal occupying the extended UE CBW region the width of the guardband is not critical.

Proposal 8: The guard band at the edge of the extended UE CBW is set to be the same value as that of the original UE CBW.

# Conclusion

In this paper we further discuss our views on the study of power domain enhancement and make the following proposals:

Proposal 1: The symmetric or asymmetric UE CBW extension in terms of RBs is signalled by the NW.

Proposal 2: Use equation-based method to determine the new inner RB region in the extended UE CBW scenario.

Proposal 3: There is no guard band between the UE CBW and the extended UE CBW

Proposal 4: SE starts no further than 2x UE CBW from channel edge when network enables UE to assume BW extension

Observation 1: The maximum allowed extension on each side is limited to (UE CBW - 5MHz)

Proposal 5: If the number of free RBs is greater than NRB/2 then the UE CBW extension is limited to NRB/2 if the number of RBs in the BW = (UE CBW - 5MHz) is greater than or equal to NRB/2. If the number of free RBs is less than NRB/2 or the number of RBs in the BW = (UE CBW - 5MHz) is less than NRB/2 then no UE CBW extension is allowed.

Proposal 6: IBE should be used from the edges of the original UE transmission bandwidth configuration to the inner edges of the extended UE guardband but not include it.

Proposal 7: The IBE from the edges of original UE transmission bandwidth configuration to the inner edges of the extended guardband region should be an extension of the IBE for the signal within the UE CBW.

Proposal 8: The guard band at the edge of the extended UE CBW is set to be the same as that of the original UE CBW.

# References

[1] R4-2505099, WF on power domain enhancements, RAN4#114-bis, April 7th – April 11th, 2025

[2] R4-2503512, Power boosting and MPR reduction, RAN4#114-bis, April 7th – April 11th, 2025

[3] TS38.101-1, User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone, V19.1.0

[4] R4-2503013, WF on power domain enhancements, RAN4#114, February 7th – February 11th, 2024

R4-2418015, Redefining allocation types for improved MPR with BS BW larger than UE CBW, USA, 18 – 22 November 2024

[3] R4-2417204, WF on power domain enhancements, RAN4#112-bis, October 12th – October 18th , 2024