3GPP TSG-RAN WG4 Meeting #115            	R4-2505970

Malta, MT, 19th-23th, May, 2024

Agenda item:	7.10.4.6

Source:	Samsung

Title:	Discussion on NTN demodulation requirements for Less than 5MHz

Document for:	Discussion

- Introduction

In RAN#104 meeting, the revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN has been approved in [1]. As one of objective is less than 5MHz for NTN with following

Less than 5MHz for NTN (Core Part) - Specify the system parameters for channel bandwidth for less than 5MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary signaling to support the above objectives Less than 5MHz for NTN (Performance part) - Specify the necessary conformance requirements for SAN for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary RRM performance requirements and test cases for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary demodulation performance requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

In the last RAN4 meeting, the test scope of performance part of NTN for less than 5MHz was under discussion. The related agreement was captured into WF in [2] as following

UE Demodulation performance requirements - PDSCH - No new PDSCH requirement - PDCCH - Option 1: No new PDCCH requirement - Option 2: Discuss the feasibility of introducing PDCCH requirement - PBCH - No new PBCH requirement - CSI reporting - No new CSI reporting requirement SAN Demodulation performance requirements - PUSCH - FFS to define new PUSCH requirements - Input from NTN operators can be considered. - PUCCH

In this contribution, the view on NTN demodulation performance requirements for less than 5MHz was provided.

# 2	Test Scope

In general, the demodulation requirement specified in Rel-18 TN NR_FR1_less than _5MHz_BW can be leveraged to specify requirements for FR1 NR NTN.

For Less than 5MHz, based on RAN1 design, the main impact on physical layer channel is SSB/PBCH, PDCCH and the available channel bandwidth for both uplink and downlink data channel, where the available channel bandwidth is 12PRB or 15PRB pending on different bands.

## 2.1	Test Scope for UE demodulation

For PDCCH, according to RAN1 agreement 12PRB and 15PRB CORESET#0 is supported for 3MHz. For 12 PRB PDCCH no puncturing is expected, while upper 9 PRBs are punctured from 24 PRB PDCCH for 15 PRB PDCCH. The punctured PDCCH requirements has already introduced in TN with supporting less than 5MHz.

Observation 1:  RAN4 has already introduced the punctured PDCCH in Rel-18 for NR supporting less than 5MHz.

For FR1 NR NTN, considering a UE to support NTN feature should meet the requirements defined for TN and NTN, specified in 38.101-4 and 38.101-5, respectively. Since PDCCH requirement with less than 5MHz has introduced in TN, based on requirement applicability rule, there is no necessary to define the corresponding requirement for FR1 NR NTN with supporting less than 5MHz

Proposal 1 No PDCCH requirement introduced for NTN with less than 5MHz

## 2.2	Test Scope for SAN demodulation

In Rel-18 NR_FR1_less than _5MHz, RAN4 has lots of discussion whether to define the dedicated requirement for 3MHz for PUSCH, PUCCH and PRACH.

For PUSCH, it is RAN4 agreement that there are no BSs that supporting only less than 5MHz, and based on the existing test applicability rule for different CBWs, 3MHz requirements will never be tested. Therefore, RAN4 agreed that not to define PUSCH requirements with 3MHz bandwidth for both non-HST and HST condition, as well as UL timing adjustment requirement.

For PUCCH, based on companies’ evaluation, there is no obvious performance different observed in PUCCH format 0/1/3/4 compared with existing 5MHz requirements. Therefore, there is no necessary to introduce new requirements for these formats from performance reliability perspective, and only introduce the new demodulation requirement for PUCCH format 2 with 3MHz CBW. And meanwhile, to reduce the effort, a test applicability rule for new introduced UCI performance with 3MHz and legacy 5MHz was introduced.

For PRACH, its requirement is no differentiate with transmission BWs if its configurations can be fitted into the transmission BW, the minimum number of RB as 15 for 3MHz excepting for n100, and the minimum number of RB as 12 for 3MHz with n100, where the available number of subcarriers is 180 for 15 RB, 144 for 12RB, respectively. Then, the existing long PRACH format with LRA=839 and short PRACH formats with LRA=139 can be fitted into the transmission BW of 3MHz. Therefore, it was agreed that no new PRACH requirement for NR with Less than 5MHz, and adding an applicability rule that for BS supporting less than 5MHz carrier bandwidth, only test requirements relating to PRACH preamble format3 with 15KHz SCS with sequence length LRA=139, and PRACH format with 1.25kHz SCS with sequence length LRA=839 shall apply.

Observation 2:  No PUSCH requirements with 3MHz bandwidth for both non-HST and HST condition, as well as UL timing adjustment requirement were introduced in Rel-18 NR with less than 5MHz

Observation 3:  Only introduce new demodulation requirement for PUCCH format 2 with 3MHz and introduce the test applicability rule for new introduced UCI performance with 3MHz and legacy 5MHz only for format 2

Observation 4:  No new PRACH requirement for NR with less than 5MHz and introduce the applicability rule for BS supporting less than 5MHz carrier bandwidth.

For NR NTN with supporting less than 5MHz, based on current RF discussion, the bands (n256, n255, n254) supporting less than 5MHz, it also supports other channel bandwidths, including 5MHz, 10MHz, and 20MHz. Therefore, there is no SAN only support less than 5MHz. So, we would like to apply the similar principle as NR supporting less than 5MHz to specify the NR NTN SAN performance requirement with supporting less than 5MHz.

For SAN PUSCH requirement, no new SAN PUSCH requirement with 3MHz bandwidth for PUSCH with transform precoding disabled, with transform precoding enabled, UL timing adjustment, PUSCH repetition Type A and PUSCH with DM-RS bundling.

For SAN PUCCH requirement, the NTN channel condition is different with NR, where the number of multi-paths is less and channel condition is LOS dominant. The channel frequency selectivity diversity gain is similar with 5MHz and 3MHz due to the frequency hopping operation.  So, we would like to further check whether there is any obvious performance difference with 3MHz under NTN channel condition compared with 5MHz

Proposal 2:  No New SAN PUSCH requirement with 3MHz bandwidth for PUSCH with transform precoding disabled, with transform precoding enabled, UL timing adjustment, PUSCH repetition Type A and PUSCH with DM-RS bundling if no SAN that supporting only less than 5MHz

Proposal 3:  FFS to introduce SAN PUCCH requirement with 3MHz bandwidth under NR NTN channel condition

Since based on PUCCH evaluation in NR with less than 5MHz, only PUCCH format 2 shows some performance different with 5MHz. Therefore, we only prefer to consider specify the PUCCH format 2  requirement with 3MHz, if there is performance different

Proposal 4:  Only considering to specify SAN PUCCH format 2 requirement if there is obvious performance difference compared to the performance in 5MHz under NTN channel condition.

For SAN PRACH, similar as NR with less than 5MHz, the test applicability rule can be added for NR NTN with supporting less than 5MHz

- For SAN supporting less than 5MHz carrier bandwidth, only test requirements relating to short RACH preamble formats with 15kHz SCS with sequence length LRA=139, and long PRACH formats with 1.25kHz SCS with sequence length LRA=839 shall apply

Proposal 5:  Adding an applicability rule for PRACH requirement targeting NR NTN SAN supporting less than 5MHz carrier bandwidth

- For SAN supporting less than 5MHz carrier bandwidth, only test requirements relating to short RACH preamble formats with 15kHz SCS with sequence length LRA=139, and long PRACH formats with 1.25kHz SCS with sequence length LRA=839 shall apply

# 3	Simulation Assumption

In this section, the simulation assumption is provided for both UE and SAN performance requirement.

## 3.1	Simulation Assumption for SAN demodulation

In order to evaluate the PUCCH performance with less than 5MHz under NTN channel condition, the existing PUCCH test cases for FR1 NTN can be considered as starting point.

PUCCH format 2

Table 4: Simulation assumption for PUCCH format 2 with NTN less than 5MHz

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| Frist PRB after frequency hopping | Option 1: The largest PRB index – (Number of PRBs – 1) Option 2: 3 |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |

Proposal 6:  The following simulation assumption can be considered for evaluation SAN PUCCH format 2

performance with less than 5MHz

PUCCH format 2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| Frist PRB after frequency hopping | Option 1: The largest PRB index – (Number of PRBs – 1) Option 2: 3 |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |

# 4	Conclusion

In this contribution, the view on test scope of demodulation requirement for NTN supporting less than 5MHz.

UE demodulation requirement

Proposal 1 No PDCCH requirement introduced for NTN with less than 5MHz

SAN demodulation requirement

Observation 2:  No PUSCH requirements with 3MHz bandwidth for both non-HST and HST condition, as well as UL timing adjustment requirement were introduced in Rel-18 NR with less than 5MHz

Observation 3:  Only introduce new demodulation requirement for PUCCH format 2 with 3MHz and introduce the test applicability rule for new introduced UCI performance with 3MHz and legacy 5MHz only for format 2

Observation 4:  No new PRACH requirement for NR with less than 5MHz and introduce the applicability rule for BS supporting less than 5MHz carrier bandwidth.

Proposal 2:  No New SAN PUSCH requirement with 3MHz bandwidth for PUSCH with transform precoding disabled, with transform precoding enabled, UL timing adjustment, PUSCH repetition Type A and PUSCH with DM-RS bundling if no SAN that supporting only less than 5MHz

Proposal 3:  FFS to introduce SAN PUCCH requirement with 3MHz bandwidth under NR NTN channel condition

Proposal 4:  Only considering to specify SAN PUCCH format 2 requirement if there is obvious performance difference compared to the performance in 5MHz under NTN channel condition.

Proposal 5:  Adding an applicability rule for PRACH requirement targeting NR NTN SAN supporting less than 5MHz carrier bandwidth

- For SAN supporting less than 5MHz carrier bandwidth, only test requirements relating to short RACH preamble formats with 15kHz SCS with sequence length LRA=139, and long PRACH formats with 1.25kHz SCS with sequence length LRA=839 shall apply

Proposal 6:  The following simulation assumption can be considered for evaluation SAN PUCCH format 2

performance with less than 5MHz

PUCCH format 2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| Frist PRB after frequency hopping | Option 1: The largest PRB index – (Number of PRBs – 1) Option 2: 3 |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |

# 5	 References

R4-241281, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN, Samsung

R4- 2412554, Workplan for enhanced requirements and conductive test methodology, Samsung

R4-2504753, Way Forward for [114bis][324] NR_IoT_NTN_less_than_5MHz_demod, Ericsson