3GPP TSG-RAN WG4 Meeting #115            	R4-2505974

Malta, MT, 19th-23th, May, 2025

Agenda item:	7.21.3

Source:	Samsung

Title:	View on CSI reporting requirements framework for CSI prediction

Document for:	Discussion

- Introduction

In the last RAN4#114bis meeting, CSI reporting requirement framework for CSI prediction was under discussed. The related agreed agreements are captured in the WF [1] as following

- CSI reporting requirement framework for CSI prediction - Codebook to be used in the tests - In Rel-19 only PMI requirements based on AI/ML CSI prediction will be defined - use Rel-18 eType II Doppler codebook for AI/ML based CSI predictions - Requirement baseline - Requirement to be introduced for Dopler domain basis N4=1 - N4>1 can be further discussed in the future if needed - Test metric - Requirements to be defined based on the ratio (same metric as for the current PMI reporting requirements in 38.101-4) - tue_AI/MLis 90 % of the maximum throughput obtained at SNRue_AI/ML using the precoders configured according to the UE reports - trnd is the throughput measured at SNRue_AI/ML with random precoding., - trnd is obtained with Rel-15 type I single panel codebook - Note: SNRue_AI/ML is calculated in the same way as SNRue and SNRfollow1, follow2 in section 6.3 of 38.101-4

Meanwhile, in order to facilitate the AI/ML model evaluation and alignment, the initial simulation assumption is agreed and captured in the [2]

In this contribution, the view on simulation assumption for step 3 is provide. Meanwhile, the initial simulation results for AI/ML based CSI prediction calibration and comparison were provided.

# 3	CSI reporting requirement

## 3.1	Test Assumption for CSI prediction

In the last meeting, in order to facilitate the AI/ML model evaluation and alignment, the initial simulation assumption was agreed. We would like to provide our view for further proceeding the AI/ML CSI prediction evaluation in step 2 and step 3.

Baseline for comparison

For RAN4 requirement perspective, although it was agreed to consider the reference throughout is obtained based on the random PMI with Rel-15 Type 1 single panel codebook, we think it is also necessary to compare the ratio of throughput obtained based on following PMI with Rel-18 eType II and random PMI with Rel-15 type I single panel codebook, considering it is expected that AI/ML based performance requirement should not be worse than the legacy (non-AI/ML based) if a relevant equivalent legacy requirement/feature exists.

Proposal 1:  RAN4 should also compare the ratio of throughput obtained based on following PMI with Rel-18 eType II and random PMI with Rel-15 type I single panel codebook, when defining the requirement of AI/ML based CSI prediction with Rel-18 eType II doppler codebook.

Channel Estimation and related SNR point(range) for AI/ML model verification

Based on simulation assumption, for the purpose of calibration and comparing the intermediate results, e.g., accuracy of AI/ML output CSI, the ideal DL channel estimation was used for evaluation as the first step. From RAN4 demodulation requirement perspective, it was straightforward to apply the practical channel estimation method for specifying the related requirement. In general, the channel estimation algorithm and SNR level will have impact on the accuracy of channel estimation of input CSI, which also will impact in the AI/ML model performance. Therefore, RAN4 may need to discuss whether the baseline assumption for practical channel estimation should be specified, or it is up to companies’ implementation

Proposal 2:  RAN4 may need to discuss whether the baseline assumption for practical channel estimation should be specified, or it is up to companies’ implementation

For SNR level, in general, it should be impossible to use one AI/ML model to cover all the SNR range. With low SNR range, the channel correlation property will be destroyed by noise signal. It is meaningless and difficult to use AI/ML model for CSI prediction training with low SNR level data. The data collection for model training should be discussed to sure the proper CSI prediction performance.  In legacy PMI test, the SNR with 90% of throughput achieved based on following PMI will be used for calculating the ratio of throughput with following PMI and that with random PMI. Therefore, we would like to propose to use the SNR value with 90% of throughput achieved based on following PMI as starting point, based on this SNR, considering ±X dB range for data collection for AI/ML training, where the SNR value is derived based on existing Rel-18 eType II Doppler codebook under the same configuration.

Meanwhile, regarding the training, RAN4 should also discuss whether the AI model is training based on dataset with specific SNR condition or mixed dataset with the selected SNR range, when applying the practical channel estimation.

Proposal 3: RAN4 shall discuss how to choose the proper SNR range for AI/ML based CSI prediction model training and test when applying the practical channel estimation.  The following method can be considered as starting point:

- Use the SNR value with 90% of throughput achieved based on following PMI as starting point, based on this SNR, considering ±X dB range for data collection for AI/ML training, where the SNR value is derived based on existing Rel-18 eType II Doppler codebook under the same configuration

RAN4 should also discuss whether the AI model is training based on dataset with specific SNR condition or mixed dataset with the selected SNR range, when applying the practical channel estimation

## 3.2	Initial simulation results

In this section, based on the agreed simulation assumption in the last meeting, the initial simulation results are provided to check the AI/ML model for CSI prediction for different UE velocity and different antenna configuration with following cases in Step1 for 2nd round simulation, based on the WF.

Table 1. FDD case for PMI reporting with AI/ML CSI prediction

| Parameter | Value |
| --- | --- |
| Carrier frequency | 2GHz for FDD |
| Bandwidth | 10MHz for FDD, |
| Symbol | 1 symbol |
| Number of subbands | 13 subbands, 4RB as a bundle for FDD |
| Subcarrier spacing | 15kHz for 2GHz |
| gNB TX antennas | Option1: 32 (N1, N2) = (4,4) as defined in Rel-15 type1 Option2: 16 (N1, N2) = (4,2) as defined in Rel-18 CSI prediction requirements |
| Doppler spread | 20Hz, 50Hz and 100Hz |
| Rank per UE | Rank 2 |
| Observation window | Observation window (number/distance): 5/5ms as baseline |
| Prediction window | Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance): 1/5ms/5ms |
| Training data and test data | Use following combination Training dataset (TDLA30); Test on (TDLA30). 20Hz Doppler spreads; 20Hz Doppler spreads. 50Hz Doppler spreads; 50Hz Doppler spreads. 100Hz Doppler spreads; 100Hz Doppler spreads. Mixed Doppler spreads (20Hz, 50Hz, 100Hz); 20Hz Doppler spreads. 50Hz Doppler spreads. 100Hz Doppler spreads. |

As step 1, the intermediate KPI based the average SGSC over all sub-bands per layer was provided for rank 2 cases.

3.2. 1	Intermediate KPI

In the performance evaluation, the raw channel matrices estimated based on the CSI-RS signals are feed into the AI model as input data. The CSI-RS interval is defined as 5ms. The AI model predicted the channel information in the next one CSI-RS slot, based on the historical 5 CSI-RS channel information, i.e., the observation window is 5/5ms and the prediction window is 1/5ms/5ms. The following table show the initial performance comparison of the AI/ML-based CSI prediction schemes under various UE moving speed scenarios.

Table 2 SGCS performance comparison over different UE speeds for 16Tx4Rx with Rank 2, Observation window (5/5ms) based on non-mixed dataset

| Testing Scenarios | AI/ML-based Model (Training, UE speed) |  |  |  |  |  | Non-AI Benchmark#1 (sample and hold) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20Hz |  | 50Hz |  | 100Hz |  |  |  |
|  | Layer 1 | Layer 2 | Layer 1 | Layer 2 | Layer 1 | Layer 2 | Layer 1 | Layer 2 |
| 20Hz | 0.9999 | 0.9997 | 0.9669 | 0.9403 | 0.3242 | 0.1669 | 0.7922 | 0.6623 |
| 50Hz | 0.7117 | 0.5187 | 0.9298 | 0.8836 | 0.2989 | 0.1843 | 0.3332 | 0.2020 |
| 100Hz | 0.1727 | 0.1379 | 0.1798 | 0.1348 | 0.1929 | 0.1424 | 0.1878 | 0.1545 |

Table 3 SGCS performance comparison over different UE speeds for 16Tx4Rx with Rank 2, Observation window (5/5ms) based mixed dataset (To be updated)

| Testing Scenarios | AI/ML-based Model (Training, UE speed) |  |  |  |  |  | Non-AI Benchmark#1 # (sample and hold) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20Hz |  | 50Hz |  | 100Hz |  |  |  |
|  | Layer 1 | Layer 2 | Layer 1 | Layer 2 | Layer 1 | Layer 2 | Layer 1 | Layer 2 |
| 20Hz |  |  |  |  |  |  |  |  |
| 50Hz |  |  |  |  |  |  |  |  |
| 100Hz |  |  |  |  |  |  |  |  |

Model complexity

In the current our evaluation, the transformer structure is used with the following model complexity information

| backbone | Transformer |
| --- | --- |
| Number of FLOPs | Layer 2 |
| Model Parameters | 2.28K |

From the initial simulation result, it can be seen that the SGCS performance of the AI/ML-based scheme is better than the nearest historical CSI scheme (benchmark#1) for the cases with doppler as 20Hz, and 50Hz, while in the case with doppler is 100Hz, AI/ML-based scheme has no gain compared to the benchmark (sample and hold)

Meanwhile, there’s a generalization problem for the AI/ML-based CSI prediction scheme, which needs further investigations.

Observation 1:  AI/ML based CSI prediction show better performance compared with nearest historical CSI scheme (benchmark#1) for cases with doppler as 20Hz and 50Hz.  Generalization problem needs further study when defining requirement

# 4	Conclusion

In this contribution, the view on CSI reporting requirements framework was provided. Meanwhile, the initial simulation results for AI/ML CSI prediction were provided to facilitate the calibration and comparison.

Proposal 1:  RAN4 should also compare the ratio of throughput obtained based on following PMI with Rel-18 eType II and random PMI with Rel-15 type I single panel codebook, when defining the requirement of AI/ML based CSI prediction with Rel-18 eType II doppler codebook.

Proposal 2:  RAN4 may need to discuss whether the baseline assumption for practical channel estimation should be specified, or it is up to companies’ implementation

Proposal 3: RAN4 shall discuss how to choose the proper SNR range for AI/ML based CSI prediction model training and test when applying the practical channel estimation.  The following method can be considered as starting point:

- Use the SNR value with 90% of throughput achieved based on following PMI as starting point, based on this SNR, considering ±X dB range for data collection for AI/ML training, where the SNR value is derived based on existing Rel-18 eType II Doppler codebook under the same configuration

RAN4 should also discuss whether the AI model is training based on dataset with specific SNR condition or mixed dataset with the selected SNR range, when applying the practical channel estimation

Observation 1:  AI/ML based CSI prediction show better performance compared with nearest historical CSI scheme (benchmark#1) for cases with doppler as 20Hz and 50Hz.  Generalization problem needs further study when defining requirement

# References

R4-2502856, WF on AI/ML for NR air interface

R4-2502970, Way forward on simulation assumptions for CSI prediction

# Appendix