3GPP RAN WG4 Meeting #115	R4-2505985

Saint Julian’s, Malta, May 19th – May 23rd, 2025

Title: 	Discussion on impacts of AIML RRM measurement prediction on RRM requirements

Source: 	CATT

Agenda item:	7.23.3.1

Document for:	Discussion

Introduction

In RAN4#114bis meeting, RAN4 discussed the RRM impacts of AI/ML based mobility and the agreements were captured in the WF [1]. In this contribution, we will further discuss the open issues listed in [2] and present our views.

Discussion

General and scope

Radio Link Failure (RFL) prediction use case

According to RAN2 agreements [3], AI/ML based radio link failure prediction will not be studied in Rel-19, and therefore RAN4 do not need to study this use case either.

Agreements on priority 1. RLF prediction will not be studied in Rel-19.

Proposal 1: RAN4 do not study radio link failure use case in Rel-19.

In the rest of contribution, we will focus on RRM measurement prediction use case. The scenarios are copied here for information (highlighted scenarios are prioritized):

| Scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

According to TR 38.744 [4], two study goals are as follows:

1st goal: to reduce measurement efforts in temporal, spatial or frequency domain by using predicted measurements.

2nd goal: to improve the handover performance (e.g., Ping-pong HO, HOF/RLF, short time of stay, Handover interruption).

Accuracy requirement for predicted measurements

In last meeting, proposals that are related to performance requirements were brought up:

Proposals and Observations: - Proposal 1 (Ericsson): RAN4 to conclude that the predicted measurements should at least meet the accuracy requirements that are defined for the non-AI/ML based RRM measurements for mobility purposes. - Proposal 1 (Huawei): RAN4 to discuss whether to define separate prediction accuracy for each sub-use case or define one prediction accuracy requirement to cover 3 cell level sub-use cases.

Regarding the first proposal, it is quite straightforward for the case with the 2nd study goal since the goal is to improve handover performance. For the case with 1st study goal which is to reduce measurement efforts, it is possible that the prediction accuracy slightly degrades when UE saves measurement efforts. Therefore RAN4 need further discuss whether a relaxed prediction accuracy requirement is defined.

On whether separate or unified prediction accuracy requirement is defined for 3 cell level sub-use cases, RAN4 can discuss after concluding whether to consider relaxed accuracy requirement for scenario 4 where the 2nd study goal applies.

Proposal 2: For scenario 4 (2nd study goal), the predicted measurements should at least meet the accuracy requirements that are defined for the non-AI/ML based RRM measurements for mobility purposes.

Proposal 3: For scenarios with 1st study goal, RAN4 discuss whether a relaxed prediction accuracy requirement is defined.

Relative RSRP Prediction Accuracy

In last meeting, RAN4 discussed the definition of relative RSRP prediction accuracy and the following agreement was achieved:

RAN4#114bis Agreement: - For intra-frequency L3-RSRP, - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 and cell 2 are on the same frequency, or - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of beam 1 – reported predicted RSRP of beam 2) – (ground truth of RSRP of beam 1 – ground truth of RSRP of beam 2), where beam 1 and beam 2 are from the same cell. - FFS if this definition will be needed. - For inter-frequency L3-RSRP, - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, if cell 1 and cell 2 are the target cells for AI/ML-based L3-RSRP inference. - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, [if cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2]. - Depending on the agreement on ground truth definition, we may revisit relative RSRP accuracy definition. - FFS: whether the reported RSRP of cell2 can be predicted or measured or a combination of both predicted and measured samples. - FFS, if there is any impact on this definition for different scenarios/different frequency ranges.

Regarding the 1st FFS, we can capture this definition in the TR as one of study outcomes.

For the 2nd FFS, the typical scenario is UE predicts the L3 RSRP of target cell 1 based on the measurements on cell 2, and therefore the reported RSRP of cell 2 is at least measured. Whether it can be predicted or not depends on the applicability of prediction window. RAN4 can further discuss this issue in WI phase after RAN2 finish the design on observation window and prediction window configuration.

Proposal 4: RAN4 to capture the relative RSRP prediction definition for intra-frequency beam L3-RSRP in the TR as one of study outcomes.

Proposal 5: The reported RSRP of cell 2 can be measured at least. RAN4 further discuss it in WI phase after RAN2 finish the design on observation window and prediction window configuration.

Impacts on beam/cell level measurements

In previous RAN4 meetings, the following agreements were achieved for further discussion:

RAN4#112bis: Agreement: - RAN4 to first study the impacts on requirements for the following case: - L3 cell level RSRP (Point C) prediction - FFS L1 beam-level (Point A1) - FFS L3 beam-level (Point E) - Other cases can be considered based on RAN4 further discussion and RAN2 further progress. RAN4#114bis: Agreement: - For Rel. 19 study: - RAN4 to not consider L1 beam-level (Point A1) measurement prediction.

Considering that RAN2 still deprioritizes L3 filtered beam-level prediction sub-use cases (E) and they focuses on FR2 intra-frequency temporal domain case A, i.e., scenario 4, at the current stage, RAN4 can deprioritize the study of impacts of L3 beam-level (Point E) on RRM requirements.

TR 38.744: 3 sub-use cases are considered for beam-level RRM measurement prediction: Sub-use case 4: L1 filtered beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 beam-level measurement result is generated Sub-use case 5: L3 beam-level measurement result(s) is predicted based on actual L3 beam-level measurement result(s) Sub-use case 6: L3 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) Editor Note 1: L3 filtered beam level prediction sub-use cases are lower priority and should focus on FR2 intra-frequency temporal domain case A.

Proposal 6: RAN4 to deprioritize the study on impact of L3 beam-level measurement prediction at point E to align with RAN2 prioritization.

Impacts from observation and prediction windows

Regarding the impacts of observation and prediction windows, agreements reached in RAN4#114 meeting were copied below:

RAN4#114 Agreement: - RAN4 to consider the impact of UE speed, MRRT and OW and PW lengths on prediction accuracy requirements for temporal predictions. - RAN4 to also consider number of measurements performed during OW and its impacts on the prediction accuracy.

RAN4 agreed to consider the impact of UE speed, MRRT and OW and PW lengths on RRM requirements for temporal predictions. But since this is a study item now and the framework of AI/ML based mobility is not designed by RAN2 yet, RAN4 can discuss the details of impact of UE speed, MRRT and OW and PW lengths in the work item phase based on more concrete foundations after RAN2 have progresses.

Proposal 7: RAN4 to discuss the impact of UE speed, MRRT and OW and PW lengths in the work item phase unless any issues are identified to be discussed.

Conclusions

This contribution discussed the impacts of AI/ML RRM measurement prediction on RRM requirements, and some observations and proposals are provided:

Proposal 1: RAN4 do not study radio link failure use case in Rel-19.

Proposal 2: For scenario 4 (2nd study goal), the predicted measurements should at least meet the accuracy requirements that are defined for the non-AI/ML based RRM measurements for mobility purposes.

Proposal 3: For scenarios with 1st study goal, RAN4 discuss whether a relaxed prediction accuracy requirement is defined.

Proposal 4: RAN4 to capture the relative RSRP prediction definition for intra-frequency beam L3-RSRP in the TR as one of study outcomes.

Proposal 5: The reported RSRP of cell 2 can be measured at least. RAN4 further discuss it in WI phase after RAN2 finish the design on observation window and prediction window configuration.

Proposal 6: RAN4 to deprioritize the study on impact of L3 beam-level measurement prediction at point E to align with RAN2 prioritization.

Proposal 7: RAN4 to discuss the impact of UE speed, MRRT and OW and PW lengths in the work item phase unless any issues are identified to be discussed.

References

[1] R4-2504901, WF on AI/ML Mobility, Nokia, RAN4#114bis.

[2] R4-2503620, Topic summary for [114bis][210] FS_NR_AIML_Mob, Nokia, RAN4#114bis.

[3] Report of 3GPP TSG RAN WG2 meeting #129, RAN2#129.

[4] 3GPP TR 38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, V0.0.4.