3GPP TSG-RAN WG4 Meeting #115                                                                                           R4-2506021

Malta, MT, 11th –23rd May, 2025

Source: 	CATT

Title: 	Further discussion on system parameters for Ku-band for NR NTN

Agenda item:	7.11.4

Document for:	Approval

- Introduction

In RAN4#114bis meeting, A WF [1] and draft CR [2] on system parameters for Ku-band for NR NTN was agreed. But the note “Note: Same subcarrier spacing is assumed for SSB and data channels” in WID requires discussion.

In this contribution, we provide our views on system parameters for Ku-band for NR NTN.

- Discussion

As per the revised WID for Ku-band NTN [2], the objectives concerning CBWs and SCSs are shown as below:

- Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B. Define two sets of bands, one for FR1 numerology and the other for FR2 numerology, including the support of the following: - Ku band #1a and #1b as FR1-NTN supporting 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing, and 10MHz, 15MHz, 25MHz, 35MHz and 50MHz channel bandwidths with 15 kHz subcarrier spacing. - Note: Same subcarrier spacing is assumed for SSB and data channels - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing. - Note: Simultaneous operation of FR1-NTN and FR2-NTN numerology is not supported in the UE

As above, the Note “Note: Same subcarrier spacing is assumed for SSB and data channels” in WID shows that the SSB SCS for synchronization raster for band n247, n248 should be aligned with that SCS for data channels. So it is reasonable to add a note “NOTE 2:    SS Block SCS is same with SCS for data channels in this release.” for band n248 and n247 for applicable SS raster in Table 5.4.3.3-1 as below:

Table 5.4.3.3-1: Applicable SS raster entries per operating band (FR1-NTN)

| NTN satellite operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 15 kHz | Case A | 5429 – <1> – 5494 |
| n255 | 15 kHz | Case A | 3818 – <1> – 3892 |
|  | 30 kHz | Case B | 3824 – <1> – 3886 |
| n254 | 15 kHz | Case A | 6215 – <1> – 6244 |
|  | 30 kHz | Case C | 6218 – <1> – 6241 |
| n248 (Note 2) | 15 kHz | Case A | 12848 – <1> – 14268 |
|  | 30 kHz | Case C | 12850 – <1> – 14266 |
| n247 (Note 2) | 15 kHz | Case A | 12848 – <1> – 14268 |
|  | 30 kHz | Case C | 12850 – <1> – 14266 |
| NOTE 1: SS Block pattern is defined in clause 4.1 in 3GPP TS 38.213 [7]. NOTE 2: SS Block SCS is same with SCS for data channels in this release. |  |  |  |

So, we propose,

Proposal 1: Add a note “NOTE 2:    SS Block SCS is same with SCS for data channels.” for band n248 and n247 for applicable SS raster in Table 5.4.3.3-1 as below:

Table 5.4.3.3-1: Applicable SS raster entries per operating band (FR1-NTN)

| NTN satellite operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 15 kHz | Case A | 5429 – <1> – 5494 |
| n255 | 15 kHz | Case A | 3818 – <1> – 3892 |
|  | 30 kHz | Case B | 3824 – <1> – 3886 |
| n254 | 15 kHz | Case A | 6215 – <1> – 6244 |
|  | 30 kHz | Case C | 6218 – <1> – 6241 |
| n248 (Note 2) | 15 kHz | Case A | 12848 – <1> – 14268 |
|  | 30 kHz | Case C | 12850 – <1> – 14266 |
| n247 (Note 2) | 15 kHz | Case A | 12848 – <1> – 14268 |
|  | 30 kHz | Case C | 12850 – <1> – 14266 |
| NOTE 1: SS Block pattern is defined in clause 4.1 in 3GPP TS 38.213 [7]. NOTE 2: SS Block SCS is same with SCS for data channels in this release. |  |  |  |

- Conclusion

This contribution provides analysis on system parameters for Ku-band for NR NTN, and the following proposal is concluded as follows:

Proposal 1: Add a note “NOTE 2:    SS Block SCS is same with SCS for data channels.” for band n248 and n247 for applicable SS raster in Table 5.4.3.3-1 as below:

Table 5.4.3.3-1: Applicable SS raster entries per operating band (FR1-NTN)

| NTN satellite operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 15 kHz | Case A | 5429 – <1> – 5494 |
| n255 | 15 kHz | Case A | 3818 – <1> – 3892 |
|  | 30 kHz | Case B | 3824 – <1> – 3886 |
| n254 | 15 kHz | Case A | 6215 – <1> – 6244 |
|  | 30 kHz | Case C | 6218 – <1> – 6241 |
| n248 (Note 2) | 15 kHz | Case A | 12848 – <1> – 14268 |
|  | 30 kHz | Case C | 12850 – <1> – 14266 |
| n247 (Note 2) | 15 kHz | Case A | 12848 – <1> – 14268 |
|  | 30 kHz | Case C | 12850 – <1> – 14266 |
| NOTE 1: SS Block pattern is defined in clause 4.1 in 3GPP TS 38.213 [7]. NOTE 2: SS Block SCS is same with SCS for data channels in this release. |  |  |  |

- References

- R4-2504761, Way Forward for [114bis][313] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group), RAN4#114bis

- R4-2504721, Draft CR for Operating bands and system parameters, Eutelsat Group, RAN4#114bis

- RP-250799, Revised WID: Introduction of Ku bands for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, Airbus, Sharp, Mitre, JSAT, RAN#107

- R4-2502290, Way Forward for [114][313] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group), RAN4#114