3GPP TSG-RAN WG4 Meeting #115                                                                                             R4-2506034

Malta, MT, May. 19 –May. 23, 2025

Source: 	CATT

Title: 	Discussion on SAN conformance requirements for the NTN new S-band

Agenda item:	6.13.4

Document for:	Approval

- Introduction

In RAN4 #114-bis meeting, the discussion on SAN core requirement of NR NTN support new S-band was terminated. Therefore, the SAN conformance requirements for NR NTN support new S-band need to be discussed in the next meeting.

In this contribution, we would like to share some views on SAN conformance requirements for introducing the NTN new S-band.

- Discussion

The impact on SAN RF requirements for NR NTN support new S band has been discussed and concluded in the RAN4 #112-bis meeting, as shown below [1]:

#### Issue 1-3-5: Other SAN RF Requirements - Agreement - Reuse existing SAN 1-H and 1-O RF requirements for band n252

Meanwhile, the system parameter including SAN Operating Band, SAN Channel Bandwidths and SCS, SAN NR-ARFCN and SS raster have been discussed in several meetings, and all conclusions have been reached in RAN4 #114-bis meeting. All the modifications resulting from the introduction of the new S band are included in Clause 5.

In the SAN conformance spec TS 38.181 [2], where SAN RF requirements are band agnostic and system parameters refer to TS 38.108, as shown below:

# 5 Operating bands and channel arrangement For the NR operating bands specification, their channel bandwidth configurations, channel spacing and raster, as well as synchronization raster specification, refer to TS 38.108 [2], clause 5 and its relevant clauses.

Notably, the reginal requirement may be impact since the correspondingly FCC regulation applies on band n252. Otherwise, no additional impact on the SAN conformance specification has been identified.

Therefore, we believed that the introduction of new S band will not have impact on the SAN conformance specification.

Proposal 1: With the exception of regional requirement, the introduction of the n252 band will not have impact on SAN conformance specifications.

- Conclusion

This contribution provides our initial views on SAN conformance requirements for introduction of the NTN new S-band. The following proposals are concluded as follows:

Proposal 1: With the exception of regional requirement, the introduction of the n252 band will not have impact on SAN conformance specifications.

- References

- R4-2417206, WF on NTN and IoT-NTN bands, Inmarsat, RAN4 #112-bis