3GPP TSG-RAN WG4 Meeting #115                                                                                             R4-2506037

Malta, MT, May. 19 –May. 23, 2025

Source: 	CATT

Title: 	Discussion on RF requirements for NTN RedCap UE

Agenda item:	7.31.3.1

Document for:	Approval

- Introduction

In RAN4 #114-bis meeting, A WF [1] on NR NTN phase 3 UE RF requirements was agreed. However, there are still some remaining issues about NR NTN UE requirements need to be discussed in the next meeting.

In this contribution, we would like to share our views on NR NTN (e) RedCap UE requirements.

- Discussion

The agreement on NTN RedCap UE RF requirements was achieved during the previous meetings, as shown below:

## Issue 1-1: Output power enhancement for NR NTN PC3 Agreements: - If defined, additional output power enhancement will be optional for UE to support. - FFS on need of new UE capability - Evaluate potential to further enhance output power for RAN4#115: - Amount of output power enhancement [0.5 to 1 dB] - Less than 0.5 dB improvement is not specified - Initially consider all RB allocations and modulation orders - Consider both MPR reduction and MOP increase - FFS how to capture MOP increase in the specification - Choice between MPR reduction and MOP increase can be different for inner/outer allocations and modulation orders - Evaluate both HD-FDD and FD-FDD - Consider at least insertion loss impact - FFS whether to apply same enhancement to both HD-FDD and FD-FDD - Evaluate the option to support both rel-18 and rel-19 output power enhancement schemes simultaneously - FFS whether simultaneous support is specified - Consider whether PC3 or PC2 ACLR requirement should apply - FFS Applicability to PC3 non-RedCap NR NTN

In the previous meeting, RAN4 has agreed that Rel-18 power boosting feature could be applicable for NR NTN including RedCap. However, for Rel-19 (e) RedCap HD-FDD UE, the applicability of Rel-18 power boosting feature should be discussed as well. Referring to the discussion of RAN4 #114 meeting, the potential implementation of power boosting could be summarized step by step as below:

- Step 1: For NTN RedCap UE operating in HD-FDD mode, load Rel-18 power boosting capabilities powerBoosting-pi2BPSK-QPSK-r18 and/or powerBoosting-pi2BPSK-QPSK-Modified-r18. Now the NTN RedCap UE operating in HD-FDD mode could be boosted 1 dB for PC 3 or 0.5 dB for PC 2.

- Step 2: On top of step 1, load Rel-19 power boosting capabilities (still under discussion), which could boost X dB for PC 3 and Y dB for PC 2 with new schemes (such as reducing the IL).

After step 1 and step 2, for NTN HD-FDD RedCap UE, the final power boosting could be configured as:

- Enable Rel-18 power boosting only

- Enable Rel-19 power boosting only

- Enable both

So far, there does not appear to be any particular problem with the logic of power boosting configuration. However, since the Rel-19 power boosting schemes are still under discussion, it seems not appropriate to take it into account until the final X and Y values have been determined. In addition, Rel-19 power boosting scheme may not be band agnostic, which brings uncertainty for final conclusion.

Observation 1: The Rel-19 power boosting WI is still under discussion.

Considering that the Rel-18 power boosting feature is applicable for NR NTN HD-FDD RedCap UE, the supported configurations should be identified. From our perspective, at least step 1 could be supported, which ensures that NTN HD-FDD RedCap UE can benefit from the power boosting immediately.

Proposal 1: For NTN RedCap UE operating in HD-FDD mode, Rel-18 power boosting capabilities powerBoosting-pi2BPSK-QPSK-r18 and/or powerBoosting-pi2BPSK-QPSK-Modified-r18 could be supported.

However, we have to recognise that new solutions such as Rel-19 Power Boosting could further enhance the competitiveness of NTN HD-FDD RedCap UE. Therefore, the discussion on new solutions such as Rel-19 Power boosting should be welcomed, and we believed that it could be further discussed.

Observation 2: The discussion on new solutions such as Rel-19 Power boosting should be welcomed, and it could be further discussed.

The simultaneous operation with NR NTN and GNSS issue was discussed during the previous meeting, the background are shown as below:

### Issue 1-2: Simultaneous operation with NR NTN and GNSS Background: RAN4 sent LS to RAN2 in RAN4#113 in R4-2419898. RAN2 replied back in R4-2503119 as below, asking feedback on both duration and periodicity of the IDC free time to be applied to IDC TDM or autonomous denial solution Relevant RAN2 parameters include: - Proposals on IDC free duration - Option 1: values for GNSS-PositionFixDuration in TS 36.331: 1, 2, 3, 4, 5, 6, 7, 13, 19, 25, 31 seconds. (Vivo) - Option 1a: Leave out some large values as cold start for GNSS is not in scope (Huawei, Sony) - Option 2: 1s to 10s, with max 50% duty Tx duty cycle (Qualcomm) - Option 3: Extend IDC-TDM activeDuration-r18 to include 2000ms (Nokia) - Proposals on IDC periodicity - Option 1: few seconds to few tens of seconds (Qualcomm) - Option 2: Extend IDC-TMD cycleLength-r18 to 40960 ms if no RRM-impact (Nokia) - Other proposals - No IDC enhancement is needed (Ericsson) - Moderator: RAN4 specifically asked for IDC improvements in the LS in R4-2419898. - Recommended WF - It is known that different UE implementations have different needs, therefore signaling needs to have flexibility and there is no need to agree a single value to send. - Respond to RAN2: - IDC free duration can range from 1s to [13] s following the GNSS-PositionFixDuration in TS 36.331 - UEs may also be able to transmit with up to 50% duty cycle within the IDC free duration but Tx cannot be continuously on for long periods of time. - Periodicity may range from 2s to up to 40s. - Detailed specification changes are up to RAN2, but RAN4 recommends enabling the above both IDC-TDM and autonomous denials solutions.

Regarding to the reply LS from RAN2, there are two IDC related IEs as described by TS 38.331:

- AutonomousDenialParameters

- IDC-TDM-Assistance-r18

The autonomous denial allow UE to deny any transmission in a particular UL slot, which means that the UE could drop up to n30 (30 slots) of UL transmission. Due to the randomness of the UE movement, the autonomous denial approach gives the UE some flexibility in GNSS measurements. However, the disadvantage of this approach is obvious: it is a huge squandering of the time and frequency resources, and impacts the network scheduling. The network needs to anticipate the timing of UE launching GNSS measurements and schedule them accordingly, which greatly increases the complexity of network design.

Observation 3: The autonomous denial solution will squander the time-frequency resources and increase the complexity of network design.

The IDC TDM solution is a different design: it allows the UE to report the time taken to solve IDC problem, and the network could avoid scheduling UL transmission within the certain period requested by UE. The advantages of this approach are obvious: the UE can notify the network when a GNSS measurement is planned, leaving the entire measurement process under the full control of the network. This approach ensures that no resources are wasted and the complexity of network deployment is minimised.

Observation 4: The IDC TDM solution will not squander the time-frequency resources and will not increase the complexity of network design.

However, if the IDC TDM solution is extended, the power consumption for RedCap UE will become unmanageable. If IDC TDM solution applies, we prefer not to extend the existing IDC TDM framework. Therefore, we believed that to solve the issue of simultaneous operation with NR NTN and GNSS, the IDC TDM solution could be considered if there is no extension introduced.

Proposal 2: To solve the issue of simultaneous operation with NR NTN and GNSS, the IDC TDM solution could be considered if there is no extension introduced.

However, considering the previous discussions on RAN4 and the time budget, it seems difficult to simply choose between the above two options. If no agreement can be reached, we believe that this issue can also be discussed in the RRM session to explore if there is a potential solution.

Proposal 3: If no agreement can be reached in RF session, RAN4 may consider seeking the issue to be resolved from RRM perspective.

- Conclusion

This contribution provides our views on NR NTN (e) RedCap UE requirements. The following proposals are concluded as follows:

Observation 1: The Rel-19 power boosting WI is still under discussion.

Proposal 1: For NTN RedCap UE operating in HD-FDD mode, Rel-18 power boosting capabilities powerBoosting-pi2BPSK-QPSK-r18 and/or powerBoosting-pi2BPSK-QPSK-Modified-r18 could be supported.

Observation 2: The discussion on new solutions such as Rel-19 Power boosting should be welcomed, and it could be further discussed.

Observation 3: The autonomous denial solution will squander the time-frequency resources and increase the complexity of network design.

Proposal 2: To solve the issue of simultaneous operation with NR NTN and GNSS, the IDC TDM solution could be considered if there is no extension introduced.

Proposal 3: If no agreement can be reached in RF session, RAN4 may consider seeking the issue to be resolved from RRM perspective.

- References

- R4-2504708, Way Forward for [114-bis][310] NR_NTN_Ph3_UE_RF, Qualcomm, RAN4# 114-bis.