3GPP TSG-RAN WG4 Meeting #115                                                                                             R4-2506042

Malta, MT, May. 19 –May. 23, 2025

Source: 	CATT

Title: 	Discussion on UE Tx requirement for NTN support HPUE

Agenda item:	7.10.3.2.1

Document for:	Approval

- Introduction

In RAN4 #114-bis meeting, a WF [1] on UE Tx requirement for NTN support HPUE was agreed. However, there are still some remaining issues need to be discussed in the next meeting.

In this contribution, we would like to share some initial views on UE Tx requirement for NTN support HPUE for NR NTN HPUE.

- Discussion

The open issues on UE Tx requirements for NTN support HPUE was concluded during the previous meetings, as shown below:

# Topic #1: RF requirements ### Sub-topic 1-1: Power classes and UE types Issue 1-1-2: Power boosting option for PC2 Agreement: - FFS on the power boosting option for PC2, including - Whether or not and how to define power boosting for PC2 as proposed above. - The power boosting discussion not to impact the requirements discussion for PC2 without power boosting, e.g. MPR/AMPR, for PC2. ### Sub-topic 1-2 Tx requirements Issue 1-4: General Agreement: - As a baseline approach, existing Tx requirements from the TN specifications are re-used for the high-power NTN UE. Issue 1-5: MOP and tolerance Agreement: - Re-use NR TN TS 38.101-1 nominal MOP and +2/-3 tolerance for PC1 and PC1.5 for NR-NTN Non-Redcap HPUE. Issue 1-6: SEM Agreement: - Use agreed baseline approach for SEM. - Apply SEM from NR-NTN PC3 to other PCs for NR-NTN as baseline. - No need to further discuss the FCC 47 CFR part 25 specification in clause 202 in this work item in this release. Issue 1-7-1: PC2 and PC1.5 ACLR Agreement: - Re-use existing TN ACLR: - For 2Tx PC1.5: Reuse TN PC1.5 ACLR - For 1Tx and 2Tx PC2: Reuse TN PC2 ACLR Issue 1-7-2: PC1 ACLR Tentative agreement: - PC1 ACLR is [34] dBc - Companies to further evaluate this, and to be confirmed in next meeting. Issue 1-8-1: PC2/1.5 MPR Ad-hoc agreement: - Agree to re-use TS 38.101-1 MPR for PC2 and PC1.5 - TS 38.101-1 Table 6.2.2-2 applies to NR-NTN PC2 HPUE MPR with 1Tx - TS 38.101-1 Table 6.2D.2-1 applies to NR-NTN PC2 HPUE MPR with 2Tx - TS 38.101-1 Table 6.2D.2-3 applies to NR-NTN PC1.5 HPUE MPR with 2Tx Issue 1-8-2: PC1 MPR Way forward: - To collect MPR analysis and determine MPR for PC1 in next meeting. Issue 1-8-3: MPR/A-MPR simulation assumptions: Ad-hoc agreement: - For PC1, ACLR used in PA calibration should be aligned with ACLR requirements. Issue 1-9: A-MPR Ad-hoc agreements: - Discuss the A-MPR in the draft CRs in next meeting. ### Sub-topic 1-3 Rx requirements Issue 1-10: General discussion of RSD analysis Ad-hoc agreements: - In addition to Tx leakage to Rx, Rx non-linearity needs to be considered in RSD evaluation. Issue 1-11: General discussion of RSD of PC1.5 and PC1 Ad-hoc agreements: - Companies agree to bring more analysis for the RSD values for PC1.5/PC1 non-handheld UE. - Further check these in next meeting. Issue 1-13: RSD requirements for PC2 on n255 Ad-hoc agreements: - For n255 PC2: 5MHz; 10MHz; 15 MHz; 20 MHz. RSD@1Tx; [0.8]; [0.8]; [0.9]; [0.9]. RSD@2Tx; [1.6]; [1.6]; [1.7]; [1.7]. Issue 1-14: RSD requirements for PC1.5 and PC1. Way forward: - Encourage companies to further discuss the following proposals: - Option 1: For PC1.5 2Tx (MTK) Operating Band; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB). n256; 2.2; 2.2; 2.2; 2.2. n255; 3.2; 3.2; 3.3; 3.3. n254; 2.2; 2.2; 2.2. - Option 2: For PC1 1Tx (MTK) Operating Band; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB). n256; [3]; [3]; [3]; [3]. n254; [3]; [3]; [3]. Issue X: Proposal to allow power reduction beyond certain duty cycle Way forward: - Encourage companies to further discuss the following proposal: - Option 1: For NR NTN PC2 and PC1.5, and IoT NTN PC2, allow but not mandate UE to reduce it’s transmit power when 50% Tx duty cycle in a radio frame is exceeded. (Qualcomm)

In fact, applying the power boost scheme to the NTN (RedCap) UE has been discussed several times at RAN4 in parallel. The initial discussion was to consider whether to introduce power boosting for the PC 3 NTN RedCap UE to increase the transmit power. The discussion quickly concluded that the Rel-18 power boost could be applied to the NTN PC3 RedCap UE. At the same time, since the power boost of Rel-19 is still being discussed, there is no clear conclusion on how to introduce Rel-19 power boosting. In addition, there is still no conclusion on whether power boosting is introduced for normal UE, as shown below:

## Issue 1-1: Output power enhancement for NR NTN PC3 Agreements: - If defined, additional output power enhancement will be optional for UE to support. - FFS on need of new UE capability - Evaluate potential to further enhance output power for RAN4#115: - Amount of output power enhancement [0.5 to 1 dB] - Less than 0.5 dB improvement is not specified - Initially consider all RB allocations and modulation orders - Consider both MPR reduction and MOP increase - FFS how to capture MOP increase in the specification - Choice between MPR reduction and MOP increase can be different for inner/outer allocations and modulation orders - Evaluate both HD-FDD and FD-FDD - Consider at least insertion loss impact - FFS whether to apply same enhancement to both HD-FDD and FD-FDD - Evaluate the option to support both rel-18 and rel-19 output power enhancement schemes simultaneously - FFS whether simultaneous support is specified - Consider whether PC3 or PC2 ACLR requirement should apply - FFS Applicability to PC3 non-RedCap NR NTN

Observation 1: The Rel-18 power boosting is already applicable for NTN PC 3 RedCap UE.

However, as an agreement was reached at the last meeting that the discussion on power boosting would not impact the PC2 requirements discussion (such as MPR/AMPR). If the Rel-18 power boosting scheme is adopted, it can be foreseen that no impact on MPR/A-MPR. If the Rel-19 power boosting scheme is adopted, it can be expected that there will be no significant impact on MPR/A-MPR as well, since the escalation of Tx power is mainly due to the reduction of IL.

Proposal 1: RAN4 may consider applying the power boosting for PC 2 NTN HPUE.

For NTN HPUE, the results of the coexistence study show that there is some difference in the required ACIR values for NTN HPUE at 1.5 km isolation distance and 0 km isolation distance. For 1.5 km isolation distances, the ACIR is relatively relaxed, meaning that defining a less stringent ACLR requirement may not cause problems for NTN HPUE. However, for 0 km isolation distances, some companies have observed that ACIR requirements are still stringent.

The TN HPUE ACLR requirements for PC 3/2/1.5/1 are shown as below in TS 38.101-1:

Table 6.5.2.4.1-2: NR ACLR requirement Power class 1; Power class 1.5; Power class 2; Power class 3. NR ACLR; 37 dB; 31 dB; 31 dB; 30 dB. NOTE 1: Void.

For PC 1 NTN HPUE ACLR requirements, it was discussed at the last meeting and decided to reuse the TN HPUE ACLR requirements. The agreements focused on how to reuse the ACLR requirements for TN HPUE, and resulted in two approaches. Regarding to simulation results, it seems acceptable to reuse PC 1 TN HPUE ACLR requirements for PC 1 NTN HPUE. However, some companies believe that reusing the PC 1 TN HPUE ACLR (i.e., 37 dBm) may be difficult to achieve. Therefore, we believe that a certain amount value of compromise can be made, but it is better not to affect the coexistence performance.

In addition, with the increasing number of NTN HPUE users, the current simulation assumptions of the coexistence simulation may not be entirely realistic. Considering that ACLR requirement is the key point for coexistence, it might be wise to reserve some margin for ACLR requirements to ensure that network deployments are future-proof.

Therefore, it is acceptable to reuse PC 1 TN HPUE ACLR requirement for PC 1 NTN HPUE with some slightly relax.

Proposal 2: For PC 1 NTN HPUE ACLR requirement, the PC 1 TN HPUE ACLR requirement could be considered with certain relaxation.

- Conclusion

This contribution provides our views on how to conduct the UE RF requirements for NR NTN HPUE. The following proposals are concluded as follows:

Observation 1: The Rel-18 power boosting is already applicable for NTN PC 3 RedCap UE.

Proposal 1: RAN4 may consider applying the power boosting for PC 2 NTN HPUE.

Proposal 2: For PC 1 NTN HPUE ACLR requirement, the PC 1 TN HPUE ACLR requirement could be considered with certain relaxation.

- References

- R4-2505113, WF on UE RF requirements for NTN HPUE, Samsung, RAN4 #114-bis