3GPP TSG RAN WG4 Meeting #115		R4-2506060

St Julian, Malta, May 19th – 23rd , 2025

Title: 	Further discussion on MPR applicability for FR1 intra-band UL CA

Agenda Item:	7.1.2.2.2

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN4#114bis, discussion on MPR applicability for FR1 intra-band UL CA has been concluded [1], and a draftCR was endorsed, which consists of the following texts added into subclause 6.2A.2.1 on UE maximum output power reduction for Intra-band contiguous CA [2]:

For UE indicating [MPRenhancement-r19] supported and if single CC is activated for intra-band contiguous CA, the allowed MPR is specified in Table 6.2.2-1 for PC3 CA bandwidth classes B and C, Table 6.2D.2-1 for PC2 CA bandwidth classes B and C when TxD is indicated, Table 6.2.2-2 for PC2 CA bandwidth classes B and C when TxD is absent.

However, it seems still not clear enough for intra-band contiguous CA, hence we propose some further revision.

# 2 Discussion

If we look at the specs texts under sub-clause 6.2A.2.1:

---------------------------------

#### 6.2A.2.1	UE maximum output power reduction for Intra-band contiguous CA

For intra-band contiguous carrier aggregation the allowed Maximum Power Reduction (MPR) for the maximum output power in 6.2A.1.1-1 with contiguous RB allocation is specified in Table 6.2A.2.1-1 for UE power class 3 CA bandwidth classes B and C. The MPR with contiguous RB allocation is specified in Table 6.2A.2.1-1a for power class 2 CA bandwidth classes B and C when the signalling is absent for dualPA-Architecture IE, and for power class 2 CA bandwidth classe C when the signalling is indicated for dualPA-Architecture IE. The MPR with contiguous RB allocation is specified in Table 6.2A.2.1-1b for power class 2 CA bandwidth classes B and C with TxD supported.

In case the modulation format or waveform type is different on different component carriers then the requirement is set by rules applied to the waveform type (DFT-s-OFDM or CP-OFDM) and modulation order used in the configuration with the largest MPR..

Unless otherwise specified, pi/2 BPSK in following MPR tables refers to both variants of pi/2 BPSK referenced in clause 6.2.2 Table 6.2.2-1.

Table 6.2A.2.1-1: Contiguous RB  allocation for Power Class 3

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | outer | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 1.0 | 3.5 | 2.5 | 7 |
|  | QPSK | 1.0 | 3.5 | 2.5 | 7 |
|  | 16QAM | 1.5 | 3.5 | 2.5 | 7 |
|  | 64QAM | 3.0 | 4.0 | 5 | 7 |
|  | 256QAM | 5.5 | 6.0 | 7 | 7.5 |
| CP-OFDM | QPSK | 2.0 | 4.0 | 3.5 | 8 |
|  | 16QAM | 2.5 | 4.0 | 3.5 | 8 |
|  | 64QAM | 3.5 | 4.0 | 5 | 8 |
|  | 256QAM | 6.5 | 6.5 | 7 | 8 |

---------------------

It can be seen that MPR for contiguous RB allocation is specified according to bandwidth class, not channel bandwidth, and the endorsed texts follow similar wording, except to referring to MPR Table 6.2.2-1 which is based on channel bandwidth, therefore, the link between the single channel bandwidth and bandwidth class in the MPR table is not clear. This is in particular critical to readers outside 3GPP RAN4 working group.

Observation: With the endorsed texts on MPR applicability for FR1 intra-band contiguous UL CA where MPR table for single carrier is referred, the link between the single channel bandwidth and bandwidth class in the MPR table is not clear.

To clarify further, we propose the following change:

For UE indicating [MPRenhancement-r19] supported and if single CC is activated for intra-band contiguous CA, the allowed MPR is specified in Table 6.2.2-1 based on aggregatd channel bandwidth for PC3 CA bandwidth classes B and C, Table 6.2D.2-1 for PC2 CA bandwidth classes B and C when TxD is indicated, Table 6.2.2-2 for PC2 CA bandwidth classes B and C when TxD is absent.

Proposal: RAN4 to update the specs wording on MPR applicability for FR1 intra-band contiguous UL CA as: “For UE indicating [MPRenhancement-r19] supported and if single CC is activated for intra-band contiguous CA, the allowed MPR is specified in Table 6.2.2-1 based on aggregatd channel bandwidth for PC3 CA bandwidth classes B and C, Table 6.2D.2-1 for PC2 CA bandwidth classes B and C when TxD is indicated, Table 6.2.2-2 for PC2 CA bandwidth classes B and C when TxD is absent.”.

# 3 Conclusion

In this contribution, we have the following observation and proposal on MPR applicability for FR1 intra-band contiguous UL CA:

Observation: With the endorsed texts on MPR applicability for FR1 intra-band contiguous UL CA where MPR table for single carrier is referred, the link between the single channel bandwidth and bandwidth class in the MPR table is not clear.

Proposal: RAN4 to update the specs wording on MPR applicability for FR1 intra-band contiguous UL CA as: “For UE indicating [MPRenhancement-r19] supported and if single CC is activated for intra-band contiguous CA, the allowed MPR is specified in Table 6.2.2-1 based on aggregatd channel bandwidth for PC3 CA bandwidth classes B and C, Table 6.2D.2-1 for PC2 CA bandwidth classes B and C when TxD is indicated, Table 6.2.2-2 for PC2 CA bandwidth classes B and C when TxD is absent.”.

# Reference

[1] R4-2505099, “WF on power domain enhancements”, Huawei, HiSilicon

[2] R4-2505212, “DraftCR to 38.101-1 MPR applicability for FR1 intra-band contiguous UL CA”, ZTE et al.