3GPP TSG-RAN WG4 Meeting #115		                                                        R4-2506098

# Malta , MT, 19th – 23rd May, 2025

Title:	Discussion on RRM requirements for on-demand SSB SCell operation

Source:	CATT

Agenda Item:	7.27.3.1

Document for:	Discussion

- Introduction

In last RAN4 meeting, the RRM requirements for Rel-19 NES enhancement were discussed. The conclusions for OD-SSB SCell operation were captured in the approved WF [1] and the open issues can be found in the summary [2]. In this paper, we further discuss the potential RRM impact for on-demand SSB SCell operation and give our views.

- Discussion

Sub-topic 1-1: OD-SSB based deactivated SCell L3 measurement (Case 1)

OD-SSB based deactivated SCell measurement – Procedure

In the previous meeting, the agreements on the length of FMW were reached for deactivated SCell L3 measurement for case 1 [3]. For the expected minimum interval(T2, sage 4) between multiple FMWs, RAN4 agreed to define measurement requirement based on T2, but what is the definition of T2, the value of T2 and the related UE behaviour are still controversial [1][3], we will further give our understandings:

Sub-issue 1: The start point of T2

Sub-issue 2: The minimum interval(T2) between multiple FMWs

For the start point of T2 and the minimum interval(T2) between multiple FMWs, we think that it is better to discuss the two issues together, as it is meaningless to discuss the starting point or the value of T2 separately. The Wayforward for the two issues are copied as below [1]:

RAN4-bis: Sub-issue 1: The start point of T2 Wayforward The start point of T2 is - Option 1: - The time point of OD-SSB deactivation - Option 2: - The time point of Tend, i.e. the time point of OD-SSB activation plus Tuncertain +Tidentify. - Option 2a: - The time point of OD-SSB activation Sub-issue 2: The minimum interval(T2) between multiple FMWs Wayforward - Option 1: 5* measCycleSCell - Option 1a: 5*(max(measCycleSCell, DRX cycle) – OD-SSB periodicity) - Option 1b: 5*max (measCycleSCell, DRX cycles) - Option 2: Minimum measurement period defined in existing spec. - Option 3: The time period of known Scell condition but with OD-SSB periodicity for FR1

In our view, the following combination can be accepted for us:

Alt A:

- The start point of T2 is the time point of OD-SSB deactivation.

- The duration is defined based on [5]*max(measCycleSCell, DRX cycle).

Alt B:

- The start point of T2 is the time point of Tend, i.e. the time point of OD-SSB activation plus Tuncertain +Tidentify.

- The duration is defined based on [5]*max(measCycleSCell, DRX cycle).

Alt C:

- The start point of T2 is the time point of OD-SSB activation.

- The duration is defined based on Tend + [5]*max(measCycleSCell, DRX cycle).

Actually, we think that there is no difference between Alt B and Alt C. For Alt A, if the time point of OD-SSB deactivation is later, the time for UE to measure based on the retransmitted OD-SSB will also be later. If the time point of OD-SSB deactivation is earlier, then there is almost no difference between Alt A and Alt B/Alt C, and the whole procedure becomes dynamic. From this point of view, we slightly prefer Alt B or Alt C, as it is more clear and fixed.

Observation 1: There is no difference between Alt B and Alt C.

Observation 2: For Alt A, if the time point of OD-SSB deactivation is later, the time for UE to measure based on the retransmitted OD-SSB will also be later. If the time point of OD-SSB deactivation is earlier, then there is almost no difference between Alt A and Alt B/Alt C.

Proposal 1: Slightly prefer the following two combinations:

- Alt B:

- The start point of T2 is the time point of Tend, i.e. the time point of OD-SSB activation plus Tuncertain +Tidentify.

- The duration is defined based on [5]*max(measCycleSCell, DRX cycle).

- Alt C:

- The start point of T2 is the time point of OD-SSB activation.

- The duration is defined based on Tend + [5]*max(measCycleSCell, DRX cycle).

Sub-issue 3 –UE’s behaviour when OD-SSB further activation/reconfiguration within T2

For UE’s behaviour when OD-SSB further activation/reconfiguration within T2, the Wayforward is copied as below [2]:

Wayforward - Option 1: The UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement. - Option 2: The UE is assumed to perform fast mode measurement without cell identification period. - Option 3: No requirement for UE within T2.

Considering this issue has been discussed during many meetings, and RAN4 has agreed to define measurement requirement based on T2 in RAN4#114 meeting, so option 1 can be a compromise for us, that is, the UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement.

Observation 3: RAN4 has agreed to define measurement requirement based on T2 in RAN4#114 meeting.

Proposal 2: The UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement.

Sub-topic 1-2: OD-SSB based deactivated SCell L3 measurement (Case 2)

Requirement for RAN1 Alt Time-C2

RAN1 have sent the LS on time location of on-demand SSB for SCell to RAN4 in RAN1#120 meeting, the specific content is as follows [4]:

Agreement Regarding the relation in terms of time location between the always-on SSB and on-demand SSB, - For the case when the center frequency locations of always-on SSB and on-demand SSB are same, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - From RAN1 perspective, - Alt Time-C1: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification). - Alt Time-C2: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported. - It is up to RAN4 to define requirements, if any, corresponding to both or either of Alt Time-C1 or Alt Time-C2 - At least the following is supported: PBCH payload for the same SSB index (other than SFN index, half frame index) is the same for AO-SSB and OD-SSB - FFS: Whether half frame index is the same or different for AO-SSB and OD-SSB - For the case when the center frequency locations of always-on SSB and on-demand SSB are different, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - UE assumes that frequency resources of always-on SSB are not overlapped with those of on-demand SSB in frequency domain. - AO-SSB and OD-SSB are located in the same BWP - FFS: PBCH payload for the same SSB index (other than SFN index, half frame index) should be the same for AO-SSB and OD-SSB - NOTE: AO-SSB periodicity is not adapted

In the last meeting, RAN4 discussed whether to define the requirements for Alt Time-C2 defined in RAN1, but there was still no consensus for this issue. The two options are copied as below [2]:

Agreement - Option 1: Define the requirement based on OD-SSB periodicity. - Option 2: Not to define requirement.

For Alt Time-C2, if the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern, we think that RAN4 also can define the related requirements for this case. In our view, RAN1 has agreed this scenario, if RAN4 doesn’t define the related requirement, the scenario is limited. Moreover, defining the relevant requirements is not very complicated, and a simple method is that UE follows the OD-SSB periodicity to perform measurement in FMW.

Also, some companies pointed that the scheduling restriction for AO-SSB is not clear enough. In our view, there is no need to consider the scheduling restriction for AO-SSB.

Proposal 3: RAN4 to define requirements for Alt Time-C2 defined by RAN1.

- UE follows the OD-SSB periodicity to perform measurement in FMW.

Proposal 4: There is no need to consider the scheduling restriction for AO-SSB.

Requirement for scenario 3 (OD-SSB and always-on SSB are within different frequencies)

For scenario 3 (OD-SSB and always-on SSB are within different frequencies), the agreements reached in RAN1#120 meeting is copied as follows:

Agreement Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - AO-SSB and OD-SSB are located in the same BWP - FFS: Additional conditions - Subject to separate UE capability - Note: UE is not required to measure both AO-SSB and OD-SSB

RAN4 discussed whether to define the related requirements for the above scenario, but there was still no consensus for this issue. The two options are copied as below [1]:

Agreement RAN4 to first discuss the RAN4 impact based on RAN2 LS R2-2503015. - Option 1: RAN4 to define requirement for different frequencies - Option 2: RAN4 not to define requirements for different frequencies

In RAN2 #129-bis, there is a working assumption in the RAN2 LS R2-2503015 [5], which is as following:

Working assumption: - When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB.

According to the above, firstly RAN4 need to discuss what is the meaning of ‘UE is not required to measure both AO-SSB and OD-SSB’, which directly affects how RAN4 defines the relevant requirements.

In our view, even though there are two servingCellMOs, but only one servingCellMO is in effect at the same time, which will also make the UE measurement behavior clearer. If RAN4 can reach the above consensus, we think that it is feasible to define requirement for this case, and then we can further discuss how to define the requirements.

Proposal 5: RAN4 need to discuss what is the meaning of ‘UE is not required to measure both AO-SSB and OD-SSB’, which directly affects how RAN4 defines the relevant requirements.

Observation 4: Even though there are two servingCellMOs, but only one servingCellMO is in effect at the same time, which will also make the UE measurement behavior clearer.

Proposal 6: If RAN4 can reach consensus on the understanding of ‘UE is not required to measure both AO-SSB and OD-SSB’, we think that it is feasible to define requirement for this case.

Sub-topic 1-3: SCell activation part

The known cell condition

In RAN4#112 meeting, it was agreed that RAN4 focuses on OD-SSB based SCell activation known and unknown requirement for both Case 1 and Case 2 and not specifically differentiate Scenario 2/2A. And the known cell condition principle was also agreed as below in RAN4#112bis meeting and RAN4#113 meeting:

RAN4#112bis： Issue 1-4-3: known cell condition design – principle Agreement: - The legacy known condition principle can be reused, which is defined based on if the UE has sent a valid measurement report during a ‘period’ before the reception of the SCell activation command. - When OD-SSB transmission is before/together with the SCell activation command(Scenario #2/Scenario #2A), UE follows the principles of legacy known/unknown SCell activation requirement once the known/unknown condition is met. - FFS how known/unknown condition is defined i.e. based on always-on SSB and/or OD-SSB - Other principles are not precluded. RAN4#113： Issue 1-4-1: The known cell condition – measurement reporting Agreement: - In case 1, known/unknown condition should be defined based on only OD-SSB. - In case 2, known/unknown condition should be defined based on either always-on SSB or OD-SSB. - Note 1: whether the always-on SSB and OD-SSB can be combined is discussed in another issue. - Note 2: the agreement is valid at least for the scenario with the same frequency for always-on SSB and OD-SSB. - Note 3: RAN4 to further clarify the relation for the SSB measured for known cell condition and the SSB used in SCell activation procedure.

For Case 2, RAN4 agreed that known/unknown condition should be defined based on either always-on SSB or OD-SSB, but RAN4 also need to further clarify the relation for the SSB measured for known cell condition and the SSB used in SCell activation procedure.

In our view, since the measurement before SCell activation is to be used during SCell activation procedure, the measured/reported SSB and the SSB used in SCell activation procedure should be with the spatial relation no matter if the known condition is determined based on always-on SSB and/or OD-SSB. Based on the principle of legacy known/unknown condition, the known cell condition in case 2 can be updated as following:

- UE has sent a valid L3-RSRP measurement report with SSB index for the SCell being activated and

- The SSB used in SCell activation which has the same SSB index as the SSB measured remains detectable

- The measured/reported SSB has the same spatial relation as the SSB used in Scell activation.

In addition, RAN1 has considered the case of different frequency location between the always-on SSB and on-demand SSB, we think that the above condition also apply to this case (corresponding to scenario 3 defined in RAN4). In RAN1#120 meeting, the agreement on transmit power of OD-SSB is as follows:

Agreement: - Downlink transmit power of on-demand SSB (i.e., od-ss-PBCH-BlockPower) - For Case #2, this parameter is absent and downlink transmit power of on-demand SSB is the same as that of always-on SSB.

According to the above, we don’t think AGC adjustment time is needed in the known SCell activation delay requirement even though OD-SSB and AO-SSB are transmitted on the different frequency.

In addition, for the period of known condition, we agree to reuse the legacy known/unknown cell condition, that is max(5*measCycleSCell, 5*DRX cycles) in FR1 or 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4  in FR2.

Proposal 7: The known cell condition in case 2 can be updated as following:

- UE has sent a valid L3-RSRP measurement report with SSB index for the SCell being activated and,

- The SSB used in SCell activation which has the same SSB index as the SSB measured remains detectable

- The measured/reported SSB has the same spatial relation as the SSB used in Scell activation.

Proposal 8: RAN4 to reuse the legacy known/unknown cell condition period for FR1 and FR2, i.e. max(5*measCycleSCell, 5*DRX cycles) in FR1 or 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4  in FR2.

Sub-topic 1-4: OD-SSB general issues

Additional Processing time for OD-SSB reception – value

In the last meeting, RAN4 discussed the value of additional processing time for OD-SSB reception and agreed to introduce different values with UE capability. The agreement is copied as below [1]:

Agreement UE applies the additional processing timing only in deactivated SCell measurement. - Note: The additional UE processing timing is not applied to SCell activation procedure. The additional processing timing is T_processing, the value of T_processing: - Introduce different values with UE capability: [2 or 3]ms and 5ms.

For the specific values with UE capability, 2ms and 3ms were still in the square bracket. Considering the time for SSB processing defined in LTM is 2ms, which defined as following in clause 6.3.1.3 [6]. Therefore, we believe that the UE can process SSB in 2ms and we suggest introducing 2ms and 5ms with UE capability for additional processing timing.

TRS-proc is the time for SSB processing. Tfirst-RS = 0 and TRS-proc= 0 under the following conditions: - The target TCI state indicated in the LTM cell switch command is in the serving cell active TCI state list, or - The UE is configured with LTM L1 intra- and/or inter-frequency measurements for the target cell, and - The target TCI state in the cell switch command is in the LTM candidate cell active TCI state list, and - the time between receiving the LTM candidate cell TCI state activation MAC-CE and the cell switch command is at least TCI state activation delay stated in section 8.25.3 and - the time between receiving the LTM candidate cell TCI state activation MAC-CE and the cell switch command is not more than TCI state activation delay stated in section 8.25.3 + 160 ms, or - the measurement period of the SSB associated to target TCI state is not larger than 160 ms after the LTM candidate cell TCI state activation MAC-CE is received, or - The target cell is an FR1 cell, and the UE is not configured with LTM L1 intra- and/or inter-frequency measurements for the target cell, and - The target TCI state in the cell switch command is in the LTM candidate cell active TCI state list, and - the time between receiving the LTM candidate cell TCI state activation MAC-CE and the cell switch command is at least TCI state activation delay stated in section 8. 25.3, and not more than - TCI state activation delay stated in section 8. 25.3 + 480 ms for an intra-frequency target cell, or NOTE: longer L3 measurement delay may be expected for 480 ms after TCI state activation delay stated in section 8.25.3. - TCI state activation delay stated in section 8. 25.3 + 160 ms for an inter-frequency target cell, or - The time between the latest PDCCH ordered RACH preamble transmission on the target cell and the cell switch command is not more than [160 ms]. Otherwise, Tfirst-RS is the time to the first SSB transmission on the target cell after Tcmd + TLTM-RRC-processing + TLTM-processing. TRS-proc = 2 ms.

Proposal 9: For the additional processing timing, the value of T_processing:

- RAN4 to introduce different values with UE capability: 2 ms and 5ms.

OD-SSB based multiple SCells activation

In the last meeting, the Wayforward for OD-SSB based multiple SCells activation is copied as follows [1]:

Wayforward - Option 1: - RAN4 to define OD-SSB based multiple SCells activation delay requirements. - The limitation is needed - It doesn’t impact the WI’s completion. - Option 2: - Not define multiple OD-SSB carriers requirement in this release.

For this issue, option 1 can be considered if there is some limitation for OD-SSB based multiple SCells activation. Considering that the workload and time budget, if the most of current measurement mechanisms can be reused, then RAN4 can define OD-SSB based multiple SCells activation delay requirements. In our view, at least for deactivated SCell measurement, the requirements only apply to the case of none-overlapping among multiple OD-SSBs frequency layers, and the current measurement mechanisms can be reused.

Proposal 10: RAN4 to define OD-SSB based multiple SCells activation delay requirements.

- At least for deactivated SCell measurement, the requirements only apply to the case of non-overlapping among multiple OD-SSBs frequency layers, and the current measurement mechanisms can be reused.

- Conclusion

In this paper, we have some further discussions on on-demand SSB SCell operation, and the following observations and proposals are given:

Observation 1: There is no difference between Alt B and Alt C.

Observation 2: For Alt A, if the time point of OD-SSB deactivation is later, the time for UE to measure based on the retransmitted OD-SSB will also be later. If the time point of OD-SSB deactivation is earlier, then there is almost no difference between Alt A and Alt B/Alt C.

Proposal 1: Slightly prefer the following two combinations:

- Alt B:

- The start point of T2 is the time point of Tend, i.e. the time point of OD-SSB activation plus Tuncertain +Tidentify.

- The duration is defined based on [5]*max(measCycleSCell, DRX cycle).

- Alt C:

- The start point of T2 is the time point of OD-SSB activation.

- The duration is defined based on Tend + [5]*max(measCycleSCell, DRX cycle).

Observation 3: RAN4 has agreed to define measurement requirement based on T2 in RAN4#114 meeting.

Proposal 2: The UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement.

Proposal 3: RAN4 to define requirements for Alt Time-C2 defined by RAN1.

- UE follows the OD-SSB periodicity to perform measurement in FMW.

Proposal 4: There is no need to consider the scheduling restriction for AO-SSB.

Proposal 5: RAN4 need to discuss what is the meaning of ‘UE is not required to measure both AO-SSB and OD-SSB’, which directly affects how RAN4 defines the relevant requirements.

Observation 4: Even though there are two servingCellMOs, but only one servingCellMO is in effect at the same time, which will also make the UE measurement behavior clearer.

Proposal 6: If RAN4 can reach consensus on the understanding of ‘UE is not required to measure both AO-SSB and OD-SSB’, we think that it is feasible to define requirement for this case.

Proposal 7: The known cell condition in case 2 can be updated as following:

- UE has sent a valid L3-RSRP measurement report with SSB index for the SCell being activated and,

- The SSB used in SCell activation which has the same SSB index as the SSB measured remains detectable

- The measured/reported SSB has the same spatial relation as the SSB used in Scell activation.

Proposal 8: RAN4 to reuse the legacy known/unknown cell condition period for FR1 and FR2, i.e. max(5*measCycleSCell, 5*DRX cycles) in FR1 or 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4  in FR2.

Proposal 9: For the additional processing timing, the value of T_processing:

- RAN4 to introduce different values with UE capability: 2 ms and 5ms.

Proposal 10: RAN4 to define OD-SSB based multiple SCells activation delay requirements.

- At least for deactivated SCell measurement, the requirements only apply to the case of non-overlapping among multiple OD-SSBs frequency layers, and the current measurement mechanisms can be reused.

- Reference

- R4-2504912, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson, RAN4#114-bis

- R4-2503625, Topic summary for [114][223] Netw_Energy_NR_enh_Part1, Ericsson, RAN4#114-bis

- R4-2502598, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson, RAN4#114

- R1-2501633, LS on time location of on-demand SSB for SCell

- R2-2503015, LS on on-demand SSB

- 3GPP TS 38.133, NR Requirements for support of radio resource management