3GPP TSG-RAN WG4 Meeting #115	R4-2506105

# Malta , MT, 19th – 23rd May, 2025

Title: 	Discussion on CSI-RS based L1 measurement for Rel-19 LTM enhancements

Source: 	CATT

Agenda item:	7.29.3.2

Document for:	Discussion

- Introduction

In the last meeting, the WF on RRM requirements for NR_Mob_Ph4_Part1 was approved [1]. This document will discuss the RRM issues about CSI-RS based L1 measurement for LTM, and give our understanding and proposals.

- Discussion

- 2.1 Whether to keep or remove SSB based L1 measurement for FR2-1 in step 2

For the whole CSI-RS based L1 measurement procedure, RAN4 has agreed the entire framework in RAN4#113 meeting and RAN4 agreed that whether step 2 can be skipped for neighboring cell or not will based on new UE capability reporting, but there was still FFS for whether step 2 can be skipped for serving cell or not. The agreement is copied as below [1]:

RAN4#113: <Agreement> - Agree the following measurement procedure for CSI-RS based LTM candidate cell measurements: - Step 1: SSB based L3 measurements and reports at least before LTM configuration. - Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell - Further discuss whether Step 2 can be skipped under certain conditions, or based on UE capability or based on network RRC configuration. FFS the details. - Note: Step 2 is removed for FR1. - Step 3: CSI-RS based L1 measurements on the same candidate cell - Depending on whether step 2 is skipped, the requirements in step 3 may be different. - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1 or 2. - [Note: For FR1, the CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.] RAN4#114-bis: Agreement: - Whether step 2 can be skipped for neighboring cell or not based on new UE capability reporting. - Further discuss whether step 2 can be skipped for serving cell or not.

The controversy on the above issue in the last meeting lies in whether step 2 can be skipped for serving cell or not. Since the above agreement for neighboring cell has been a compromise, and considering that the candidate cell may also be a serving cell for LTM and skipping step 2 will introduce a new measurement behavior for serving cell, we also agree that using two different measurement procedures for the same cell is too complex. Therefore, always to keep step 2 for serving cell is also acceptable for us.

Observation 1: Using two different measurement procedures for the same cell is too complex.

Proposal 1: Always to keep step 2 for serving cell is acceptable for us.

- 2.2 CSI-RS based intra-frequency/inter-frequency measurement definition

In the last meeting, the definition of CSI-RS based intra-frequency/inter-frequency measurement hasn’t been discussed, and the candidate options are copied as below [1]:

Candidate options: - Option 1: Apple, CMCC A measurement is defined as a CSI-RS based intra-frequency measurement provided that: - the SCS of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of neighbor cell configured for L1 measurement is the same as the CP type of active DL BWP, and - It is applied for SCS = 60KHz - the center frequency of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the center frequency of any of the CSI-RS resources within DL BWP (if configured). - Option 2: MediaTek, ZTE, Ericsson, Nokia, Qualcomm A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and - It is applied for SCS = 60KHz - the CSI-RS resource of the neighbour cell configured for L1 measurement is included within the active DL BWP. - at least 48 RBs of the CSI-RS resource, needs to be confined within the active DL BWP. - Option 3: Huawei If L1 CSI-RS RSRP measurement with gap is not supported, either option of intra-frequency CSI-RS based L1 RSRP measurement definition has no big difference. If CSI-RS measurement with gap is supported, to define measurement requirements, the “CSI-RS frequency layer” concept will be touched. Then the intra-frequency definition based on center frequency is more preferred. - Option 4: vivo - A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and - the CSI-RS resources of both the neighbour cell and serving cell configured for L1 measurement is included within the active DL BWP. - at least 48 RBs of the CSI-RS resource are confined within the active DL BWP.

In the last meeting, RAN4 has agreed not to define the requirements for L1 measurement with gap for CSI-RS based L1 measurement on neighbor cell and the related agreements are copied as follows [1]:

- Issue 4-3: Definition of “inside/outside active BWP” Agreement: - RRM requirements (delay and accuracy) apply under the following condition: - At least 48 RBs of the CSI-RS configured for measurement is within the active BWP. - Issue 4-6: measurement GAP and RTD assumption Agreement: - From RAN4 perspective, - Not to define RAN4 requirements for L1 measurement with gap for CSI-RS based L1 measurement on neighbor cell. - Not to define RAN4 requirements for RTD > CP.

According to the above, RRM requirements (delay and accuracy) only apply under the condition of at least 48 RBs of the CSI-RS configured for measurement within the active BWP and CSI-RS based L1-RSRP measurement with gap has been not supported from RAN4 requirement perspective. Option 1 is to follow the principle of L3 measurements, and option 2 cares about whether CSI-RS is configured within the active BWP and whether gap is used.

Since what option 2 concerned has been solved, if companies want to keep the consistency of the definition as L3 measurement and make it clearer for all of WGs, option 1 is slightly preferred. Anyway, it is just a term for measurements, there is no much difference between intra-frequency and inter-frequency CSI-RS based L1 RSRP measurement from RAN4 requirements perspective, both are ok for us.

Proposal 2: If companies want to keep the consistency of the definition as L3 measurement and make it clearer for all of WGs, option 1 is slightly preferred.

Observation 2: It is just a term for measurements, there is no much difference between intra-frequency and inter-frequency CSI-RS based L1 RSRP measurement from RAN4 requirements perspective.

- Conclusion

This document discussed the issues for CSI-RS based L1 measurement for LTM in Rel-19 and the following proposals and observations are presented:

Observation 1: Using two different measurement procedures for the same cell is too complex.

Proposal 1: Always to keep step 2 for serving cell is acceptable for us.

Proposal 2: If companies want to keep the consistency of the definition as L3 measurement and make it clearer for all of WGs, option 1 is slightly preferred.

Observation 2: It is just a term for measurements, there is no much difference between intra-frequency and inter-frequency CSI-RS based L1 RSRP measurement from RAN4 requirements perspective.

- Reference

[1]     R4-2504965, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple