3GPP TSG-RAN WG4 Meeting #115	R4-2506115

Malta, MT, May 19th – May 23rd, 2025

Source:	Nokia

Title:	Fundamental RX requirements for LP-WUR

Agenda item:	7.28.3.2

Document for:	Discussion

Introduction

In this document we present our views on open issues regarding REFSENS, ACS, and ASCS requirements. Following agreements were made in the last RAN4 meeting [1].

# Topic #2: REFSENS, ASCS and ACS requirements ### Sub-topic 2-1 Sub-topic 2-1 FR1 SNR, NF and IM for REFSENS requirements Issue 2-1-1: SNR for OOK-based LP-WUS - Proposals - Proposal 1: Conclude SNR for OOK-based LR. (Moderator) FR1 OOK-based receiver; SNR; RM coding; Length of information bits; ADC; CRC; Note; REFSENS. Huawei; -4dB for both; 8/16/32; 5bit?; 4-bit; No; SNR: -2.15 dB (8 bit), -4.63 dB (16 bit), -5.94 dB (32 bit). Sony; -7 to -3dB. Spread trum; -3; SNR last meeting. vivo; -6.5dB; 16/20; 5bit; 4-bit; No; SNR: -6.5dB for 16bit RM coding without Te or 20bit RM coding with Te. CATT; -3.3dB; SNR last meeting; -96.3/-93.3. CMCC; -2.5dB; SNR last meeting. ZTE; -1.5dB; in TS38.212 Clause 5.3.3.3; 5bit; 4-bit; No; SNR -1.5dB (without ML), -7.5dB (with ML); 9dB higher than MR. Nokia; >RAN1 SNR; SNR last meeting. E///; -1.5 dB; SNR last meeting. QC; -4.0 dB; SNR as side condition. Assume RAN1: NF + IM = 17.0 dB, and 14.0 dB?. LGE; -7 to 8 dB; 4bit; SNR last meeting. Agreements: - Companies are encouraged to provide the simulation results with the assumption of the RM coding of 8bit, 16bit and 32bit to align simulations in RAN4. It is targeted to conclude SNR value for envelope detection and OFDM sequence detection next RAN4 meeting. Issue 2-1-5: How to map two LP-WUR types with UE capability Agreement: - RAN4 define two LP-WUR types (Type 1 and Type 2), no need 1-to-1 mapping between waveform and LP-WUR type. - Requirement applicability should be clearly defined. - RAN4 need to further discuss how to design the RMC for each type, how to ensure the performance of each type properly. - Sub-topic 2-2 FR2 SNR, NF and IM for REFSENS requirements Issue 2-2-2: Diversity gain value for FR2 LP-WUR Agreement: - [-3]dB diversity gain is assumed for FR2 LP-WUR REFSENS derivation Issue 2-2-3: NF+IM for FR2 LP-WUR Agreements: - RAN4 further discuss FR2 LPWUR, NF+IM value. Issue 2-2-4: Spherical coverage for FR2 LP-WUR Agreement: - The coverage matching requires the same spherical coverage percentile of WUR and MR for different FR2 power classes. - FFS on whether to skip the test of FR2 LP-WUR for spherical coverage. - Sub-topic 2-3 ASCS simulation and requirements Issue 2-3-1: ASCS requirements for OOK-based receiver Agreement: - For Type1 UE requirements, which are derived based on the assumption that demodulation of OOK content of LP-WUS is targeted - 1 RB guard for ASCS and 0.5dB de-sens - For Type2 UE requirements, which are derived based on the assumption that demodulation of OFDM content of LP-WUS is targeted - 0 RB guard for ASCS and 0 dB de-sens - For Type 1 and Type2 UE are not limited to specific LP-WUS waveform. - FFS how to define test case and requirement applicability for Type 1 and Type 2 - FFS how type 1 and type 2 capability related to RAN1 UE feature - A UE can only support either Type 1 or Type 2 but not both for LP-WUR Issue 2-3-3: Channel bandwidth for ASCS Test case Agreements: - ASCS requirement should be applied to all supported UE CBW. Conformance test case and potential down-selection will be defined by RAN5. - For conformance testing, it is expected that not all CBW need to be tested. Issue 2-3-4: whether high in-band signal (both LP-WUS and NR) case is considered for alternative ASCS requirements Agreements: - FFS For ASCS, whether additional test case that the wanted signal is 14dB higher than LP-WUS REFSENS, i.e., REFSENSLP_WUR + 14 dBm, should be considered or not. - Sub-topic 2-4 ACS simulation and requirements Issue 2-4-1: Baseline ACS requirements (same interference) and required guard RB for SET-1 (worse NF) LP-WUR - Proposals - Proposal 1: check the following input from companies and converge the range for set-1 LR. (Moderator) vivo; CMCC; ZTE; Huawei; Nokia; E///; Sony; QC. ACS (dB); 28; 22-24; 24; 11 (Note1); 22; Note 2; Higher wanted signal (Note3); Higher wanted signal Note4. Guard RB (as test parameter, not requirements); 1RB; 1RB; 1RB; Less RB; 2RBs. Note 1: For LP-WUR type aimed at moderate power consumption, the guard RBs could be derived with better phase noise profile and the interferer level could similar to that of MR. Note 2: The same set of the inteference signal and wanted signal defined for both two set of the LP-WUS requirments. Further discuss the number of guard RB for two set of requirment. Note 3: A higher wanted signal can be specified for OOK type LP-WUR ACS requirement to balance the power consumption and interference mitigation capability. Note 4: LPWUS ACS is specified as (assuming higher wanted signal is higher than refsens by 14 dB). DgMR is the diversity gain used for legacy PDSCH REFSENS derivation ACSLPWUS= NFlegacy-NFLPWUS+10logCBWBWLPWUS+ACSlegacy-DgMR - Proposal 2: Whether consider a higher the wanted signal for set-1 ACS requirements, e.g., LP REFSENS+ >14dB. (Moderator) WF: - Encourage companies to update the simulation analysis based on latest agreements. Target to conclude ACS requirements and test parameters next meeting. Issue 2-4-2: Whether adopt the same wanted power level for Set-2 LR? Agreement: - The same set of the interference signal level defined for both two types of the LP-WUS requirements. Issue 2-4-4: Case 1 ACS test parameter Agreements: Specify case 1 and case 2 ACS test parameters for Type 1 and Type 2. - Define interferer as BWinterferer of 5 MHz.

Discussion

Timing error impact

During the last meeting it was raised that there is a need to relax the requirements based on the residual timing error as noted in the RAN1 Release 18 SI TR. It was mentioned that there is an impact on the effective SNR based on residual timing error. However, inspecting the TR results in detail we reached on the following observations.

- There is no performance impact with a timing error of up to 4us irrespective of modulation type.

- The simulations to measure the SNR impact were done with TDLC-300ns channel and an impact of 2dB was found for timing error greater than 4 us.

Regarding timing error estimation, we did simulations with AWGN channel for different values of M and L, and following are the results.

Table 1 Timing error estimate in AWGN channel.

| M | 1 | 1 | 2 | 2 | 4 | 4 |
| --- | --- | --- | --- | --- | --- | --- |
| L | 6 | 8 | 8 | 16 | 16 | 32 |
| Timing Accuracy (us) | 2.5 | 1 | 0.3 | 0 | 0.1 | 0 |

From the simulated results, it can be concluded that for the current values of modulation being considered, there is no impact of timing error on the performance of LR and there should not be any relaxation in requirements based on it.

The results in TR 38.869 show no performance degradation with timing error under 4us irrespective of modulation type.

Simulated results show that the observable timing error is less than 2.5 us, for all the modulations currently being considered.

Agree to have no impact on RAN4 RF requirements because of timing error.

SNR value for OOK

Based on the RAN1 agreement to use the RM coding, link-level simulations were performed in the AWGN channel for OOK4 modulation. Given that for larger than 2 information bits, RAN1 has decided to have a code of length 31 bits, so only those simulations were performed. Based on the 1% MDR criterion, the required SNR according to our simulations is -8.0 dB. For the OFDM sequence decoding, since there is no repetition of sequence and there is no RM encoding, the required SNR is much worse at -3.5 dB for AWGN channel.

Agree to use -8.0 dB as SNR for type 1 LR.

Agree to use -3.5 dB SNR for type 2 LR.

ASCS requirement

For legacy UEs, the ACS is tested at two different power levels, one at REFSENS + 14 dB and another at a fixed level of -56.5 dBm. This enables testing filter suppression performance across both optimal and worst-case dynamic range scenarios. In our opinion this power level should be significantly higher than the REFSENS level in order to eliminate the impact of noise. Thus, we propose to have an additional ASCS requirement test with a signal level of REFSENSLR + 14 dB.

Agree to have an additional ASCS requirement validation with power of LP_WUS and NR signal at least 14 dB higher than LP-WUS REFSENS.

Conclusion

The following Observations and Proposals were made:

Observation 1: The results in TR 38.869 show no performance degradation with timing error under 4us irrespective of modulation type.

Observation 2: Simulated results show that the observable timing error is less than 2.5 us, for all the modulations currently being considered.

Proposal 1: Agree to have no impact on RAN4 RF requirements because of timing error.

Proposal 2: Agree to use -8.0 dB as SNR for type 1 LR.

Proposal 3: Agree to use -3.5 dB SNR for type 2 LR.

Proposal 4: Agree to have an additional ASCS requirement validation with power of LP_WUS and NR signal at least 14 dB higher than LP-WUS REFSENS.

- References

- R4-2505234, WF for [114bis][131] NR_LPWUS_UERF, Vivo, RAN4#114bis, Wuhan, China, 7th – 11th April, 2025.

- R4-2503028, WF on UE RF requirements for LP-WUS, Vivo, RAN4#114, Athens, Greece, 17th – 21st Feb, 2025.

- R4-2415707, Fundamental RX requirements for LP-WUR, Nokia, RAN4#112bis, Hefei, China, 14th – 18th October, 2024.