3GPP TSG-RAN4 Meeting #115	R4-2506127

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 5497 | rev | - | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.133 on RMSI CORESET Reference Channel for FDD with SCS=15KHz for Less than 5MHz UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_lessthan_5MHz_BW-Perf |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR provides a correction to RMSI CORESET Reference Channel for FDD with SCS=15KHz table Table A.3.1.2.1-1. The original Note 7 is not correctly referring to the tables / indexes in TS 38.213 for CR.1.2 FDD, CR.1.3 FDD and CR.1.4 FDD. The correct indexes and tables are: CR.1.1 FDD - The configuration of set of resource blocks, slot symbols, RB offset and REG to CCE mapping of control resource set for Type0-PDCCH search space corresponds to index 0 in table 13-1 in TS 38.213 [3] CR.1.2 FDD - The configuration of set of resource blocks, slot symbols, RB offset and REG to CCE mapping of control resource set for Type0-PDCCH search space corresponds to index 0 in table 13-0 in TS 38.213 [3] CR.1.3 FDD - The configuration of set of resource blocks, slot symbols, RB offset and REG to CCE mapping of control resource set for Type0-PDCCH search space corresponds to index 8 in table 13-0 in TS 38.213 [3] CR.1.4 FDD - The configuration of set of resource blocks, slot symbols, RB offset and REG to CCE mapping of control resource set for Type0-PDCCH search space corresponds to index 11 in table 13-0 in TS 38.213 [3] New note 8 is old note 7, applicable only to the 24 PRB case Row: “SSB and RMSI CORESET multiplexing configuration” Note 7 is removed as Multiplexing pattern is not specific to any index of the CORESET tables. Row: “Offset between SSB and RMSI CORESET” note 8 is changed to note 7: Old note 8 is the new note 7 applying to all rows. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A.3.1.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification remains faulty. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change 1>

### A.3.1.2	CORESET for RMSI scheduling

#### A.3.1.2.1	FDD

Table A.3.1.2.1-1: RMSI CORESET Reference Channel for FDD with SCS=15KHz

| Parameter | Unit |  | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CR.1.1 FDD | CR.1.2 FDD | CR.1.3 FDD | CR.1.4 FDD |  |  |  |
| Channel bandwidth | MHz | Defined in test case | Defined in test case | Defined in test case | Defined in test case |  |  |  |
| Subcarrier spacing for RMSI CORESET | kHz | 15 | 15 | 15 | 15 |  |  |  |
| Allocated resource blocks for RMSI CORESET |  | 24 (Note 8) | 12 (Note 9) | 15 (Note 10) | 20 (Note 11) |  |  |  |
| Subcarrier spacing for SSB | kHz | 15 | 15 | 15 | 15 |  |  |  |
| SSB and RMSI CORESET multiplexing configuration |  | Pattern 1 | Pattern 1 | Pattern 1 | Pattern 1 |  |  |  |
| Offset between SSB and RMSI CORESET Note 3, 7 | PRB | 0 (Note 8) | 0 (Note 9) | 0 (Note 10) | 0 (Note 11) |  |  |  |
| Configuration of PDCCH monitoring occasions for RMSI CORESET Note 4 |  | Index 4 | Index 4 | Index 4 | Index 4 |  |  |  |
| Number of transmitter antennas |  | 1 | 1 | 1 | 1 |  |  |  |
| Duration of RMSI CORESET | symbols | 2 (Note 8) | 2 (Note 9) | 3 (Note 10) | 3 (Note 11) |  |  |  |
| DCI Format Note 1 |  | Note 2 | Note 2 | Note 2 | Note 2 |  |  |  |
| Aggregation level | CCE | 8 | 4 | 8 | 8 |  |  |  |
| DMRS precoder granularity |  | 6 | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size |  |  |  |
| REG bundle size |  | 6 | 6 | 6 | 6 |  |  |  |
| Mapping from REG to CCE |  | Distributed (Note 8) | Distributed (Note 9) | Non-Distributed (Note 10) | Distributed (Note 11) |  |  |  |
| Cell ID |  | Note 5 | Note 5 | Note 5 | Note 5 |  |  |  |
| Payload (without CRC) | bits | Note 6 | Note 6 | Note 6 | Note 6 |  |  |  |
| NOTE 1: DCI formats are defined in TS 38.212 [8]. NOTE 2: DCI format shall depend upon the test configuration. NOTE 3: The offset is defined with respect to the subcarrier spacing of the CORESET from the smallest PRB index of RMSI CORESET to the smallest PRB index of the common PRB overlapping with the first PRB of the SS/PBCH block. NOTE 4: The configuration of PDCCH monitoring occasions for RMSI CORESET is defined in table 13-11 in TS 38.213 [3]. NOTE 5: Cell ID shall depend upon the test configuration. NOTE 6: Payload size shall depend upon the test configuration. NOTE 7: Other values can be used to align with GSCN [13] as long as SSB does not overlap the RMC. NOTE 8: The configuration of set of resource blocks, slot symbols, RB offset and REG to CCE mapping of control resource set for Type0-PDCCH search space corresponds to index 0 in table 13-1 in TS 38.213 [3] NOTE 9: The configuration of set of resource blocks, slot symbols, RB offset and REG to CCE mapping of control resource set for Type0-PDCCH search space corresponds to index 0 in table 13-0 in TS 38.213 [3] NOTE 10: The configuration of set of resource blocks, slot symbols, RB offset and REG to CCE mapping of control resource set for Type0-PDCCH search space corresponds to index 8 in table 13-0 in TS 38.213 [3] NOTE 11: The configuration of set of resource blocks, slot symbols, RB offset and REG to CCE mapping of control resource set for Type0-PDCCH search space corresponds to index 11 in table 13-0 in TS 38.213 [3] |  |  |  |  |  |  |  |  |

<End of change 1>