3GPP TSG-RAN4 Meeting #115	R4-2506143

# Malta, Malta, 19th May 2025 - 23rd May 2025

Source:			Ericsson

Title:			NTN Ku bands – system aspects

Agenda item:		7.11.4

Document for:		Approval

# Introduction

In last RAN#107, the NTN KU-band WI was revised ([1]) including support for 15 kHz SCS.

In RAN4#114bis meeting, RAN4 agreed ([2]) on the system parameters for the NTN Ku-bands. This contribution is further discussing this aspect for FR1-NTN.

# Discussion

In RAN4#114bis meeting, RAN4 agreed on the system parameters for the NTN Ku-bands, and more specifically the supported channel bandwidths and SCS for each FR1-NTN Ku-band:

Nevertheless, the WI ([1]) clarified that the same SCS is assumed for SSB and data channels:

In other words, this should mean that if the SAN and the NTN VSAT supports 15 kHz SCS, they will not support 30 kHz SCS (and vice versa).

Observation: According to the WID, SAN and NTN VSAT should only support one subcarrier spacing in FR1-NTN, either 15kHz or 30kHz, but not both.

When specifying the channel bandwidth and subcarrier spacing per NTN Ku-band, it should then be clarified that only one subcarrier spacing shall be supported.

Proposal: Add a note to Table 5.3.5-1 to clarify that only one subcarrier spacing shall be supported in n248 and n249.

Table 5.3.5-1: Channel bandwidths for each NTN satellite band in FR1-NTN

| NTN satellite band | SCS kHz | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 (Note 1) | 35 | 50 | 70 | 100 |
| n248 | 15 (Note 2) | 5 | 10 | 15 | 20 | 25 |  | 35 | 50 |  |  |
|  | 30 (Note 2) |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
| n247 | 15 (Note 2) |  | 10 | 15 |  | 25 |  | 35 | 50 |  |  |
|  | 30 (Note 2) |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
| NOTE 1: Deployment of 30 MHz channel bandwidth for NTN SAN needs to be preceded by introduction of all applicable Tx RF, Rx RF, and demodulation requirements. NOTE 2: Either 15kHz or 30 kHz subcarrier spacing shall be supported in this band, not both. |  |  |  |  |  |  |  |  |  |  |  |

# Conclusion

In this contribution, we discussed the NTN Ku-bands system parameters. We made the following observation and proposal:

Observation: According to the WID, SAN and NTN VSAT should only support one subcarrier spacing in FR1-NTN, either 15kHz or 30kHz, but not both.

Proposal: Add a note to Table 5.3.5-1 to clarify that only one subcarrier spacing shall be supported in n248 and n249.

Table 5.3.5-1: Channel bandwidths for each NTN satellite band in FR1-NTN

| NTN satellite band | SCS kHz | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 (Note 1) | 35 | 50 | 70 | 100 |
| n248 | 15 (Note 2) | 5 | 10 | 15 | 20 | 25 |  | 35 | 50 |  |  |
|  | 30 (Note 2) |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
| n247 | 15 (Note 2) |  | 10 | 15 |  | 25 |  | 35 | 50 |  |  |
|  | 30 (Note 2) |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
| NOTE 1: Deployment of 30 MHz channel bandwidth for NTN SAN needs to be preceded by introduction of all applicable Tx RF, Rx RF, and demodulation requirements. NOTE 2: Either 15kHz or 30 kHz subcarrier spacing shall be supported in this band, not both. |  |  |  |  |  |  |  |  |  |  |  |

# References

- RP-250799, Introduction of Ku bands for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, Airbus, Sharp, Mitre, JSAT

- R4-2504761, Way Forward for [114bis][313] NR_NTN_Ku_Band_General, Eutelsat