3GPP TSG-RAN4 Meeting #115	R4-2506144

# Malta, Malta, 19th May 2025 - 23rd May 2025

Source:			Ericsson

Title:			NTN Ku bands –VSAT RF

Agenda item:		7.11.5

Document for:		Approval

# Introduction

In last RAN#107, the NTN KU-band WI was revised ([1]) including support for 15 kHz SCS.

In last 2 RAN4 meetings, RAN4 agreed on many aspects of VSAT RF requirements ([4] and [6]) but few of them still need further discussion. This contribution is addressing those remaining VSAT RF open issues.

# Discussion

## PFD limits

In last RAN4#114bis meeting, it was still left open ([6]) if PFD limits should be captured in TS 38.101-5:

As we already explained in our previous contribution (XX), if PFD limits for SAN might not be relevant for 3GPP as they are depending on SAN deployment, the situation is not the same for the VSAT as the PFD limits are usually not depending on the VSAT location.

Also, the PFD limits being part of the Regulation, they are anyway mandatory to support for any VSAT operating in the corresponding NTN Ku-bands.

In last RAN4#114bis meeting, companies were concerned with the testing aspects of such requirements, how to test PFD requirements as this type of requirement has not yet been specified in 3GPP.

As we already mentioned in our RAN4#114bis contribution (XX), the PFD requirements have already been captured in EN 302 186 (XX). This EN aiming to be a Harmonized Standard, the corresponding test methods have been specified as well in this specification, proving that PFD testability is not an issue. Referring to EN 302 186, the PFD requirement is tested in 3 steps:

- Measuring the antenna radiation pattern.

- Measuring the on-axis EIRP.

- And then computing the PFD at the surface of the Earth.

Based on these clarifications, we make the following proposal:

Proposal1: Capture PFD limits for NTN VSAT when specified by the Regulation.

And the following PFD should then be captured:

- FCC:

  - A-ESIM, to protect fixed service network (when they have primary allocation in that band). 25.228(i):

- CEPT:

  - A-ESIM with NGSO – ECC Dec(18)05

    - In 14.25-14.5 GHz where FS systems are authorized in that band:

  - L-ESIM with NGSO – ECC Dec(18)05

    - In 14.25-14.5 GHz where FS systems are authorized in that band:

PFD limit of -116 dBW/m²/MHz at 30 m height above ground

    - In 14.47-14.5 GHz:

PFD threshold value at the observatory of -169 dBW/m²/(150 kHz), not to be exceeded more than 2% of the time (Ref: Recommendation ITU-R RA.769)

  - M-ESIM with NGSO – ECC Dec(18)05

    - In 14.25-14.5 GHz where FS systems are authorized in that band:

the PFD threshold value is -116 dBW/m²/MHz at a height of 80 metres above mean sea level at the low-water mark

    - In 14.47-14.5 GHz:

PFD threshold value at the observatory of -169 dBW/m²/(150 kHz), not to be exceeded more than 2% of the time (Ref: Recommendation ITU-R RA.769)

## Other open issue

RAN4 discussed in RAN4#114 meeting if the 3 types of ESIM should be considered for NTN Mobile VSAT operating in the Ku-bands. It was finally agreed to consider the 3 ESIM types and only considering Land ESIM for the coexistence study, assuming that Aeronautical ESIM and Maritime ESIM will be switched off when approaching the ground/port or at least comply with Land ESIM requirements.

From the Chair’s minutes:

Companies were encouraged to confirm those assumptions which would be then captured in the TR. It would be good to keep track of this open issue.

Proposal2: RAN4 should confirm that A-ESIM and M-ESIM features shall be switched off when approaching the ground (or the port).

# Conclusion

In this contribution, we analyzed the NTN VSAT RF specification TS 38.101-5 for the new NTN Ku-bands considering the last update of the WID. We made the following observations and proposals:

Proposal1: Capture PFD limits for NTN VSAT when specified by the Regulation.

Proposal2: RAN4 should confirm that A-ESIM and M-ESIM features shall be switched off when approaching the ground (or the port).

# References

- RP-250799, Introduction of Ku bands for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, Airbus, Sharp, Mitre, JSAT

- R4-2502290, Way Forward for [114][313] NR_NTN_Ku_Band_General, Eutelsat

- R4-2503944, Draft CR to TS 38.101-5 – Needed updates to support NTN VSAT in FR1-NTN, Ericsson

- R4-2502288, WF for [114][314] NR_NTN_Ku_Band_UE_SAN_RF, CHTTL

- R4-2501356, NTN Ku bands –WI update impacts on NTN VSAT RF, Ericsson

- R4-2504720, Way Forward for [114bis][314] NR_NTN_Ku_Band_UE_SAN_RF, CHTTL