3GPP TSG-RAN WG4 Meeting #115	R4-2506160

# St Julian’s, Malta, 19th – 25th May 2025

Title:	n1 PHS, splitting A2 for PUCCH benefit

Source:	Qualcomm Incorporated

Agenda item:	5.26.1

Release:	Rel-19

Document for:	Approval

# 1.	Background

n1 NS_05 requirements are relaxed since the PHS service is closed. However, the DECT service in the same range 1884.5MHz - 1915.7MHz must still be protected in Japan. The updated Japanese regulations were captured, the PHS protection requirement was modified to protect DECT [1], and PC3 A-MPR definitions and values were agreed in WF [2] for NS_05 requirement in the previous meeting. In [3], proposes increasing range for common PUCCH configuration and correspondingly proposes revised A-MPR region definitions. In this paper we verify the proposal in [3] and provide our analysis.

# 2. 	Discussion

## 2.1	Background and Motivation

In [3], it is noted that for RB allocations starting in the Region A2 with ≤ 5.04 MHz/12/SCS at the lower carrier edge, the required A-MPR is reduced when the RBstart is such that the IMD3 falls in the -25 dBm/MHz range below 1906.6 MHz as compared to that needed when the IMD3 falls in the -30 dBm/MHz range.

Considering the above observation in [3] and motivation from [4] to propose to study improvements of the A-MPR regions such that the PUCCH resource sets can be configured without A-MPR using PRB offset (clause 9.2.1 in 38.213), [3] proposes to split Region A2 into two regions. Region A2 to end at RBstart ≤ 10 (1.80MHz) and Region A3 to be between 10 (1.80MHz) < RBstart ≤ 28 (5.04 MHz). This enables to place the common PUCCH for DFT-s-OFDM QPSK case in region A3 with less backoff than in current Region A2 (and the new proposed Region A2) which will contain a higher backoff need to meet the requirements.

Table 1 below contains the A-MPR region definitions change proposed in [3] to enable PUCCH in Region A

Table 1: A-MPR regions for NS_05 with exception for PUCCH

| Channel bandwidth [MHz] | Carrier centre frequency FC [MHz] | Region A1 |  | Region A2 |  | Region A3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | RBstart | LCRB | RBstart | LCRB |
| 10 | FC < 1930.7 | ≥0 | ≥ 1.5*RBstart + 4.7 MHz/12/SCS | ≤ 1.8 MHz/12/SCS | ≤ 2.16 MHz/12/SCS |  |  |
| 15 | 1927.5 ≤ FC < 1932.5 | ≥0 | ≥ 1.45*RBstart + 4.68 MHz/12/SCS | ≤ 3.6 MHz/12/SCS | ≤ 3.6 MHz/12/SCS |  |  |
| 15 | 1932.5 ≤ FC < 1938.7 | ≤ 0.54 MHz/12/SCS | ≥ 12.96 MHz/12/SCS | ≤ 2.16 MHz/12/SCS | ≤ 2.16 MHz/12/SCS |  |  |
| 20 NOTE 3 | FC < 1945.7 | ≥0 | ≥ 1.1*RBstart + 4.14 MHz/12/SCS | ≤ 1.80 MHz/12/SCS | ≤ 5.76 MHz/12/SCS and < 1.1*RBstart + 4.14MHz/12/SCS | > 1.80 MHz/12/SCS and ≤ 5.04 MHz/12/SCS | ≤ 5.76 MHz/12/SCS |
| 20 NOTE 4 | FC < 1945.7 | ≥0 | ≥ 1.1*RBstart + 4.14 MHz/12/SCS | ≤ 1.80 MHz/12/SCS | ≤ 5.76 MHz/12/SCS and < 1.1*RBstart + 4.14MHz/12/SCS | > 1.80 MHz/12/SCS and ≤ 5.04 MHz/12/SCS | ≤ [0.9] MHz/12/SCS |
| NOTE 1: The A-MPR values are specified in Table TBD NOTE 2: The sub-carrier spacing SCS expressed in MHz NOTE 3: Not applicable for DFT-s-OFDM QPSK for FC < 1945.7 MHz NOTE 4: Only applicable for DFT-s-OFDM QPSK (for accommodating PUCCH transmissions with 0.5dB A-MPR with a reduced PRB offset from the carrier bandwidth edge) |  |  |  |  |  |  |  |

And the Table 2 shows the corresponding reworked A-MPR values for NS_05 in [3]

Table 2: Reworking A-MPR for NS_05

| CBW = 20 MHz, DFT-s-OFDM QPSK Fc < 1945.7 MHz | Region A1: A-MPR | Region A2: A-MPR | Region A3: A-MPR |
| --- | --- | --- | --- |
| IQ Image= -28 dB and Carrier Leakage = -28dBc | 4 | 4 | 2 |
| IQ Image = -34 dB and Carrier Leakage = -34dBc | 4 | 3 | 0.5 |

To meet the reduced A-MPR value of 0.5dB in Region A3, the following improved simulation assumptions are assumed in [3] for DFT-s-OFDM QPSK case:

- Power Class 3

- NR Waveform

- Calibration: 1dB MPR: DFT-s-OFDM QPSK 20MHz, 100RB0

- Carrier Leakage: -34dBc

- Image: -34dBc

- CIM3: -60dBc

- CIM5: -70dBc

We use the observations and conclusions from [3] to base our motivation for this paper and in the following sections, we show how we verify the proposals from [3] and provide our analysis on the same.

## 2.2 	Measurement and simulation verifications

### 2.2.1	Measurement and simulations details

According to the proposed changes in [3] that is also mentioned in above section 2.1, we first verify the A-MPR region changes as shown in below figures.

The original A-MPR regions proposed in the Way-Forward in [2] is as shown in Figure 1.

Figure 1. Graphical representation of proposed PC3 NS_05 A-MPR region definitions from WF [2]

The proposed change for 20MHz channel bandwidth A-MPR region definitions, for all cases other than DFT-s-OFDM QPSK is as shown in Figure 2. This represents the first 20MHz channel bandwidth row in Table 1.

Figure 2. Graphical representation of proposed PC3 NS_05 A-MPR region definitions for all 20MHz channel bandwidth cases except DFT-s-OFDM QPSK

The proposed change for A_MPR region for channel bandwidth 20MHz DFT-s-OFDM QPSK to accommodate PUCCH transmission enabling in represented in Figure 3 below. As seen, the Region A3 is much smaller with lower A-MPR allowance.

Figure 3. Graphical representation of proposed PC3 NS_05 A-MPR region definitions for 20MHz DFT-s-OFDM QPSK for PUCCH transmission

We obtain the following result shown in Figure 4, for channel bandwidth 20MHz DFT-s-OFDM QPSK case using the observations and assumptions mentioned in [3].

Figure 4. Measurement/simulation verification for lower A-MPR proposal for PUCCH enabling in channel bandwidth 20MHz DFT-s-OFDM QPSK

Based on our measurement and simulation analysis, it is observed that it is viable to split the Region A2 into two parts and propose lower A-MPR for new Region A3 in order to enable placement of the common PUCCH for DFT-s-OFDM QPSK.

## 2.3	Specification changes

Band n1 is an old band and solutions are mature. In some cases the real performance is better than what 3GPP simulation procedures indicate but they maybe difficult to generalise. While the adapted simulation assumptions in [3] yield improved performance, they should not be used as baseline for anything.

Proposal 1: Do not assume improved LO leakage or IQ image as baseline for any work in 3GPP

Regarding the proposed AMPR enhancement, as discussed, we are favouring to adopt such improvement but the actual specification changes may need some rework. In the proposed specification (copied as Table 1 and Table 2 in this paper) the actual AMPR is in the note 4 of the table. It would be preferable to write the Table 5 from [2] like this

Table 5: A-MPR values for NS_05 (Power Class 3, 20MHz, Fc<1945.7MHz)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) | A4 (dB) |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 3 | ≤ 3 |  |
|  | QPSK | ≤ 4 | ≤ 3.5 | ≤ 3.5 | 0.5 |
|  | 16 QAM | ≤ 5 | ≤ 4 | ≤ 4 |  |
|  | 64 QAM | ≤ 5 | ≤ 4 | ≤ 4 |  |
|  | 256 QAM | ≤ 5 | ≤ 4.5 | ≤ 4.5 |  |
| CP-OFDM | QPSK | ≤ 6 | ≤ 5.5 | ≤ 5.5 |  |
|  | 16 QAM | ≤ 6 | ≤ 5.5 | ≤ 5.5 |  |
|  | 64 QAM | ≤ 6 | ≤ 5.5 | ≤ 5.5 |  |
|  | 256 QAM | ≤ 7 | ≤ 6 | ≤ 6 |  |

And corresponding changes in the proposed region table as

| Channel bandwidth [MHz] | Carrier centre frequency FC [MHz] | Region A1 |  | Region A2 |  | Region A3 |  | Region A4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | RBstart | LCRB | RBstart | LCRB | RBstart | LCRB |
| 10 | FC < 1930.7 | ≥0 | ≥ 1.5*RBstart + 4.7 MHz/12/SCS | ≤ 1.8 MHz/12/SCS | ≤ 2.16 MHz/12/SCS |  |  |  |  |
| 15 | 1927.5 ≤ FC < 1932.5 | ≥0 | ≥ 1.45*RBstart + 4.68 MHz/12/SCS | ≤ 3.6 MHz/12/SCS | ≤ 3.6 MHz/12/SCS |  |  |  |  |
| 15 | 1932.5 ≤ FC < 1938.7 | ≤ 0.54 MHz/12/SCS | ≥ 12.96 MHz/12/SCS | ≤ 2.16 MHz/12/SCS | ≤ 2.16 MHz/12/SCS |  |  |  |  |
| 20 NOTE 3 | FC < 1945.7 | ≥0 | ≥ 1.1*RBstart + 4.14 MHz/12/SCS | ≤ 1.80 MHz/12/SCS | ≤ 5.76 MHz/12/SCS and < 1.1*RBstart + 4.14MHz/12/SCS | > 1.80 MHz/12/SCS and ≤ 5.04 MHz/12/SCS | ≤ 5.76 MHz/12/SCS |  |  |
| 20 NOTE 4 | FC < 1945.7 | ≥0 | ≥ 1.1*RBstart + 4.14 MHz/12/SCS | ≤ 1.80 MHz/12/SCS | ≤ 5.76 MHz/12/SCS and < 1.1*RBstart + 4.14MHz/12/SCS |  |  | > 1.80 MHz/12/SCS and ≤ 5.04 MHz/12/SCS | ≤ [0.9] MHz/12/SCS |
| NOTE 1: The A-MPR values are specified in Table TBD NOTE 2: The sub-carrier spacing SCS expressed in MHz NOTE 3: Not applicable for DFT-s-OFDM QPSK for FC < 1945.7 MHz NOTE 4: Applies only for 15 kHz SCS. |  |  |  |  |  |  |  |  |  |

Proposal 2: Adopt the proposed AMPR as written above

Further limitation should be done to agree this only for 15 kHz SCS and mark the limitation in the specification. This is because no work was done for the 30 kHz SCS.

# Conclusion

XXXXX

# Reference

[1] R4-2420481, “WF on PHS protection update”, KDDI, OPPO, 3GPP TSG-RAN WG4 Meeting #113, Orlando, US, 18th – 22nd November, 2024

[2] R4- 2502980, “WF on UE requirements for PHS protection”, KDDI, OPPO, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greek, 17th – 21st February, 2025

[3] R4-2504569, “A-MPR regions for NS_05/NS_05U and optionality in earlier releases by modified MPR behaviour”, Ericsson, TSG-RAN Working Group 4 (Radio) meeting #114bis, Wuhan, China, 7 – 11 April 2025

[4] R4-2500231, “(NR_newRAT-Core) Further study on NR A-MPR reevaluation scenarios on n1 for Japan”, KDDI

[5]	R4-2505222, “WF on PHS protection update”, KDDI, 3GPP TSG-RAN WG4 Meeting #114bis, Wuhan, China, 7th – 11st April, 2025