3GPP TSG-RAN WG4 Meeting #115	R4-2506210

# Malta, MT, 19 May 2025 - 23 May 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM requirements for SBFD operation

Agenda item:	7.25.4.2

Document for:	Discussion

Introduction

In the last RAN4 meeting, the following agreements were made [1]:

### Sub-topic 1-2: RRM impacts of SBFD operation #### Issue 2-1-1a: CSI-RS L1 measurement – collision with UL - Agreements (adhoc session on Thursday) - Dynamically scheduled UL transmission is prioritized over periodic/semi-persistent CSI-RS resource for L1-RSRP/SINR and RLM/BFD/CBD measurement. - Measurement period for periodic/semi-persistent CSI-RS based L1-RSRP/SINR and RLM/BFD/CBD is extended for collision with dynamically scheduled UL transmission following the R16 NR-U approach. #### Issue 2-1-1b: CSI-RS L1 measurement – puncturing in frequency domain - Agreements (adhoc session on Thursday with additions) - Note: the following are for UE that can support simultaneous PDSCH reception on the two DL subbands for DUD - Existing delay (number of samples) requirements and accuracy requirements apply for Case 1 and Case 2 - If UE indicates that CSI processing time is scaled by 2, further discuss whether and how to update the delay requirements - Option 1: Add a lower bound of 8ms on top of existing delay requirements. - Option 2: other - For Case 3 - For L1-RSRP and L1-SINR, existing delay requirements apply and accuracy requirements are relaxed provided that X1 > X4 or X2 > X4, where the accuracy requirements depend on the X4 = 24 PRBs - In the test case, not model the interfering UE. #### Issue 2-1-1c: CSI-RS L1 measurement – different symbol types - Agreements - For L1-RSRP and L1-SINR - For CSI-RS based L1-RSRP/SINR measurements, UE measures the CSI-RS occasions on the valid symbol type indicated by network - For L1-RSRP and L1-SINR measurement period, further discuss the following options. - Option 1: Measurement period for CSI-RS based L1-RSRP/L1-SINR is extended by excluding resource occasions on invalid symbol type following the R16 NR-U approach. - Option 2: other - For RLM/BFD/CBD, further discuss the following options - RAN4 to define requirements based on the assumption that occasions of a single CSI-RS resource are all in the same symbol type - Existing measurement period apply - FFS whether to add Note: For the potential power difference between SBFD symbol and non-SBFD symbol, no RAN4 enhancement on power difference handling in Rel-19

Discussion

Requirements for CSI-RS based L1 measurement

CSI-RS puncturing in frequency domain in SBFD symbols

The L1 measurement requirements for the case in which the CSI-RS resource is split across two subbands (DUD) have been discussed in RAN4 for the cases below:

- Case 1: X1 >= X3 and X2 >= X3

- Case 2: X1 >= X3 and X2 < X3

- Case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3

where X1 and X2 are CSI-RS BW in DL sub-band #1 and sub-band #2, and X3 is the min BW with which the existing accuracy requirements apply (X3=48 for L1-RSRP/L1-SINR and X3=24 for RLM/BFD/CBD).

The following other issues related to CSI-RS based L1 measurement were discussed in RAN4 #114:

- Agreements (adhoc session on Thursday with additions) - Note: the following are for UE that can support simultaneous PDSCH reception on the two DL subbands for DUD - Existing delay (number of samples) requirements and accuracy requirements apply for Case 1 and Case 2 - If UE indicates that CSI processing time is scaled by 2, further discuss whether and how to update the delay requirements - Option 1: Add a lower bound of 8ms on top of existing delay requirements. - Option 2: other - For Case 3 - For L1-RSRP and L1-SINR, existing delay requirements apply and accuracy requirements are relaxed provided that X1 > X4 or X2 > X4, where the accuracy requirements depend on the X4 = 24 PRBs - In the test case, not model the interfering UE.

The support for PDSCH reception on two subbands for DUD has been discussed in past RAN1 meetings. Below we include the list of relevant agreements from RAN1. The following has been agreed in R1-2502965:

| 60-1 | Basic feature for SBFD-aware UE TX/RX/measurement behaviour and procedures | 1. Support of cell-common configuration of time and frequency location of SBFD subbands 2. Support of UL transmission in one UL subband and DL reception in one [and two] DL subband in SBFD symbols 3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling 4. Support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols (Configuration 1) 5. Support of separate CSI reports for SBFD and non-SBFD symbols FFS: Whether to include two DL subband support in component 2 FFS: other component (which may be separate FG) |  | YES | n/a | Basic feature for SBFD-aware UE TX/RX/ measurement behaviour and procedures is not supported | Per band | TDD only | n/a | n/a | [Note: Only UL transmissions are allowed within UL subband and only DL receptions (except for CLI measurement) are allowed within DL subband(s)] [Note: The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one.] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For CSI-RS reception processing, RAN1 agreed that:

Agreement (RAN1#117) For a contiguous CSI-RS resource which overlaps with SBFD subband boundaries, only CSI-RS frequency resources within DL usable PRBs are valid for SBFD-aware. - No impact on CSI-RS sequence generation

CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured)

Agreement RAN1#118b For CSI processing timeline in SBFD symbols to process the CSI-RS across two DL subbands, - Option 3: It is up to UE capability to indicate one of the following: - CSI processing timeline and CPU counting are the same as legacy. Scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy.

Our view is that:

  - The ability of the UE to support simultaneous reception on 2 DL subbands has been discussed to be part of the basic feature set of SBFD aware UEs, but not agreed yet. Our understanding is that this might be part of the basic feature set, or part of a capability, depending on the RAN1 discussions on this topic. Anyway, RAN4 should specify the requirements for UEs that support reception in 1 DL subband, and for UEs that support reception in 2 DL subbands. It is up to RAN1 to decide if the simultaneous reception in 2 DL subbands is the basic feature or additional capability is needed.

  - For CSI-RS across two DL subbands, according to RAN1 agreement, it is up to UE capability to indicate if the processing timeline and CPU time are the same as legacy, or whether the processing timeline is scaled by a factor of 2. We understand this capability is conditioned on that UE supports simultaneous reception of 2 DL subbands. Otherwise, if UE only receives 1 DL subband, the processing time is not needed.

The UE capability of CSI processing timeline is conditioned on that UE supports simultaneous reception of 2 DL subbands.

Meanwhile in last RAN4 meeting, we agreed to introduce a degraded accuracy performance based on 24PRB as some UE vendors were not able to measure simultaneously the CSI-RS resource in 2 DL subbands in Case 3. As the accuracy was compromised implying UE only measures the CSI-RS in single DL subband, the CSI processing time for measuring 2 DL subbands shall not apply. Therefore, for Case 3, if the accuracy requirements depend on the X4 = 24 PRBs, the existing measurement delay shall apply i.e. lower bound is not needed. This applies when UE does not support simultaneous DL reception in 2 DL subbands.

For Case 3, the accuracy was compromised implying UE only measures the CSI-RS in single DL subband, hence the CSI processing time does not apply.

For Case 3, if the accuracy requirements is adopted dependent on the X4 = 24 PRBs, the existing measurement delay shall apply i.e. lower bound is not needed. This applies when UE does not support simultaneous DL reception in 2 DL subbands.

In addition, some UEs may be able to support simultaneous DL reception in 2 DL subbands. We understood this comprises simultaneous reception of both data and signalings i.e. simultaneous measurement shall also be supported. In this case, we can consider the CSI processing time which may be scaled by 2. That is, a lower bound may be introduced to the existing measurement delay but the accuracy performance shall not be relaxed as the UE is measuring 2 DL subbands and the sum of the usable PRBs exceeds X3. Hence we propose for Case 3, if UE supports simultaneous DL reception in 2 DL subbands, a lower bound can be introduced to the existing measurement delay but the accuracy performance shall be based on 48 PRBs.

For Case 3, if UE supports simultaneous DL reception in 2 DL subbands, a lower bound can be introduced to the existing measurement delay but the accuracy performance shall be based on 48 PRBs.

CSI-RS distributed over SBFD and non-SBFD symbols

The RAN4 measurement requirements for CSI-RS L1-RSRP and L1-SINR, and for CSI-RS based RLM/BFD/CBD are based on the periodicity of the CSI-RS resources. In the RAN1 LS response, RAN1 replied that the agreement below is applicable to CSI-RS based L1-RSRP/SINR:

Agreement (RAN1#118bis) For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured. Only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation.

#### Issue 2-1-1c: CSI-RS L1 measurement – different symbol types - Agreements - For L1-RSRP and L1-SINR - For CSI-RS based L1-RSRP/SINR measurements, UE measures the CSI-RS occasions on the valid symbol type indicated by network - For L1-RSRP and L1-SINR measurement period, further discuss the following options. - Option 1: Measurement period for CSI-RS based L1-RSRP/L1-SINR is extended by excluding resource occasions on invalid symbol type following the R16 NR-U approach. - Option 2: other - For RLM/BFD/CBD, further discuss the following options - RAN4 to define requirements based on the assumption that occasions of a single CSI-RS resource are all in the same symbol type - Existing measurement period apply FFS whether to add Note: For the potential power difference between SBFD symbol and non-SBFD symbol, no RAN4 enhancement on power difference handling in Rel-19

L1-RSRP and L1-SINR measurement period

For L1-RSRP and L1-SINR measurement period considering a valid symbol type, it has been discussed whether to extend the measurement period by excluding the resource occasion on invalid symbol type, following the R16 NR-U approach. Given that this approach was already selected for the cases in which the CSI-RS overlaps in time domain with dynamic UL transmissions, we think that the approach can be reused for the case of samples overlapping with unvalid symbol types.

The L1-RSRP and L1-SINR period is extended to consider the CSI-RS occasions that overlap with the valid symbol type, following the NR-U approach.

- RLM/CBD/BFD

Regarding RLM/ CBD/BFD, it has been captured in the agreements the options in which the requirements of RLM/BFD/CBD could be defined based on the assumption that occasions of a single CSI-RS resource are all in the same symbol type. We believe that this is the most straightforward solution, therefore we support this option:

RAN4 to assume the CSI-RS configuration for RLM/CBD/BFD is limited to one symbol type on the definition of the requirements. The legacy evaluation periods can be reused for SBFD.

In addition, as the measurement on SBFD symbols are expected to be subject to higher interference levels. RAN4 to discuss whether different side conditions should be considered based on the indicated symbol type in order to fulfill the same accuracy requirements.

RAN4 to discuss whether different side conditions should be considered based on the indicated symbol type in the CSI-RS based L1-RSRP performance requirements.

Requirements for CSI-RS based L3 measurement

The problem of CSI-RS being punctured in SBFD symbols is also relevant for CSI-RS L3 measurements. For instance, we expect that CSI-RS that is transmitted from a neighbour cell for the purposes of inter-cell mobility would need to be punctured in case the CSI-RS resources are located in a SBFD symbol (of the neighbour cell) and overlap with the UL sub band (of the neighbour cell). This was discussed by RAN2 in RAN2#128 where the following agreement was reached:

- RAN2 wait for input from the other WGs regarding whether for inter-cell CSI-RS measurements, UE needs to be provided with information of the SBFD configuration of neighbouring cells.

In RAN4 #114, it has been agreed that:

- For DU case: - UE measurement requirements would be same as in legacy with the condition that all CSI-RS resources configured for measurement are available in DL subband of SBFD symbols and/or on non-SBFD DL symbols. - FFS on DUD case

In RAN4 #114b, the following has been discussed in the topic summary [2]:

Issue 2-1-2: Requirements for CSI-RS based L3 measurement - Proposals for requirements for DUD case - Proposal 1 (CATT, Nokia, E///, ZTE): - RAN4 to define the measurement requirement assuming the UE is provided with information of the SBFD configuration of neighboring cells. - UE needs to puncture the CSI-RS resources on SBFD symbols in frequency domain when target neighbour cell is a SBFD cell. (CATT, ZTE) - RAN4 can send LS to RAN2 to ask for supporting the configuration (E///) - RE-use the conclusion of L1-RSRP/SINR CSI-RS resource in different symbol types (ZTE) - Proposal 2 (Samsung, CMCC, HW): - For DUD case, use the same principle as agreed in last meeting for DU case - UE measurement requirements would be same as in legacy with the condition that all CSI-RS resources configured for measurement are available in one DL subband of SBFD symbols and/or on non-SBFD DL symbols of the cell transmitting the CSI-RS. (HW) - Proposal 3 (MTK): - RAN4 shall deprioritize CSI-RS based L3 measurements. - Proposals for collision between CSI-RS resource and dynamic UL transmission - Proposal 1 (CATT): - Measurement is prioritized over UL transmission - Proposal 2 (ZTE): - For serving cell CSI-RS based L3 measurement, apply same principle as that for CSI-RS based L1 measurement. - For neighbour cell CSI-RS based L3 measurement, follow existing scheduling restriction to handle the collision. - Recommended WF - Discuss the following options on principles for defining requirements for DUD case - Option 1: define requirement assuming the UE is provided with information of the SBFD configuration of neighboring cells - Option 2: same as DU case, UE measurement requirements would be same as in legacy under certain conditions - Discuss the following options for collision between CSI-RS resource and dynamic UL transmission - Option 1: existing scheduling restriction apply, i.e. measurement is prioritized - Option 2: Serving cell measurement is de-prioritized and neighbour cell measurement is prioritized over dynamic UL transmission.

The agreement from the last RAN4 #114 meeting considers only the DU case. In this case, we do not expect that the configured resources of the neighbor cell or the serving cell will overlap with the UL subband hence puncturing is not needed. For the DUD case, there is the possibility that the CSI-RS resources are configured with an overlap with the UL subband. RAN4 is still discussing the applicability of requirements depending on the CSI-RS puncturing and the number of resources available within one or two DL subbands.

We believe that this discussion of the puncturing of the CSI-RS resources (discussed in the context of L1 measurements) is also relevant for the CSI-RS layer 3 measurement case. In our contribution to the last RAN4 meeting, we discussed the following options:

- The UE has information about the neighbour cell SBFD configuration

- For the intra-frequency measurement, the UE assumes that the neighbour cell valid CSI-RS resources are the same as the serving cell.  There are three possible cases:

  - The neighbour cell SBFD configuration is the same as the serving cell.

  - The neighbour cell SBFD configuration is different from the serving cell,

  - The neighbour cell is not configured with SBFD.

In our view, in order to measure CSI-RS from neighbor cell, the UE would need to be aware of the SBFD configuration from the neighbour cell, e.g. at least whether the neighbouring cell operates in SBFD mode and the potential UL-DL RB split in SBFD symbols. Without being aware of the neighbour cell SBFD configuration, in case the SBFD configuration is different from the serving cells, as in cases 2b and 2c above, the UE might puncture incorrectly the CSI-RS for L3 measurements, impacting the measurement performance.

Without being aware of the SBFD configuration of the neighbor cell, the UE cannot correctly derive the valid neighbor cell CSI-RS resources, which might impact the measurement performance.

To avoid blocking the progress of other working groups on this issue, we think RAN4 should discuss requirements for CSI-RS based L3 measurement under the assumption that the UE is aware or provided with information of the SBFD configuration of neighbouring cells. RAN4 may inform RAN2 on this assumption which may then trigger RAN2 to discuss the signaling details.

For CSI-RS based L3 measurements, RAN4 to define the measurement requirement assuming the UE is provided with information of the SBFD configuration of neighboring cells.

If the UE is provided with the SBFD configuration of the neighbor cell, it can derive the resources available for measurement also in the DUD scenario both in intra-frequency measurements and inter-frequency measurements. Then for the intra-frequency case, the requirements are applicable if at least the same resources as the serving cell are available in DL subband of SBFD symbols on the neighbor cell.

Apart from this discussion, we note that RAN2 has had an email discussion in which the topic of layer 3 CSI-RS measurement report has been raised ([Post129bis][212][SBFD] Running CR for 38.331 (Huawei)). The majority of companies support that: the separate Layer 3 measurement report for CSI-RS resources in SBFD symbol is not supported, no RRC spec impact is expected.

We note that if the CSI-RS measurement resource for L3 measurements is configured in SBFD and non-SBFD symbols there might be a difference between the available resources in each symbol type. Mobility can be based on RSRP or RSRQ.  Below, we present the definitions of CSI-RSRP and CSI-RSRQ from TS 38.215:

CSI reference signal received power (CSI-RSRP), is defined as the linear average over the power contributions (in [W]) of the resource elements of the antenna port(s) that carry CSI reference signals configured for RSRP measurements within the considered measurement frequency bandwidth in the configured CSI-RS occasions. … CSI reference signal received quality (CSI-RSRQ) is defined as the ratio of N×CSI-RSRP to CSIRSSI, where N is the number of resource blocks in the CSI-RSSI measurement bandwidth. The measurements in the numerator and denominator shall be made over the same set of resource blocks. CSI Received Signal Strength Indicator (CSI-RSSI), comprises the linear average of the total received power (in [W]) observed only in OFDM symbols of measurement time resource(s), in the measurement bandwidth, over N number of resource blocks from all sources, including cochannel serving and non-serving cells, adjacent channel interference, thermal noise etc. The measurement time resource(s) for CSI-RSSI corresponds to OFDM symbols containing configured CSI-RS occasions.

We note that the SBFD symbols are subject to different interference levels due to UE-to-UE CLI, when compared to non-SBFD symbols. While the CSI-RSRP between SBFD and non-SBFD symbols might not be impacted (assuming that the UE is aware of the resource elements that carry CSI reference signals), the RSSI between SBFD and non-SBFD symbols might be significantly different, impacting on the RSRQ if symbols of different types are averaged. Our view is that the UE should not average CSI-RS values from different symbol types in the same report.

The interference levels between SBFD and non-SBFD symbols might be different, depending on the UE-to-UE CLI.

For CSI-RS based L3 measurements, the UE assumes that the CSI-RS measurement resources are configured on the same symbol type.

Requirements for scheduling restriction

In RAN4 #114b the scheduling restrictions have been discussed, as follows [2]:

#### Issue 2-1-3: Requirements for scheduling restriction - Proposals related to UL/DL transition - Proposal 1 (CATT, Nokia, E///, CMCC, HW, CTC, MTK): - Do not define new scheduling restriction requirements for DL/UL transition. - Proposals related to inter-frequency SSB measurement without gap - Proposal 1 (vivo, MTK): - For the UE support Rel-16 inter-frequency measurement without gap, when the neighbor cell SSB is outside DL subband however still within the active BWP, update on existing scheduling restriction is needed - the scheduling restriction in section 9.3.9.4 can be used as a reference (vivo) - UE is not required to receive DL in DL subbands on SBFD symbols (MTK) - Proposal 2 (CATT, Nokia, Samsung, HW): - RAN4 to use the existing scheduling restriction requirements for inter-frequency SSB measurement without gap as a starting point. - Recommended WF for UL/DL transition - It is suggested to agree on P1. - Recommended WF for inter-frequency SSB measurement outside MG - FFS whether additional scheduling restriction applies for inter-frequency SSB measurement outside MG on SBFD symbols, when the SSB is fully within DL active BWP but not completely contained in DL sub-band. - Option 1: yes, UE is not required to receive DL in DL subbands on SBFD symbols - Option 2: no, UE is required to receive DL in DL subbands on SBFD symbols

DL/UL transition

During the RAN4#113 meeting, it has been agreed to check RAN1 agreements and potential impact to RAN4 scheduling restrictions due to DL/UL transition.

In RAN1 #116, there was an agreement regarding the collisions between receptions in DL subband, and transmissions in UL subband captured below:

Agreement: For SBFD-aware UEs, collisions between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol may be addressed or alleviated with proper scheduling. The following cases of potential collisions, [if link direction indication is not supported or provided], can be further studied to see if any change to the current specs is necessary: - Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission - e.g., dynamic PDSCH or CSI-RS collides with configured SRS, PUCCH, or CG PUSCH - Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission - e.g., PDCCH or SPS PDSCH collides with dynamic PUSCH or PUCCH - Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission - Case 4: Dynamically scheduled DL reception vs. dynamic scheduled UL transmission - Case 5: SSB vs. dynamically scheduled or configured UL transmission - e.g., PUSCH, PUCCH, PRACH, SRS - Case 6: Dynamic or semi-static DL vs. valid RO Note: In addition to collision between UL transmission and DL reception in the same SBFD symbol(s), collision between UL transmission and DL reception in different symbol(s) due to lack of sufficient transition time between Tx/Rx at UE side is also included.

According to the scheduling availability requirements in TS 38.133, the UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured and on K data symbols before and after each consecutive SSB symbols to be measured within SMTC window duration. In both FR1 and FR2-1, K equals to 1 or 2 (60 kHz SCS).

The Tx/Rx transition time is specified in TS 38.211 as summarized below. RAN1 has not had any discussions regarding transition time in the current Rel-19 work item, so we assume that these existing requirements should also apply for SBFD operation.

A UE not capable of full-duplex communication is not expected to transmit in the uplink earlier than NRx-TxTc after the end of the last received downlink symbol in the same cell where NRx-Tx is given by Table 4.3.2-3. A UE not capable of full-duplex communication is not expected to receive in the downlink earlier than NTx-RxTc after the end of the last transmitted uplink symbol in the same cell where NTx-Rx is given by Table 4.3.2-3. … Table 4.3.2-3: Transition time NRx-Tx and NTx-Rx Transition time; FR1; FR2. NTx-Rx; 25600; 13792. NRx-Tx; 25600; 13792.

Tc is approximately 5.09 ns, so the transition time in FR1 is approximately 13 µs, and 7 µs in FR2. The duration of 1 OFDM symbol for different subcarrier spacings is given in the table below. Note that in all cases, the duration of an K OFDM symbos are longer than the transition time.

| Subcarrier spacing [kHz] | OFDM symbol duration [µs] |
| --- | --- |
| 15 | 66.73 |
| 30 | 33.37 |
| 60 | 16.68 |
| 120 | 8.35 |

In FR1 and FR2-1, the Tx-Rx transition time according to TS 38.211 is shorter than the duration of an OFDM symbol, which is the margin around SSB symbols in which the UE is not expected to transmit in UL.

The existing RAN4 scheduling restriction requirements for SSB measurements are sufficient to accommodate the Tx-Rx transition time in FR1 and FR2-1. There is no need to modify the existing scheduling restrictions.

Inter-frequency SSB measurement without gap

In the last RAN4 meeting, it was discussed whether additional scheduling restriction applies for inter-frequency SSB measurements outside MG on SBFD symbols, when the SSB is fully within DL active BWP but not completely contained in DL sub-band.

Some of the conditions for an inter-frequency measurement without gap are copied below from TS 38.133:

A measurement is defined as an inter-frequency SSB based measurements without measurement gaps (either legacy measurement gap or NCSG) in active BWP, for UE capable of interFrequencyMeas-NoGap provided that - the UE supports interFrequencyMeas-Nogap-r16 [15], and - the SSB is completely contained in the active BWP of the UE. - For inter-frequency SSB based measurements without measurement gaps, UE may cause scheduling restriction as specified in clause 9.3.9.3. - Note: Non-CA capable UE is not expected to indicate support of interFrequencyMeas-Nogap-r16 [15].

Additionally, it is important to discuss the impact of some relevant RAN1 agreements and working assumptions:

Working Assumption (RAN1#118b) For SSB symbols configured with SBFD subbands, - Option 1: The SSB symbols configured with SBFD subbands are SBFD symbols. Only DL receptions within DL usable PRBs are allowed for SBFD aware UEs. - Note: The SSB block is assumed to be within DL subband

The working assumption is that for SSBs symbols configured with SBFD subbands, the SSB is assumed to be within the DL subband. For the SBFD aware UE, only DL receptions within DL usable PRBs are allowed. Our understanding is that the working assumption applies to Serving cells configured with SBFD. The scenario we are discussing is the one in which the neighbor cell either does not support SBFD or has a different SBFD configuration from the serving cell.

Our understanding of the RAN1 working assumption is that the serving cell SSB block is assumed to be within DL subband. The expected behavior for SBFD aware UEs that “only DL receptions within usable PRBs are allowed” only applies to signals/ channels from the serving cell.

In meeting RAN#117, it has been agreed that:

Agreement (RAN1#117) Re-use the existing collision handling principles for NR TDD that SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission FFS whether a slot consisting of SSB symbols is considered as a full DL slot or SSB symbols configured with SBFD subbands are SBFD symbols and only DL receptions within DL usable PRBs are allowed for SBFD aware UEs.

SSB measurements are prioritized over configured UL transmission and dynamically scheduled UL transmission as per agreement in RAN1#117.

If a UE is configured to measure an SSB, let’s say the neighbor cell SSB, we assume that the downlink reception is prioritized over configured UL transmission as per RAN1 agreement copied above. So, while configured to perform inter-frequency measurements without gaps, there should be UL scheduling restrictions. In FR1, According to TS 38.133, there are already UL scheduling restrictions specified in clause 9.3.9.3.1. There are also DL scheduling restrictions specified if the subcarrier spacing from the SSB and the data are different. In FR2, there are already DL and UL scheduling restrictions in place.

Therefore, for the scenario in which the neighbor cell SSB overlaps with the UL subband, if the UE is capable of performing inter-frequency measurements without gaps, and the neighbor cell SSB is overlapping with the serving cell UL subband, the UE should still be able to receive DL signals from the serving cell within the DL subband, so there is no need to define additional DL scheduling restrictions. Our preferred option is Option 2: UE is required to receive DL in DL subbands on SBFD symbols for inter-frequency SSB measurement outside MG on SBFD symbols, when the SSB is fully within the DL active BWP but not completely contained in DL sub-band.

There is no need to define additional DL scheduling restrictions for inter-frequency measurements without measurement gaps when the neighbour cell SSB or CSI-RS is overlapping with the UL subband, if all the conditions for supporting inter-frequency measurements without measurement gaps are met.

RACH requirements

In RAN4 meeting #114b, the following was agreed:

Issue 2-1-4: Requirements for RACH requirements - Agreements - Discuss whether and how RACH requirements are to be updated for SBFD operation based on RAN2 agreement.

In TS 38.133, the correct behaviour when not receiving Random Access Response is:

The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [7] for the next available PRACH occasion, and transmit the preamble with the calculated PRACH transmission power, if no Random Access Response is received within the RA Response window configured in RACH-ConfigCommon or if no PDCCH addressed to UE’s C-RNTI is received within the RA Response window configured in BeamFailureRecoveryConfig, as defined in clause 5.1.4 in TS 38.321 [7].

Currently, the correct UE behavior is to update the PRACH transmission power if the Random Access Response is not received. In the RAN2 #127bis[3], RAN2 has agreed that:

For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.

In RAN2 #128bis, RAN2 agreed that:

- RO-Type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols is supported when the number of PRACH transmission attempts exceed a threshold (we assume it is the same threshold with the fallback from additional RO to legacy RO). If fallback from legacy RO to additional RO occurs, no further fallback to legacy RO is supported.

According to the agreements above, the UE is allowed to switch from additional RACH occasions to legacy RACH occasions, and from legacy RACH occasions to additional RACH occasions. This is not captured in the RAN4 requirements so far.

The requirements for 4-step RA type in clause 6.2.2.2 are updated to take into account the RO-type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols, and the RO type selection from additional RO to legacy RO, as agreed in RAN2. The exact update can be discussed during the CR drafting.

Conclusion

The following Observations and Proposals were made:

Observation 1: The UE capability of CSI processing timeline is conditioned on that UE supports simultaneous reception of 2 DL subbands.

Observation 2: For Case 3, the accuracy was compromised implying UE only measures the CSI-RS in single DL subband, hence the CSI processing time does not apply.

Proposal 1: For Case 3, if the accuracy requirements is adopted dependent on the X4 = 24 PRBs, the existing measurement delay shall apply i.e. lower bound is not needed. This applies when UE does not support simultaneous DL reception in 2 DL subbands.

Proposal 2: For Case 3, if UE supports simultaneous DL reception in 2 DL subbands, a lower bound can be introduced to the existing measurement delay but the accuracy performance shall be based on 48 PRBs.

Proposal 3: The L1-RSRP and L1-SINR period is extended to consider the CSI-RS occasions that overlap with the valid symbol type, following the NR-U approach.

RLM/CBD/BFD

Proposal 4: RAN4 to assume the CSI-RS configuration for RLM/CBD/BFD is limited to one symbol type on the definition of the requirements. The legacy evaluation periods can be reused for SBFD.

Proposal 5: RAN4 to discuss whether different side conditions should be considered based on the indicated symbol type in the CSI-RS based L1-RSRP performance requirements.

Observation 3: Without being aware of the SBFD configuration of the neighbor cell, the UE cannot correctly derive the valid neighbor cell CSI-RS resources, which might impact the measurement performance.

Proposal 6: For CSI-RS based L3 measurements, RAN4 to define the measurement requirement assuming the UE is provided with information of the SBFD configuration of neighboring cells.

Observation 4: The interference levels between SBFD and non-SBFD symbols might be different, depending on the UE-to-UE CLI.

Proposal 7: For CSI-RS based L3 measurements, the UE assumes that the CSI-RS measurement resources are configured on the same symbol type.

Observation 5: In FR1 and FR2-1, the Tx-Rx transition time according to TS 38.211 is shorter than the duration of an OFDM symbol, which is the margin around SSB symbols in which the UE is not expected to transmit in UL.

Proposal 8: The existing RAN4 scheduling restriction requirements for SSB measurements are sufficient to accommodate the Tx-Rx transition time in FR1 and FR2-1. There is no need to modify the existing scheduling restrictions.

Observation 6: Our understanding of the RAN1 working assumption is that the serving cell SSB block is assumed to be within DL subband. The expected behavior for SBFD aware UEs that “only DL receptions within usable PRBs are allowed” only applies to signals/ channels from the serving cell.

Observation 7: SSB measurements are prioritized over configured UL transmission and dynamically scheduled UL transmission as per agreement in RAN1#117.

Proposal 9: There is no need to define additional DL scheduling restrictions for inter-frequency measurements without measurement gaps when the neighbour cell SSB or CSI-RS is overlapping with the UL subband, if all the conditions for supporting inter-frequency measurements without measurement gaps are met.

Proposal 10: The requirements for 4-step RA type in clause 6.2.2.2 are updated to take into account the RO-type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols, and the RO type selection from additional RO to legacy RO, as agreed in RAN2. The exact update can be discussed during the CR drafting.

- References

- R4-2504904, WF on SBFD RRM, Huawei

- R4-2503623, summary [114][222] NR_duplex_evo, Huawei