3GPP TSG-RAN WG4 Meeting#115		R4-2506230

Malta, MT, May 19th -23rd, 2025

Agenda item:	7.26.3.2

Source: 	Spreadtrum, UNISOC

Title: 	Discussion on RF requirements for ambient IoT device 1

Document for:	Discussion

Introduction

In RAN4#114bis meeting, a WF [1] was agreed. This contribution continues to discuss remaining issues as follows.

- Topic 3-1: Transmit output power Issue 3-1-1: Transmit output power Agreement: - Performance metric of transmit output power: - Backscattering loss - Backscattering output power is defined as the minimum backscattered power of Device 1 at certain input CW Tx power - Minimum Backscattering power at device antenna= Input CW Tx power at device antenna-Backscattering loss - For the Input CW Tx power to define requirements: - FFS on the whether to use one value or a range of CW Tx power - FFS on the value(s) of CW Tx power - Consider following aspects to determine backscattering loss to define requirements: - modulation loss - antenna efficiency - backscattering circuitry loss - impairment margin - minimum distance to the reader - differential radar cross section in two different modulation states - differential reflection coefficient in two different modulation states - the spectral components of the backscatter signal to be included in the power measurement, e.g. only the 1st Lower SB and 1st Upper SB, excluding the reflected carrier itself - Other factors not excluded - Topic 3-2: Modulation quality Issue 3-2-1: SFO requirement Agreement: - 10% SFO is assumed as the minimum performance for devices. - FFS on whether to define SFO requirement and how to test the SFO requirement if defined. Issue 3-2-2: Modulation quality requirements Agreement: - Define modulation quality requirements for both BPSK and OOK. - Consider to define same modulation quality metric for both BPSK and OOK if possible. - Reference measurement process needs to be discussed - Topic 3-3: Emission requirements Issue 3-3-1: Emission requirements Agreement: - Discuss whether to define EIPR or TRP as performance metric for emission requirements - Spurious emission: reuse legacy NR UE spurious emission requirement and further discuss the OOB boundary. - FFS on CW maximum input power level for testing - Topic 3-4: Reference sensitivity Issue 3-4-1: Reference sensitivity Agreement: - Define minimum requirements of REFSENS based on link budget (target is [15m]) and measurement results reported by companies - Consider to use inventory procedure for REFSENS testing, success rate ([90%]) or miss detection rate ([10%]) as the metric for inventory - At least to define maximum gain direction sensitivity requirements - FFS on how to different sensitivity requirements on different orientations - Topic 3-5: Others Issue 3-5-1: Maximum input power Agreement: - Define maximum input level, consider following options: - Option 1: using [1m] distance assumption from reader to device - Option 2: using square law appliance range - Option 3: using dynamic range of device receiver Issue 3-5-2: Occupied bandwidth Agreement: Further discuss whether to define occupied bandwidth

Discussion

TX requirements

- Transmitter output power

Backscattered modulated power

According to the last meeting agreement, we need to define backscattering loss first, considering factors including modulation loss, antenna efficiency, and impairment margin and so on. For modulation loss, we can refer to the paper [2]. the amount of reflected power in a backscatter link is not only a function of the antenna properties of the RF tag and its surrounding materials. Since 1s and 0s must be coded with two different reflection states, the power of the digital information scattered toward the reader is also related to the difference in load states. Different materials have different modulation factors. It is observed that modulation loss is 6dB for cardboard based on the paper.

Proposal 1: We need to assume the tag surrounding material such as cardboard.

Observation1: Modulation loss is 6dB for cardboard material.

Regarding the antenna efficiency, we can take the half-wave dipole as an example. Its structure is simple, and the conductor loss is low. According to the survey, its antenna efficiency is about 90%~99%, so we can use its low value 90% as a reference.

Regarding the impairment margin, considering the differences in implementation and tolerance of other factors, we can use 3dB for the impairment margin.

Proposal 2: Impairment margin can be 3dB.

Based on the above analysis, considering modulation loss, antenna efficiency and impairment margin, we can calculate the backscattering loss can be 10 dB.

Proposal 3: Define backscattering loss as 10dB.

In order to derive the minimum backscattered power, we can use the link budget to calculate. Similar to the reader to device, we can get the sensitivity of the reader, then we can get the minimum backscattered power. On the whole, we have two approaches to get the minimum backscattered power. Approach 1: input CW power – backscattering loss. Approach 2: link budget (target coverage is 15m).  However, how to get the proper input CW power. We need to assume the distance between CW node and tag at first, we can assume the CW node is in the middle of the four Readers, then we can get the distance is about 15m. If the result of approach 1 is lower than approach 2, we can use the minimum backscattered power by approach 1. Otherwise, we can use the minimum backscatter power by approach 2.

- Approach 1: input power – backscattering loss= minimum backscattered power.

Proposal 4: We need to assume the distance between CW node and tag, such as 15m to get the input CW power.

- Approach 2: we can use the link budge to calculate. For D2R.   [4A] = [1M] + [2C] - [2X] - [2L] - [3A] - [3B] + [3C] + [3D]. We can assume REFSENS of reader is -106dBm. According to the formula, we can get the minimum backscatter power is -36dBm. Additionally, we need to maintain some margin, such as 3dB. We can get the minimum backscattered power is -33dBm.

Proposal 5: Based on the link budget, we can define the minimum backscattered power is -33dBm.

- Modulation quality requirements

According to the last meeting agreement, define modulation quality requirements for both BPSK and OOK. And define the same metric for OOK and BPSK. Based on the EPC RFIC Standard, RF envelope parameters including modulation Depth, RF envelope Ripple, RF envelope Rise Time, RF envelope Fall Time and RF Plusewidth. In our understanding, these parameters are related to baseband design. We need to consider baseband factors.

Proposal 6: To define modulation Depth, RF envelope Ripple, RF envelope Rise Time, RF envelope Fall Time and RF Plusewidth for OOK and BPSK. Details needs to be FFS.

- SFO requirement

According to the previous meeting, 10% SFO is assumed as the minimum performance for devices. Some companies are concerned about how to test SFO requirement. From our point of view, we can base on an ideal receiver, the time domain waveform is received at a higher clock frequency, and the ideal and actual sampling points are calculated, and the deviation is calculated to get the SFO result.

Proposal 7: To define SFO requirement. If SFO defined, SFO can be obtained by calculating the deviation number of sampling points.

- Occupied bandwidth

For NR requirements, occupied bandwidth is defined as the bandwidth containing 99% of the total integrated mean power of the transmitted spectrum on the assigned channel. According to the definition of channel bandwidth, which doesn’t include 3rd and 5rd harmonics. The power of the 3rd harmonics occupied of the signal power is 1/9. Obviously, (1-1/9) is lower than 99%, which cannot meet the definition of occupied bandwidth. Occupied bandwidth can be indirectly reflected by emissions requirements. Hence, there is no need to define occupied bandwidth.

Proposal 8: No need to define occupied bandwidth.

- Emission requirements

One composite emission requirement covers the emission frequency range of SEM and spurious emissions. In FR2, all out of band emissions for frequency range 2 are TRP. In the actual test, TRP and EIRP are needed to scan 3D and they are basically measured at the same time. TRP can also reflect the efficiency of the antenna. Hence, we can define TRP requirements for unwanted emissions. Regarding the boundary of OOB, we can reuse legacy NR, for different channel bandwidth, define D2R channel bandwidth plus minimum D2R channel bandwidth as the boundary between A-IoT out of band and spurious emission domain.

Proposal 9: Define TRP requirements for unwanted emissions. For different channel bandwidth, define D2R channel bandwidth plus minimum D2R channel bandwidth as the boundary between A-IoT out of band and spurious emission domain.

RX requirements

- Reference sensitivity

We can derive reference sensitivity according to the link budget table 1 from TR 38.769. According to the last meeting, the coverage target is 15m.  The pathloss isPL=33.63+21.9log10d3D+20log10fc.

| No. | Item | Reader-to-Device | Device-to-Reader |
| --- | --- | --- | --- |
| (0) System configuration |  |  |  |
| [0A] | Scenarios | D1T1-A1/A2/B/C D2T2-A1/A2/B/C | D1T1-A1/A2/B/C D2T2-A1/A2/B/C |
| [0A1] | CW case | N/A | 1-1/1-2/1-4/2-2/2-3/2-4 |
| [0B] | Device 1/2a/2b | Device 1/2a/2b | Device 1/2a/2b |
| [0C] | Center frequency (MHz) | 900MHz (M), 2GHz (O) | 900MHz (M), 2GHz (O) |
| [0D] | Topology/Pathloss model | For D2T2: BS pathloss model is reused for intermediate UE with antenna height = 1.5 m [0D]-Alt1: InF-DL NLOS [0D]-Alt2: InH-Office LOS For D1T1: InF-DH NLOS | For D2T2: BS pathloss model is reused for intermediate UE with antenna height = 1.5 m [0D]-Alt1: InF-DL NLOS [0D]-Alt2: InH-Office LOS For D1T1: InF-DH NLOS |
| (1) Transmitter |  |  |  |
| [1D] | Number of Tx antenna elements / TxRU/ Tx chains modelled in LLS | For BS: 2(M) or 4(O) antenna elements for 0.9 GHz For Intermediate UE: 1(M) or 2(O) | 1 |
| [1E] | Total Tx Power (dBm) | For BS in DL spectrum for indoor - [1E]-R2D-Alt1: 33dBm(M), - [1E]-R2D-Alt2: 38dBm(O), - [1E]-R2D-Alt3: 24dBm(M) - Companies to report if PSD constraints are imposed (company to report the condition for applying PSD constraints in Row [5A]) For UL spectrum for indoor, - [1E]-R2D-Alt4:23dBm (M) - [1E]-R2D-Alt5:26dBm(O) | For device 1/2a: (see note 1) - [1E]-D2R-Alt1: For scenarios ‘B’, the device Tx Power is calculated by CW received power which can be derived by at least CW2D distance (m) value and other related factors. - [1E]-D2R-Alt2: For scenarios ‘A1’ and ‘A2’, the device Tx Power is calculated by assuming CW2D pathloss = D2R pathloss. For device 2b: For scenarios ‘C’ - [1E]-D2R-Alt3: -20 dBm(M) - [1E]-D2R-Alt4: -10 dBm(O) |
| [1E1] | CW Tx power (dBm) | N/A | For scenario ‘A1’, ‘A2’ and ‘B’ - Report a value from the candidate values [1E]-R2D-Alt1 / [1E]-R2D-Alt2 / [1E]-R2D-Alt3 from [1E]-R2D if CW in DL spectrum - Report a value from the candidate values [1E]-R2D-Alt4 / [1E]-R2D-Alt5 from [1E]-R2D if CW in UL spectrum. Note: only applicable for device 1/2a |
| [1E2] | CW Tx antenna gain (dBi) | N/A | Company to report, the value equals: - UE Tx ant gain, or - BS Tx ant gain Note: Only applicable for device 1/2a |
| [1E3] | CW2D distance (m) | N/A | For scenarios ‘B’ D1T1-B: - 5m, - 10m, - 20m - CW2D distance is derived assuming CW node is located with the same position as ‘R1’ in ‘A1’ scenario. (See note 1) D2T2-B: - 5m, - 10m, For scenarios ‘A1’ and ‘A2’: Calculated (see note 1), (i.e., CW2D distance is calculated by assuming CW2D pathloss = D2R pathloss) Note 1: Only applicable for device 1/2a. Note 2: Companies to report which value(s) are evaluated. |
| [1E4] | CW2D pathloss (dB) | N/A | Calculated (see note1) Note 1: Only applicable for device 1/2a Note 2: For CW2D pathloss model, use the same pathloss model as used for R2D/D2R. |
| [1E5] | CW received power (dBm) | N/A | Calculated (see note1) Note: Only applicable for device 1/2a |
| [1F] | Transmission Bandwidth used for the evaluated channel (Hz) | 180kHz(M), 360kHz(O), 1.08MHz(O) | Refer to LLS table [2a1] |
| [1G] | Tx antenna gain (dBi) | For BS for indoor, 6 dBi(M), 2dBi(M) For intermediate UE, 0 dBi | For A-IoT device, 0dBi |
| [1H] | Ambient IoT backscatter loss (dB) due to Modulation factor | N/A | OOK: 6 dB PSK: 0 dB FSK: Y dB It is applicable for device 1 and 2a. Companies to report and justify their assumptions for Y. Companies to report in row 3D if they assume any additional related loss. |
| [1J] | Ambient IoT on-object antenna penalty | N/A | 0.9dB or 4.7dB |
| [1K] | Ambient IoT backscatter amplifier gain (dB) | N/A | 10 dB (M) 15 dB (O) Note: Only for device 2a |
| [1N] | Cable, connector, combiner, body losses, etc. (dB) | For BS, X dB, X <=3 to be reported by companies with justification provided in row 5A For intermediate UE, 1 dB | N/A |
| [1M] | EIRP (dBm) | Calculated (see Note 1) | Calculated (see Note 1) |
| (2) Receiver |  |  |  |
| [2A] | Number of receive antenna elements / TxRU / chains modelled in LLS | Same as [1D]-D2R | Same as [1D]-R2D |
| [2B] | Bandwidth used for the evaluated channel (Hz) | Refer to LLS table [1b] ED bandwidth | Refer to LLS table [2a3] |
| [2C] | Receiver antenna gain (dBi) | same as [1G]-D2R | Same as [1G]-R2D |
| [2X] | Cable, connector, combiner, body losses, etc. (dB) | N/A | Same as [1N]-R2D |
| [2D] | Receiver Noise Figure (dB) | For RF-ED receiver 20dB, Device 2 For IF/ZIF receiver 15dB, Device 2 Other values can be optionally used and reported by companies | For BS as reader: 5dB For intermediate UE as reader: 7dB |
| [2E] | Thermal Noise power spectrum density (dBm/Hz) | -174 | -174 |
| [2F] | Noise Power (dBm) | Calculated (see Note 1) | Calculated (see Note 1) |
| [2G] | Required SNR/CNR | Reported by companies for Budget-Alt2 | Reported by companies for Budget-Alt2 |
| [2H] | Ambient IoT on-object antenna penalty | 0.9 dB or 4.7 dB | Not applicable |
| [2J] | Budget-Alt1/ Budget-Alt2 | Budget-Alt1/ Budget-Alt2 (see note1) | Budget-Alt2 |
| [2K] | CW cancellation (dB) | N/A | Companies to report for scenario A2/A1/B for BS and intermediate UE. Notes: - Only applicable for device 1/2a - The value provided is for the unmodulated single-tone CW. The value for other cases (e.g. multi-tone CW) can be reported by companies. |
| [2K1] | Remaining CW interference (dBm) | N/A | Calculated (see Note 1) Note: only applicable for device 1/2a |
| [2K2] | Receiver sensitivity loss(dB) | N/A | Calculated (see Note 1) Note: only applicable for device 1/2a |
| [2L] | Receiver Sensitivity (dBm) | For Budget-Alt1 For device 1 (RF-ED), for example: {-30 dBm, -36 dBm, -40 dBm, etc} For device 2 (RF-ED), for example: {-40 dBm, -45 dBm, etc} For Budget-Alt2 Calculated (see note1) | Calculated (see Note 1) Note 2L: the receiver sensitivity includes the receiver sensitivity loss [2K2], i.e. after CW cancellation at least if ‘A2’ scenario is used |
| (3) System margins |  |  |  |
| [3A] | Shadow fading margin (dB) | For D1T1: 4 dB For D2T2: 3dB for InH-LOS 7.2dB for InF-DL-NLOS | For D1T1: 4 dB For D2T2: 3dB for InH-LOS 7.2dB for InF-DL-NLOS |
| [3B] | polarization mismatching loss (dB) | 3 dB | 3 dB |
| [3C] | BS selection/macro-diversity gain (dB) | 0 dB Note: only applicable for D1T1 | 0 dB Note: only applicable for D1T1 |
| [3D] | Other gains (dB) (if any please specify) | Reported by companies with justification | Reported by companies with justification |
| (4) MPL / distance |  |  |  |
| [4A] | MPL (dB) | Calculated (see Note 1) | Calculated (see Note 1) |
| [4B] | Distance (m) | Calculated (see Note 1) | Calculated (see Note 1) |
| （5）Other |  |  |  |
| [5A] | Other notes | Companies to report | Companies to report |

For R2D, [4A] = [1M] + [2C] - [2H] - [2L] - [3A] - [3B] + [3C] + [3D], we can get the [2L] = [1M] + [2C] - [2H] - [3A] - [3B] + [3C] + [3D] - pathloss, reference sensitivity = -25.37dBm. In addition, we need to maintain some margin, such as 3dB. The reference sensitivity can be -29dBm.

Proposal 10:  The reference sensitivity can be -29dBm.

- Maximum input level

The purpose of the definition is to prevent the whole link from being saturated, thus causing the deterioration of SNR and performance. According to the investigation for envelop detector, the dynamic range of envelop detector is about 30-45dB, and the reference sensitivity of device 1 is -29dBm. At the same time, we need to maintain some margin, such as 3dB.  The maximum input level can be -2dBm.

Proposal 11: Using dynamic range of device receiver to define maximum input level. The maximum input level can be -2dBm.

Conclusions

In this paper, we provide our views on RF requirements for ambient IoT device, we have made the following proposals.

Proposal 1: We need to assume the tag surrounding material such as cardboard.

Observation1: Modulation loss is 6dB for cardboard material.

Proposal 2: Impairment margin can be 3dB.

Proposal 3: Define backscattering loss as 10dB.

Proposal 4: We need to assume the distance between CW node and tag, such as 15m to get the input CW power.

Proposal 5: Based on the link budget, we can define the minimum backscattered power is -33dBm.

Proposal 6: There is no need to define modulation quality requirements. The modulation quality requirements can be reflected by demodulation requirements.

Proposal 7: To define SFO requirement. If SFO defined, SFO can be obtained by calculating the deviation number of sampling points.

Proposal 8: No need to define occupied bandwidth.

Proposal 9: Define TRP requirements for unwanted emissions. For different channel bandwidth, define D2R channel bandwidth plus minimum D2R channel bandwidth as the boundary between A-IoT out of band and spurious emission domain.

Proposal 10:  The reference sensitivity can be -29dBm.

Proposal 11: Using dynamic range of device receiver to define maximum input level. The maximum input level can be -2dBm.

Reference

- RP-2505230, WF on requirements for system parameters and A-IoT device, CMCC, TSG RAN4 meeting #114bis

- Joshua D. Griffin and Gregory D. Durgin “Complete Link Budgets for Backscatter-Radio and RFID Systems”Georgia Institute of Technology 777 Atlantic Ave., Atlanta, GA 30332-0250 USA

- EPC™ Radio-Frequency Identity Protocols Generation-2 UHF RFID Standard