3GPP TSG-RAN WG4 Meeting #115	R4-2506239

# Malta, MT, 19th – 23rd May, 2025

Source:	LG Electronics

Title:	Discussion on RRM performance requirements for ATG CA operation

Agenda item:	7.12.6

Document for:	Discussion

Introduction

In this contribution, we provide our views on RRM performance requirements for ATG with DL CA operation based on the WF [1].

Discussion

Test applicability rule

For the test applicability rule for ATG CA test, existing applicability rule for test configurations can be considered as below

- UE is only required to be tested in one of the supported test configurations

- UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration

- Proposal 1: for test applicability for ATG CA test,

- UE is only required to be tested in one of the supported test configurations

- UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration

For ATG UE to support common Rx beam, since conducted test without beam sweeping for ATG test cases are considered, RAN4 does not need to consider separate test cases for common Rx beam ATG UE.

- Proposal 2: RAN4 not to consider separate test cases for ATG UE to support common Rx beam

Test setups

Agreement: - Following test setups agreed in R18 can be reused: - TDD pattern: The legacy TDD pattern are used for PCell, FFS for SCell - Channel model: AWGN with residual doppler channel model - Number of neighbour cells: one neighbour cell in the test if needed - GNSS and UE mobility setup: UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system. - BS location: The specific gNB reference location is emulated by test system - RMC: the legacy NR RMCs defined in section A.3.1 of TS 38.133 are reused for ATG test cases. - OCNG: - Generic OCNG pattern for all unused REs defined in A.3.2.1.1 in TS 38.133. - Generic OCNG pattern for unused REs in the same bandwidth as CORESET defined in A.3.2.1.3 of TS 38.133. - Generic OCNG pattern for all unused REs outside SSB slot(s) defined in A.3.2.1.4 in TS 38.133

For test setup for SCell, SCell is always DL since ATG CA is only DL CA. Generally, if PCell is FDD and SCell is TDD, the SCell could be used all DL slots, and if both PCell and SCell are TDD, the same TDD configuration would be used for both PCell and SCell. However, in the test configuration of TDD pattern, the legacy TDD pattern can be used regardless of PCell’s duplex type for simple test configuration.

- Proposal 3: For TDD pattern, the legacy TDD pattern can be used for SCell to simple test configuration.

Test Scope

In the last meeting, the candidate test cases for ATG CA RRM performance were listed. The lists are based on the test cases for TN CA RRM performance. RAN4 needs to discuss whether all test cases should be introduced. To minimize test cases, we provide our views (mark ‘O’ in table) which test cases should be introduced.

- Proposal 4: At least test cases marked ‘O’ in the table below should be introduced

| Test category | Test cases | Comments |
| --- | --- | --- |
| Interruption | The interruption of SCell addition/release | O |
|  | The interruption of SCell activation/deactivation | O |
|  | Interruptions during measurements on deactivated NR SCC in FR1 |  |
|  | Interruption requirement at SRS antenna port switching with 1 SRS symbol in a slot |  |
|  | Interruption requirement at SRS antenna port switching with more than 1 SRS symbol in a slot |  |
|  | Interruption at NR SRS antenna port switching |  |
|  | Interruptions at NR SRS carrier based switching |  |
| SCell Activation and deactivation | SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle | O |
|  | SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle |  |
|  | SCell Activation and deactivation of unknown SCell in FR1 in non-DRX | O |
|  | Direct SCell activation at SCell addition of known SCell in FR1 |  |
|  | Direct SCell activation at handover with known SCell in FR1 |  |
|  | Direct SCell Activation at RRC Resume |  |
|  | Fast SCell Activation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle |  |
|  | Fast SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle |  |
|  | SCell Activation of unknown SCell with valid L3 measurement results in FR1 in non-DRX for 160 ms SCell measurement cycle |  |
|  | TRS based SCell Activation of SSB-less SCell in FR1 inter-band CA in non-DRX |  |
|  | Inter-band SSB-less SCell Activation based on A-TRS |  |
| Beam Failure Recovery | Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode for ATG | O |
| Active BWP Switch | Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs |  |
|  | Simultaneous RRC-based Active BWP Switch on multiple CCs |  |
|  | non-simultaneous Timer based BWP switch delay on multiple CCs |  |
| SCell dormancy switch | SCell dormancy switch |  |
| deactivated SCell measurement | Event triggered reporting on SCC with deactivated SCell test under non-DRX | O |
| Pre-configured measurement gap | Test case for Pre-configured measurement gap requirement |  |
| Others | Intra-frequency measurement(with scheduling restriction verified as well) |  |
|  | Inter-frequency measurement(with scheduling restriction verified as well) |  |
|  | L1-RSRP measurement | O |
|  | CSI-RS based intra and inter frequency Measurements for event-triggered reporting tests without gap under non-DRX |  |

Conclusion

In this contribution, we provide views on remaining issues for ATG with CA operation, and we propose

- Proposal 1: for test applicability for ATG CA test,

- UE is only required to be tested in one of the supported test configurations

- UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration

- Proposal 2: RAN4 not to consider separate test cases for ATG UE to support common Rx beam

- Proposal 3: For TDD pattern, the legacy TDD pattern can be used for SCell to simple test configuration.

- Proposal 4: At least test cases marked ‘O’ in the table below should be introduced

| Test category | Test cases | Comments |
| --- | --- | --- |
| Interruption | The interruption of SCell addition/release | O |
|  | The interruption of SCell activation/deactivation | O |
|  | Interruptions during measurements on deactivated NR SCC in FR1 |  |
|  | Interruption requirement at SRS antenna port switching with 1 SRS symbol in a slot |  |
|  | Interruption requirement at SRS antenna port switching with more than 1 SRS symbol in a slot |  |
|  | Interruption at NR SRS antenna port switching |  |
|  | Interruptions at NR SRS carrier based switching |  |
| SCell Activation and deactivation | SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle | O |
|  | SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle |  |
|  | SCell Activation and deactivation of unknown SCell in FR1 in non-DRX | O |
|  | Direct SCell activation at SCell addition of known SCell in FR1 |  |
|  | Direct SCell activation at handover with known SCell in FR1 |  |
|  | Direct SCell Activation at RRC Resume |  |
|  | Fast SCell Activation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle |  |
|  | Fast SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle |  |
|  | SCell Activation of unknown SCell with valid L3 measurement results in FR1 in non-DRX for 160 ms SCell measurement cycle |  |
|  | TRS based SCell Activation of SSB-less SCell in FR1 inter-band CA in non-DRX |  |
|  | Inter-band SSB-less SCell Activation based on A-TRS |  |
| Beam Failure Recovery | Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode for ATG | O |
| Active BWP Switch | Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs |  |
|  | Simultaneous RRC-based Active BWP Switch on multiple CCs |  |
|  | non-simultaneous Timer based BWP switch delay on multiple CCs |  |
| SCell dormancy switch | SCell dormancy switch |  |
| deactivated SCell measurement | Event triggered reporting on SCC with deactivated SCell test under non-DRX | O |
| Pre-configured measurement gap | Test case for Pre-configured measurement gap requirement |  |
| Others | Intra-frequency measurement(with scheduling restriction verified as well) |  |
|  | Inter-frequency measurement(with scheduling restriction verified as well) |  |
|  | L1-RSRP measurement | O |
|  | CSI-RS based intra and inter frequency Measurements for event-triggered reporting tests without gap under non-DRX |  |

Reference

[1] R4-2504898, “WF on RRM requirements for NR_ATG_enh,” CMCC