3GPP TSG-RAN WG4 Meeting #115	R4-2506242

# Malta, MT, 19th – 23rd May, 2025

Source:	LG Electronics

Title:	Discussion on performance requirements for FR2-1 L3 measurement delay reduction

Agenda item:	7.18.5

Document for:	Discussion

Introduction

In this contritubtion, we provide our views on test cases for L3 measurement delay reduction with multi-Rx simultaneous reception based on [1].

Discussion

In the last meeting, high level test scenarios were agreed below.

Agreement in online session: Further discuss based on the following table: Index; Scenario; Need or not; Detailed configurations and scopes. 1; Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra; 1 or 2 TCs; TBA(e.g., DRX or non-DRX, testing quitting FBS or not). 3; Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter; TBA(e.g., DRX or non-DRX, testing quitting FBS or not). 2; Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra; 1 or 2 TCs; TBA(e.g., DRX or non-DRX, testing quitting FBS or not). 4; Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter; TBA(e.g., DRX or non-DRX, testing quitting FBS or not). 5; Scenario 5: Handover; 1 or 2 TCs; TBA(e.g., Intra-frequency or inter-frequency, unknown). 6; Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection; 1 or 2 TCs; TBA(e.g., re-establishment or RRC redirection, Intra-frequency or inter-frequency, [or known] or unknown).

For scenarios 1 to 4, RAN4 is discussing the condition for deactivation of FBS L3 measurement in core requirements. However, it is up to UE implementation whether to deactive FBS L3 measuerment after the conditions to quitting FBS are met. Therefore, RAN4 does not need to introduce test case for deactivation of FBS L3 measurements.

- Proposal 1: Do not introduce test cases for quitting FBS L3 measurements for scenario 1 to 4.

From the functional test perspective, since the purpose of the test is to verify the FBS operation according to the triggering condition, instead of introducing test cases based on all existing test case scenarios, it is sufficient to introduce only one test case that can verify the operation for each intra-frequency and inter-frequency measurements.

- Proposal 2: Introduce test cases to verify UE behaviors of FBS L3 measurements below.

| Index | Scenario | Number of TC | Proposed detailed configurations and scopes |
| --- | --- | --- | --- |
| 1 | Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra | 1 | - Intra-frequency measurement - Non-DRX - Reuse existing test configurations in FR2-1 adding trggering condition |
| 3 | Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter |  |  |
| 2 | Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra | 1 | - Inter-frequency measurement - Non-DRX or DRX - Reuse existing test configurations in FR2-1 adding trggering condition |
| 4 | Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter |  |  |
| 5 | Scenario 5: Handover | 1 | - Intra-frequency handover from FR2 to FR2 - Unknown target cell - Reuse existing test configurations in FR2 Handover |
| 6 | Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection | 2 | - Intra-frequency RRC Re-establishment; unknown cell - RRC Connection release with redirection - Reuse existing test configurations in FR2 |

Conclusion

In this contribution, we provide views on test cases for multi-Rx simultaneous reception, and we propose

- Proposal 1: Do not introduce test cases for quitting FBS L3 measurements for scenario 1 to 4.

- Proposal 2: Introduce test cases to verify UE behaviors of FBS L3 measurements below.

| Index | Scenario | Number of TC | Proposed detailed configurations and scopes |
| --- | --- | --- | --- |
| 1 | Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra | 1 | - Intra-frequency measurement - Non-DRX - Reuse existing test configurations in FR2-1 adding trggering condition |
| 3 | Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter |  |  |
| 2 | Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra | 1 | - Inter-frequency measurement - Non-DRX or DRX - Reuse existing test configurations in FR2-1 adding trggering condition |
| 4 | Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter |  |  |
| 5 | Scenario 5: Handover | 1 | - Intra-frequency handover from FR2 to FR2 - Unknown target cell - Reuse existing test configurations in FR2 Handover |
| 6 | Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection | 2 | - Intra-frequency RRC Re-establishment; unknown cell - RRC Connection release with redirection - Reuse existing test configurations in FR2 |

Reference

[1] R4-2504899, “WF on RRM requirements for NR_RRM_Ph5_Part1,” Apple.