3GPP TSG-RAN WG4 Meeting #115	R4-2506253

St Julian’s, Malta, 19th – 23rd May 2025

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on Rx requirements for NR NTN HPUE

# Agenda item:	7.10.3.2.2

# Document for:	Approval

# 1 Introduction

In RAN4#114 meeting, the following WF on Rx requirements for NR NTN HPUE[1] was approved. However, some remaining issues need further discussion. In this contribution, we want to share some further views about NR NTN HPUE from Rx perspective.

- Sub-topic 1-3 Rx requirements Issue 1-10: General discussion of RSD analysis Ad-hoc agreements: - In addition to Tx leakage to Rx, Rx non-linearity needs to be considered in RSD evaluation. Issue 1-11: General discussion of RSD of PC1.5 and PC1 Ad-hoc agreements: - Companies agree to bring more analysis for the RSD values for PC1.5/PC1 non-handheld UE. - Further check these in next meeting. Issue 1-13: RSD requirements for PC2 on n255 Ad-hoc agreements: - For n255 PC2: 5MHz; 10MHz; 15 MHz; 20 MHz. RSD@1Tx; [0.8]; [0.8]; [0.9]; [0.9]. RSD@2Tx; [1.6]; [1.6]; [1.7]; [1.7]. Issue 1-14: RSD requirements for PC1.5 and PC1. Way forward: - Encourage companies to further discuss the following proposals: - Option 1: For PC1.5 2Tx (MTK) Operating Band; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB). n256; 2.2; 2.2; 2.2; 2.2. n255; 3.2; 3.2; 3.3; 3.3. n254; 2.2; 2.2; 2.2. - Option 2: For PC1 1Tx (MTK) Operating Band; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB). n256; [3]; [3]; [3]; [3]. n254; [3]; [3]; [3]. Issue X: Proposal to allow power reduction beyond certain duty cycle Way forward: - Encourage companies to further discuss the following proposal: - Option 1: For NR NTN PC2 and PC1.5, and IoT NTN PC2, allow but not mandate UE to reduce it’s transmit power when 50% Tx duty cycle in a radio frame is exceeded. (Qualcomm)

# 2 Discussion

  - 2.1 General discussion of RSD of PC1.5 and PC1

Based on the agreements in last few meetings, support Tx number and device type (handheld and/ or non-handheld) can be summarized as follows:

Table 2.1-1: NR-NTN HPUE

| Power Class | Antenna Numbers | Handheld and/or non-handheld |
| --- | --- | --- |
| PC2 | 1Tx, 2Tx | Both |
| PC1.5 | 2Tx * | Non-handheld * |
| PC1 | 1Tx, [4Tx] | Non-handheld |
| Note *: FFS on other cases if time is allowed |  |  |

From Table 2.1-1, we can observe that only non-handheld UE was agreed for PC1.5 and PC1. During TN HPUE discussion, PC1 and PC1.5 UE also only support non-handheld type, and there is no RSD defined for PC1 and PC1.5 since non-handheld form factor terminals can provide better isolation performance than handheld types. From this perspective, there is no need to specify RSD if only non-handheld HPUE is introduced for PC1.5 and PC1.

Proposal 1: Not to specify RSD if only non-handheld HPUE is introduced for PC1.5 and PC1.

# 3 Conclusion

In this contribution, we shared some views on Rx requirements for NR NTN HPUE and the proposals are made as follows:

Proposal 1: Not to specify RSD if only non-handheld HPUE is introduced for PC1.5 and PC1.

# 4 Reference

- R4-2505113, WF on HPUE RF requirements for NR-NTN, Samsung