3GPP TSG-RAN WG4 Meeting #115	R4-2506265

St Julian’s, Malta, 19th – 23rd May 2025

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on UE RF requirements for NR NTN phase3

# Agenda item:	7.31.3.1

# Document for:	Approval

# 1 Introduction

In RAN#102 meeting, a new Work Item [1] was approved on Non-Terrestrial Networks (NTN) for NR Phase 3 with the following RAN4 objective. In this contribution, we want to share some further views from RedCap UE RF perspective.

- Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, check whether any essential changes are needed for their support (i.e. focusing on HD collision rules) by end of Q2/2024 [RAN1] - Depending on feasibility assessment above, define the RF and RRM requirements [RAN4] - Notes for this objective: - GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

# 2 Discussion

  - 2.1 Output power enhancement for NR NTN PC3

- Issue 1-1: Output power enhancement for NR NTN PC3 Agreements: - If defined, additional output power enhancement will be optional for UE to support. - FFS on need of new UE capability - Evaluate potential to further enhance output power for RAN4#115: - Amount of output power enhancement [0.5 to 1 dB] - Less than 0.5 dB improvement is not specified - Initially consider all RB allocations and modulation orders - Consider both MPR reduction and MOP increase - FFS how to capture MOP increase in the specification - Choice between MPR reduction and MOP increase can be different for inner/outer allocations and modulation orders - Evaluate both HD-FDD and FD-FDD - Consider at least insertion loss impact - FFS whether to apply same enhancement to both HD-FDD and FD-FDD - Evaluate the option to support both rel-18 and rel-19 output power enhancement schemes simultaneously - FFS whether simultaneous support is specified - Consider whether PC3 or PC2 ACLR requirement should apply - FFS Applicability to PC3 non-RedCap NR NTN

The above is the agreement and way forward we reached in last meeting [2]. Due to lower RF front-end insertion loss,  additional output power enhancement will be introduced as optional feature for (e)RedCap UE. The potential output power enhancement is based on the hardware capability. It is also different from Rel-18 power boosting scheme. Hence, a new UE capability is needed to support additional output power enhancement for (e)RedCap UE.

Proposal 1: A new UE capability is needed to support additional output power enhancement for (e)RedCap UE.

Regarding Rel-18 and Rel-19 output power enhancement schemes, there are entirely different schemes. Rel-18 power boosting is based on frequency domain spectrum shaping with and without spectrum extension, and only applicable to inner RB allocation. For Rel-19 output power enhancement, it is motivated by hardware capability, and could be applied to all Rb allocations. From our understanding, Rel-18 and Rel-19 output power enhancement schemes could be support simultaneously.

Proposal 2: Rel-18 and Rel-19 output power enhancement schemes could be support simultaneously.

In terms of applicable ACLR, we agreed that target power class is PC3 for NTN (e)RedCap UE in RAN4 #110bis meeting. Take Rel-18 power boosting as an example, PC3 ACLR requirement is applied to PC3 UE when Rel-18 power boosting capability is supported. And the following is cited from 38.101-1.

When the IE powerBoostPi2BPSK-r18 or powerBoostQPSK-r18 is set to 1 for a UE supporting the capability of powerBoosting-pi2BPSK-QPSK-r18 or capability of powerBoosting-pi2BPSK-QPSK-Modified-r18, for power class 2 UE, the ACLR requirement of PC2 applies. For power class 3 UE, the ACLR requirement of PC3 applies.

For output power enhancement, the out of band emissions of the corresponding power class should be guaranteed. Hence, for NTN PC3 (e)RedCap UE, PC3 ACLR requirement should apply.

Proposal 3: For NTN PC3 (e)RedCap UE, PC3 ACLR requirement should apply.

  - 2.2 Simultaneous operation with NR NTN and GNSS

The above is reply LS from RAN2 to ask feedback on both duration and periodicity of the IDC free time to be applied to IDC TDM or autonomous denial solution. For IDC free time, we can use the value in IE GNSS-PositionFixDuration as a basis. IE GNSS-PositionFixDuration is introduced in 36.331, and indicates the time duration required for the UE to acquire a GNSS position. The values indicated in IE GNSS-PositionFixDuration are from 1 seconds to 31 seconds to accommodate various scenarios. In reality, GNSS receivers need to go through different start modes—cold, warm, and hot—each impacting the time to first fix (TTFF), or the time it takes for the receiver to compute its initial position. Cold start mode usually need above 20 seconds, while warm start mode needs about 10-20 seconds. For simultaneous operation with NR NTN and GNSS, we only need to consider hot start mode. Hence, the duration of IDC free time could be less than 10 seconds.

Proposal 4: The duration of IDC free time could be less than 10 seconds.

- – GNSS-PositionFixDuration The IE GNSS-PositionFixDuration indicates the time duration required for the UE to acquire a GNSS position. Value s1 corresponds to 1 second, s2 corresponds to 2 seconds and so on. GNSS-PositionFixDuration information element -- ASN1START GNSS-PositionFixDuration-r18 ::= ENUMERATED{ s1, s2, s3, s4, s5, s6, s7, s13, s19, s25, s31} -- ASN1STOP

# 3 Conclusion

In this contribution, we share some views on UE RF requirements for NR NTN phase3 and the proposals are made as follows:

Proposal 1: A new UE capability is needed to support additional output power enhancement for (e)RedCap UE.

Proposal 2: Rel-18 and Rel-19 output power enhancement schemes could be support simultaneously.

Proposal 3: For NTN PC3 (e)RedCap UE, PC3 ACLR requirement should apply.

Proposal 4: The duration of IDC free time could be less than 10 seconds.

# 4 Reference

- RP-234078, New WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Huawei

- R4-2504708, Way Forward for [114bis][309] NR_NTN_Ph3_General_UE_SAN_RF, Qualcomm Inc.