3GPP TSG-RAN WG4 Meeting # 115	R4-2506293

# Malta, MT, 19th – 23th May, 2025

Source:	LG Electronics

Title:	Discussion on SAN EIS requirements for NR NTN Ku band

Agenda item:	7.11.6

Document for:	Discussion

Introduction

In this contribution, we want to share some views about OTA reference sensitivity level for the Ku band SAN, which does not have agreement at way forward R4-2504720[1].

Discussion

1.1 Validation Check for Satellite Parameters

Table 1 shows system parameter from way forward R4-2504723[3] for Ku band and TR38.821 [2] for Ka and S band.

Table 1 Satellite Parameters

| Satellite orbit |  | GEO | LEO-600 |
| --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 600 km |
| Payload characteristics for UL transmissions |  | Set1 |  |
| Equivalent satellite antenna aperture (Note1) | S-band | 22 m | 2 m |
| G/T (dB/K) | (i.e. 2 GHz for UL) | 19 | 1.1 |
| Satellite RX max Gain (dBi) |  | 51 | 30 |
| Equivalent satellite antenna aperture (Note1) | Ku-band | 4 m | 0.714 m |
| G/T (dB/K) | (i.e. 14 GHz for UL) | 16.5 | 10.9 |
| Satellite RX max Gain (dBi) |  | 44 | 38.5 |
| Equivalent satellite antenna aperture (Note1) | KA-band | 3.33 m | 0.33 m |
| G/T (dB/K) | (i.e. 30 GHz for UL) | 28 | 13 |
| Satellite RX max Gain (dBi) |  | 58.5 | 38.5 |

From the information Satellite RX max Gain & G/T, we can get system noise temperature using eq. (1) [4,5]

TSYS=10((Grx -G/T)/10)                                                            (1)

Using Eq(1), the system noise temperature was calculated. As shown in Table 2, the result for the Ku band in GEO exhibits a notably improved performance, with a system noise temperature of 565 K. This observation is rather unusual, particularly when compared to the system noise temperatures of the S band and Ka band, both of which exceed 1000 K.(1584.K and 1122.0 K). In this regard, two potential explanations can be considered: (1) the Ku band system in GEO may exhibit inherently lower noise characteristics due to optimized RF front-end design and superior thermal management, which are typically less achievable in other bands or LEO systems, or (2) the system noise temperature for the Ku band may have been underestimated.  Assuming the latter — that the Ku band value was underestimated — an adjusted system noise temperature for the Ku band was derived. This was calculated by applying a linear interpolation approach based on the system noise temperatures of the S band and Ka band.  The adjusted Ku band system noise temperature is summarized in Table 2.

Table 2 System Noise Temperature

| Frequency (GHz) | Original System Noise Temperature (K) (calculated with antenna gain and G/T from Table 1) |  | Adjusted System Noise Temperature (K) (Ku band, via interpolation) |  |
| --- | --- | --- | --- | --- |
|  | GEO | LEO 600 | GEO | LEO 600 |
| 2 (S band) | 1584.9 | 776.2 | 1584.9 | 776.2 |
| 14 (Ku band) | 562.3 | 575.4 | 1288.0 | 585.6 |
| 30 (Ka band) | 1122.0 | 354.8 | 1122.0 | 354.8 |

For the LEO Ku band, the values in Table 1 ware based on a typical dish antenna efficiency, while for the GEO Ku band, equation(2) confirms that a much lower efficiency of 7~8% was assumed.  In table 2, the adjusted system noise temperature for the ku band, estimated using linear interpolation from the S and Ka bands, shows a minimal gap for LEO600 (about 10K), resulting in a negligible G/T  change (<0.1 dB/K).  In contrast, GEO shows a large gap of approximately 726K, leading to a significant G/T deviation.  As a result, assuming 62% antenna efficiency and the adjusted system noise temperature, the estimated Rx max antenna gain and G/T are approximately 53 dBi and 21.9 dB/K.

G=10log10k(πDλ)2                                                              (2)

Where k : antenna efficiency (typically 60~65%), D : equivalent satellite antenna aperture (Diameter)

Table 3 presents the corrected values of Table 1

Table 3 Satellite Parameters after adjust system noise temperature

| Satellite orbit |  | GEO | LEO-600 |
| --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 600 km |
| Payload characteristics for UL transmissions |  | Set1 |  |
| Equivalent satellite antenna aperture (Note1) | S-band | 22 m | 2 m |
| G/T (dB/K) | (i.e. 2 GHz for UL) | 19 | 1.1 |
| Satellite RX max Gain (dBi) |  | 51 | 30 |
| Equivalent satellite antenna aperture (Note1) | Ku-band | 4 m | 0.714 m |
| G/T (dB/K) | (i.e. 14 GHz for UL) | 16.5 21.9 | 10.9 |
| Satellite RX max Gain (dBi) |  | 44 53 | 38.5 |
| Equivalent satellite antenna aperture (Note1) | KA-band | 3.33 m | 0.33 m |
| G/T (dB/K) | (i.e. 30 GHz for UL) | 28 | 13 |
| Satellite RX max Gain (dBi) |  | 58.5 | 38.5 |

1.2 Additional Loss directly affecting EIS degradation for S- and KA- band LEO

Given a specified G/T, the effective Isotropic Sensitivity (EIS) can be calculated using the following equation [4,5]

EIS=KB+10LOG10BW-G/T                                                          (3)

Where KB : the thermal noise powr spectral density, referred to 1 K and 1 Hz bandwidth, is -198.6 dBm/K/Hz (=10LOG101.38*10-23+30), BW :channel BW, and G/T : Antenna gain-to-noise-temperature (dB/K)

The Upperbound of EISREFSENS_50M for KA band of GEO can be calculated as  149.8 (=KB+10LOG1060*66*12*103-G/T),  Where G/T is 28 dB/k. And the integer value of -149.8 dBm is rounded down to [-149] dBm.  The lower bound corresponds to an EIS of [-140] dBm is 9 dB less sensitive compared to the upper bound.

As I understand, Although the Saltllite Rx max gain is fixed, the EIS can vary by up to 9 dB across different vendors due to implementation-dependent factors.  That is; EIS is inherently dependent on the design quality and receiver adopted by each manufacturer.

EIS_upperbound=KB+10LOG10BW-G/T                                                               (4)

EIS_lowerbound=KB+10LOG10BW-G/T+(I.M.=9dB)                                        (5)

The Upperbound of EISREFSENS_50M for KA band of LEO can be calculated as  134.8 (=KB+10LOG1060*66*12*103-G/T), Where G/T is 13 dB/k. And the integer value of -134.8 dBm is represented as [-134] dBm. However, as defined in the 3GPP specification, the upper bound EIS for the Ka band of LEO is specified as [-129] dBm. This implies that an additional implementation/operating loss of approximately 5 dB has been accounted for in the Ka band of LEO. As demonstrated in the table4, this additional implementation/operating loss term was not considered in the case of the S band of LEO.

Therefore, as specified in TS38.108, EISREFSENS_50M for KA band is defined as an integer value in the range of [-140] to [-149] dBm for GEO class SAN and [-120] to [-129] dBm for LEO class SAN, with the specific value declared by the vendor in each case.

Table 4 EIS for S band of LEO

| Reference channel | OTA reference sensitivity level, EISREFSENS |  |  | EIS using G/T |
| --- | --- | --- | --- | --- |
|  |  | ΔOTAREFSENS = 31.2 | ΔOTAREFSENS = 30 | G/T = 1.1 |
| G-FR1-NTN-A1-7 | -104.3 - ΔOTAREFSENS | -135.5 | -134.3 | -135.4 |
| G-FR1-NTN-A1-1 | -102.4 - ΔOTAREFSENS | -133.6 | -132.4 | -133.2 |
| G-FR1-NTN-A1-2 | -102.5 - ΔOTAREFSENS | -133.7 | -132.5 | -133.7 |
| G-FR1-NTN-A1-3 | -99.6 - ΔOTAREFSENS | -130.8 | -129.6 | -130.7 |
| G-FR1-NTN-A1-4 | -96.0 - ΔOTAREFSENS | -127.2 | -126 | -126.9 |
| G-FR1-NTN-A1-5 | -96.3 - ΔOTAREFSENS | -127.5 | -126.3 | -127.1 |
| G-FR1-NTN-A1-6 | -96.4 - ΔOTAREFSENS | -127.6 | -126.4 | -127.3 |
| ΔOTAREFSENS = 44.1 - 10 * log10(BeWθ,REFSENS * BeWφ,REFSENS) dB represents the antenna gain [4] derived by considering the typical efficiency (η = 62%) of the dish antenna. ΔOTAREFSENS = 31.2 (BeWθ,REFSENS = BeWφ,REFSENS = 4.4127 degree [TS38.108] ΔOTAREFSENS = 30 (Rx Max Gain from Table1) |  |  |  |  |

1.3 Additional Loss of LEO Ku band affecting EIS degradation

In high-mobility scenarios such as those involving Low Earth Orbit (LEO) satellites, beam misalignment is a key factor that can lead to degradation in effective isotropic sensitivity (EIS). The primary cause is tracking latency due to rapid satellite movement and associated Doppler shifts. Additional contributors include re-beamforming delays from limited update periodicity and beam squint caused by wideband frequency dispersion. These effects reduce the received antenna gain in the desired direction, directly impacting EIS performance, especially for high mobility, narrow-beam, high-frequency, wideband  systems. In contrast, Geostationary Earth Orbit (GEO) satellites experience negligible beam misalignment due to their fixed relative position and lack of Doppler-induced or tracking-related effects.

Based on the equivalent satellite antenna apertures and operating frequencies provided in Table 1, the estimated 3 dB beamwidths (HPBW= 60λ/D) are approximately 4.4° for S-band (2 GHz) and 1.8° for both Ku-band (14 GHz) and Ka-band (30 GHz), indicating that the HPBW of Ku- and Ka-band systems are nearly identical.

For LEO systems operating in the S-band, due to the wide HPBW and very narrow fractional bandwidth, the impact of beam misalignment is minimal, resulting in negligible EIS performance degradation.  In contrast, for LEO systems operating in Ku- and Ka-band, the HPBW is comparable, and thus the impact of beam misalignment, which is primarily caused by beam tracking latency and re-beamforming delays, is expected to be nearly identical.  Furthermore, the beam squint loss for n509 and n511, which is influenced by HPBW, is not only relatively small but also can be reduced to a negligible level through techniques such as codebook-based compensation.

Therefore, in the context of Ka-band LEO systems, the additional implementation and operating margin of around 5 dB—which is assumed due to unmeasurable beam misalignment in G/T measurements—is also considered reasonable to apply to Ku-band LEO systems under similar operating conditions.

1.4 FR1 & FR2-NTN OTA reference sensitivity for Ku band

Table 5 and Table 6 show various channel BWs of Ku band in logarithmic scale for FR1 and FR2

Table 5 BW in logarithmic scale for FR1

| SCS (kHz) | 10 | 20 | 25 | 35 | 50 | 70 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | MHz | MHz | MHz | MHz | MHz | MHz | MHz |
|  | 10*LOG10 (BW) | 10*LOG10 (BW) | 10*LOG10 (BW) | 10*LOG10 (BW) | 10*LOG10 (BW) | 10*LOG10 (BW) | 10*LOG10 (BW) |
| 15 | 69.71 | 72.81 | 73.79 | 75.29 | 76.87 | N/A | N/A |
| 30 | 69.37 | 72.64 | 73.69 | 75.20 | 76.80 | 78.33 | 79.92 |

Table 6 BW in logarithmic scale for FR2

| SCS (kHz) | 50 | 100 | 200 | 400 |
| --- | --- | --- | --- | --- |
|  | MHz | MHz | MHz | MHz |
|  | 10*LOG10(BW) | 10*LOG10(BW) | 10*LOG10(BW) | 10*LOG10(BW) |
| 120 | 76.64 | 79.78 | 82.79 | 85.80 |

According to Table 1 and Table 3, where the G/T for the Ku band of GEO is given as 16.5 and 21.9 respectively, the upper bound EIS values have been calculated for both cases as follows, depending on the channel bandwidths (BWs).

Tables 7 and 8 present the upper bound EIS values for the Ku band of GEO in FR1 and FR2 channel bandwidths, as specified in Tables 3 and 4.

Table 7 EIS_upperbound Ku band of GEO for FR1

| G/T = 16.5 | SCS (kHz) | 10 | 20 | 25 | 35 | 50 | 70 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MHz | MHz | MHz | MHz | MHz | MHz | MHz |
|  |  | EIS upperbound | EIS upperbound | EIS upperbound | EIS Upperbound | EIS upperbound | EIS upperbound | EIS upperbound |
|  | real value (15KHz) | -145.39 | -142.30 | -141.31 | -139.81 | -138.23 |  |  |
|  | real value (30KHz) | -145.74 | -142.46 | -141.41 | -139.90 | -138.30 | -136.77 | -135.18 |
|  | integer value w/ difference | -7 | -4 | -3 | -1.5 | [-138] | 1.5 | 3 |
| G/T = 21.9 | SCS (kHz) | 10 MHz | 20 MHz | 25 MHz | 35 MHz | 50 MHz | 70 MHz | 100 MHz |
|  |  | EIS upperbound | EIS upperbound | EIS upperbound | EIS Upperbound | EIS upperbound | EIS upperbound | EIS upperbound |
|  | real value (15KHz) | -150.79 | -147.7 | -146.71 | -145.21 | -143.63 |  |  |
|  | real value (30KHz) | -151.14 | -147.86 | -146.81 | -145.3 | -143.7 | -142.17 | -140.58 |
|  | integer value w/ difference | -7 | -4 | -3 | -1.5 | [-143] | 1.5 | 3 |

Table 8 EIS_upperbound Ku band of GEO for FR2

| G/T = 16.5 | SCS (kHz) | 50 | 100 | 200 | 400 |
| --- | --- | --- | --- | --- | --- |
|  |  | MHz | MHz | MHz | MHz |
|  | 120 KHz | EISupperbound | EISupperbound | EISupperbound | EISupperbound |
|  | real value | -138.47 | -135.32 | -132.31 | -129.30 |
|  | integer value w/difference | [-138] | 3 | 6 | 9 |
| G/T = 21.9 | SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
|  | 120 KHz | EISupperbound | EISupperbound | EISupperbound | EISupperbound |
|  | real value | -143.87 | -140.72 | -137.71 | -134.7 |
|  | integer value w/ difference | [-143] | 3 | 6 | 9 |

According to Table 1 and Table 3, The G/T is given as 10.9 for the Ku band of LEO, therefore the upper bound EIS is calculated as follows, depending on the channel bandwidths (BWs).

Tables 9 and 10 present the upper bound EIS values including additional loss for the Ku band of LEO in FR1 and FR2 channel bandwidths, as specified in Tables 3 and 4.

Table 9 EIS_upperbound Ku band of LEO for FR1

| SCS (kHz) | 10 | 20 | 25 | 35 | 50 | 70 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | MHz | MHz | MHz | MHz | MHz | MHz | MHz |
|  | EIS upperbound | EIS upperbound | EIS upperbound | EIS upperbound | EIS upperbound | EIS upperbound | EIS upperbound |
| real value (15KHz) | -134.79 | -131.70 | -130.71 | -129.21 | -127.63 |  |  |
| real value (30KHz) | -135.14 | -131.86 | -130.81 | -129.30 | -127.70 | -126.17 | -124.58 |
| integer value w/ difference | -7 | -4 | -3 | -1.5 | [-127] | +1.5 | +3 |

Table 10 EIS_upperbound Ku band of LEO for FR2

| SCS (kHz) | 50 | 100 | 200 | 400 |
| --- | --- | --- | --- | --- |
|  | MHz | MHz | MHz | MHz |
| 120 KHz | EISupperbound | EISupperbound | EISupperbound | EISupperbound |
| real value | -127.87 | -124.72 | -121.71 | -118.70 |
| integer value w/ difference | [-127] | +3 | +6 | +9 |

Therefore, we can conclude:

For GEO class SAN, EISREFSENS_50M  for Ku band is an integer value in the range -129 to -138 dBm. The specific value is declared by the vendor. (If the Table 1 [3] is valid)

For GEO class SAN, EISREFSENS_50M  for Ku band is an integer value in the range -134 to -143 dBm. The specific value is declared by the vendor. (If typical antenna efficiency and adjusted system noise temperature are applied. Table 3)

For LEO class SAN, EISREFSENS_50M for Ku band is an integer value in the range -118 to -127 dBm. The specific value is declared by the vendor.

Or in the future, G/T improvement may be achieved by improving LNA characteristics, reducing antenna and feed losses and optimizing the operating environment conditions.  Therefore, defining EISREFSENS_50M as a range is not required.

For GEO class SAN, EISREFSENS_50M  for Ku band, which is represented as an integer value, is required to be better than -129 dBm. The specific value is declared by the vendor. (If the Table 1 [3] is valid)

For GEO class SAN, EISREFSENS_50M for Ku band, which is represented as an integer value, is required to be better than -134 dBm. The specific value is declared by the vendor. (If typical antenna efficiency and adjusted system noise temperature are applied. Table 3)

For LEO class SAN, EISREFSENS_50M for Ku band, which is represented as an integer value, is required to be better than -118 dBm.  The specific value is declared by the vendor.

Table 11: FR1-NTN OTA reference sensitivity requirement for Ku band

| SAN channel Bandwidth (MHz) | Sub-carrier spacing (kHz) | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- |
| 10 | 15, 30 | EISREFSENS_50M -7 ΔFR1_REFSENS |
| 20 | 15, 30 | EISREFSENS_50M - 4+ ΔFR1_REFSENS |
| 25 | 15, 30 | EISREFSENS_50M - 3+ ΔFR1_REFSENS |
| 35 | 15, 30 | EISREFSENS_50M – 1.5+ ΔFR1_REFSENS |
| 50 | 15, 30 | EISREFSENS_50M + ΔFR1_REFSENS |
| 70 | 15, 30 | EISREFSENS_50M – 1.5+ ΔFR1_REFSENS |
| 100 | 15, 30 | EISREFSENS_50M + 3+ ΔFR1_REFSENS |

Table 12: FR2-NTN OTA reference sensitivity requirement for Ku band

| SAN channel Bandwidth (MHz) | Sub-carrier spacing (kHz) | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- |
| 50 | 120 | EISREFSENS_50M + ΔFR2_REFSENS |
| 100 | 120 | EISREFSENS_50M + 3+ ΔFR2_REFSENS |
| 200 | 120 | EISREFSENS_50M + 6+ ΔFR2_REFSENS |
| 400 | 120 | EISREFSENS_50M + 9+ ΔFR2_REFSENS |

Conclusion

In this contribution, we want to share some views on SAN RF requirements (OTA ref. sensitivity level) for NR NTN ku band and proposals are made as following

Observation 1. For the LEO Ku band, the values in Table 1 ware based on a typical dish antenna efficiency about 60 ~ 65%, while for the GEO Ku band, eq.(2) confirms that a much lower efficiency of 7 ~8 % was assumed.  In table 2, the result for the Ku band in GEO exhibits a notably improved performance, with a system noise temperature of 562.3 K. This observation is rather unusual, particularly when compared to the system noise temperatures of the S band and Ka band, both of which exceed 1000 K (1584.9K and 1122.0 K), leading to a significant G/T deviation.  In contrast, a minimal gap of system noise temperature for LEO600 (about 10K), resulting in a negligible G/T deviation (<0.1 dB/K).

Observation 2. In defining the EIS reference sensitivity for S band and Ka band in LEO systems, additional losses directly affecting the EIS have been considered for Ka band. It is observed that the estimated HPBW(= 60λ/D)  for Ku-band and Ka-band systems is nearly identical (~1.8°), which suggests comparable tolerance to angular beam misalignment, and therefore similar susceptibility to tracking latency, beam update intervals, and associated EIS degradation.

Observation 3. It is observed that a 50 MHz channel bandwidth is supported in both FR1 and FR2.

Observation 4. In FR2-NTN, OTA REFSENS RoAoA absolute requirement values are defined with an angular offset, specifically −3 dB for the reference direction and 0 dB for all other directions.  However, for FR1 frequencies, no such offset requirement is defined, as the original FR1 specifications did not cover frequencies above 10 GHz.  Given that Ku-band SAN systems operate above 10 GHz while following FR1 numerology, the absence of a corresponding FR1_REFSENS offset leads to inconsistency in evaluating directional OTA performance.

Observation 5. Except for 35 and 70 MHz channel bandwidths, the differences from the 50 MHz reference channel bandwidth can be represented as integer values.  In the case of 35 and 70 MHz, however, the deviations are almost ±1.5 dB relative to the 50 MHz reference.

Proposal 1. It is proposed to define EISREFSENS based on the G/T (=21.9) calculated from the adjusted system noise temperature with 53 dBi Rx max antenna gain, which is 62% typical antenna efficiency, as this approach could provide a more reasonable and consistent reference. However, if the G/T (=16.5) with Rx max antenna gain (44dBi), which corresponding 7~8% antenna efficiency  from Table 1 is deemed valid, it may be used.

Proposal 2. It is proposed to use EISREFSENS_50M for Ku band commonly for both FR1 and FR2 for simplicity. This is because, if different reference channel bandwidths for FR1 and FR2 are applied, additional statements would be required to define them.

Proposal 3. It is proposed that we need to define ΔFR1_REFSENS specifically −3 dB for the reference direction and 0 dB for all other directions.

Proposal 4. It is proposed to define OTA REFSENS for both FR1-NTN Ku bands and FR2-NTN Ku bands as follows

Therefore, we can conclude:

For GEO class SAN, EISREFSENS_50M  for Ku band is an integer value in the range -129 to -138 dBm. The specific value is declared by the vendor. (If the Table 1 [3] is valid)

For GEO class SAN, EISREFSENS_50M  for Ku band is an integer value in the range -134 to -143 dBm. The specific value is declared by the vendor. (If typical antenna efficiency and adjusted system noise temperature are applied. Table 3)

For LEO class SAN, EISREFSENS_50M for Ku band is an integer value in the range -118 to -127 dBm. The specific value is declared by the vendor.

Or in the future, G/T improvement may be achieved by improving LNA characteristics, reducing antenna and feed losses and optimizing the operating environment conditions.  Therefore, defining EISREFSENS_50M as a range is not required.

For GEO class SAN, EISREFSENS_50M  for Ku band, which is represented as an integer value, is required to be better than -129 dBm. The specific value is declared by the vendor. (If the Table 1 [3] is valid)

For GEO class SAN, EISREFSENS_50M for Ku band, which is represented as an integer value, is required to be better than -134 dBm. The specific value is declared by the vendor. (If typical antenna efficiency and adjusted system noise temperature are applied. Table 3)

For LEO class SAN, EISREFSENS_50M for Ku band, which is represented as an integer value, is required to be better than -118 dBm.  The specific value is declared by the vendor.

Proposal 5. It is proposed to define FR1 and FR2 OTA reference sensitivity requirement for Ku bands as follows :

Table 11: FR1-NTN OTA reference sensitivity requirement for Ku band

| SAN channel Bandwidth (MHz) | Sub-carrier spacing (kHz) | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- |
| 10 | 15, 30 | EISREFSENS_50M -7 ΔFR1_REFSENS |
| 20 | 15, 30 | EISREFSENS_50M - 4+ ΔFR1_REFSENS |
| 25 | 15, 30 | EISREFSENS_50M - 3+ ΔFR1_REFSENS |
| 35 | 15, 30 | EISREFSENS_50M - 1.5+ ΔFR1_REFSENS |
| 50 | 15, 30 | EISREFSENS_50M + ΔFR1_REFSENS |
| 70 | 15, 30 | EISREFSENS_50M + 1.5+ ΔFR1_REFSENS |
| 100 | 15, 30 | EISREFSENS_50M + 3+ ΔFR1_REFSENS |

Table 12: FR2-NTN OTA reference sensitivity requirement for Ku band

| SAN channel Bandwidth (MHz) | Sub-carrier spacing (kHz) | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- |
| 50 | 120 | EISREFSENS_50M + ΔFR2_REFSENS |
| 100 | 120 | EISREFSENS_50M + 3+ ΔFR2_REFSENS |
| 200 | 120 | EISREFSENS_50M + 6+ ΔFR2_REFSENS |
| 400 | 120 | EISREFSENS_50M + 9+ ΔFR2_REFSENS |

- Reference

[1] R4-2504720, Way Forward for [114bis][314] NR_NTN_Ku_Band_UE_SAN_RF, Moderator (CHTTL)

[2] 3GPP TR 38.821 V16.2.0  Solutions for NR to support non-terrestrial networks (NTN)

[3] R4-2504723, Way Forward for  [114bis][313] NR_NTN_Ku_Band_Coexistence, Moderator: Intelsat

[4] T. A. Milligan, “Modern Antenna Design,” 2nd Edition, John Wiley & Sons, Inc., Hoboken, 2005

[5] Soyi Jung, " Fundamental Link Budget Analysis for Low Earth Orbit Satellite Downlink Communications," 2023 ICTC,