3GPP TSG-RAN WG4 Meeting # 115	R4- 2506300

# Malta, MT, 19th – 23rd May, 2025

Source:	LG Electronics

Title:	Discussion on RRM impacts of UE-to-UE CLI handling

Agenda item:	7.25.4.1

Document for:	Discussion

Introduction

In this contribution, we provide our views on the some remain open issues as below on RRM impacts of UE-to-UE CLI handle described in WF [1] and more detail in last meeting’s moderator summary [2].

- Issue 1-1-5: Requirement applicability

- Issue 1-2-4: Side conditions

Discussion

Sub-topic 1-1: General issues

Issue 1-1-5: Requirement applicability - Proposals - Proposal 1 (vivo, CATT, Nokia, LGE, Samsung, HW, CTC, QC): - The L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency: - when SRS-RSRP measurement resource is fully confined within BW of DL active BWP - when CLI-RSSI measurement resource is configured within active BWP - Proposal 2 (Samsung): - The L1 CLI measurement requirements are applicable provided that L1 CLI measurement resource are measurable - Proposal 3 (ZTE): - The following applicability rules are specified for UE-to-UE CLI measurement: - For SRS-RSRP resource, it should be within the UL active BWP, UE could measure the SRS-RSRP resource within the UL active BWP. - For CLI-RSSI resource - For CLI-RSSI measurement within UL SB, no restriction on resource configuration, but UE only measure the resource within UL active BWP. - For CLI-RSSI measurement within DL SB, no matter the CLI-RSSI resource cross two DL SBs or not, UE measures the CLI-RSSI resource within one or two DL SBs, no restriction on resource configuration. - No gap is needed for UE-to-UE CLI measurement - Proposal 4 (MTK): - L1 CLI measurement resources must be within the DL active BWP of the UE, where L1-SRS-RSRP confined within UL subband, while L1-CLI-RSSI based method #1 and #3 to be confined within DL and UL subband, respectively.

The legacy applicability can be reused. So, we support proposal 1.

Proposal 1: We support the proposal 1. The legacy applicability can be reused.

- The L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency: - when SRS-RSRP measurement resource is fully confined within BW of DL active BWP - when CLI-RSSI measurement resource is configured within active BWP

Sub-topic 1-2: L1-SRS-RSRP

Issue 1-2-4: Side conditions - Proposals for timing offset - Proposal 1 (CATT, Nokia, Apple): - Do not consider the cell phase error - Proposal 2 (LGE): - RAN4 also need to review again whether the inter-cell scenario should be discussed at core part stage - Recommended WF - For timing offset, it is suggested to discuss whether to consider the cell phase error in the side condition during the Performance part, unless impacts to core requirements are identified.

To cover both the intra-cell and inter-cell scenario, RAN4 need to check whether the core requirements are necessary for the intra/inter cell scenario. Because timing offset can be different from intra-cell case and inter-cell case. So, it could be required the different UE operation between them. However current SRS configuration does not include the information about the scenario type whether it is intra cell or inter cell. So, currently the UE cannot work normally according to the intra or inter cell scenario. If other WGs are support the inter-cell scenario, RAN4 also need to check whether the core requirements are necessary for the intra/inter cell scenario. And if necessary RAN4 can request to send the LS to RAN2 to update the current SRS configuration to include the information about the intra cell or inter cell.

Proposal 2: RAN4 also need to check whether the core requirements are necessary for the intra/inter cell scenario. And if necessary RAN4 can request to send the LS to RAN2 to update the current SRS configuration to include the information about the intra cell or inter cell.

Summary

In this contribution, we provide our views on the remain open issues on RRM impacts of UE-to-UE CLI handle described in WF [1] and more detail in last meeting’s moderator summary [2]. Our proposals are as below.

Proposal 1: We support the proposal 1. The legacy applicability can be reused.

- The L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency: - when SRS-RSRP measurement resource is fully confined within BW of DL active BWP - when CLI-RSSI measurement resource is configured within active BWP

Proposal 2: RAN4 also need to check whether the core requirements are necessary for the intra/inter cell scenario. And if necessary RAN4 can request to send the LS to RAN2 to update the current SRS configuration to include the information about the intra cell or inter cell.

Reference

[1] R4-2504904, “WF on RRM requirements for NR_duplex_evo”, Huawei

[2] R4-2503623, “Topic summary for [114bis][213] NR_duplex_evo”, Moderator (Huawei)