3GPP TSG-RAN WG4 Meeting # 115	R4-2506311

# Malta, MT, 19th – 23rd May, 2025

Source:	LG Electronics

Title:	Discussion for LP-WUS/WUR REFSENS

Agenda item:	7.28.3.2

Document for:	Discussion

Introduction

In RAN4#114bis meeting, WF on LP-WUS requirements was approved. Based on the WF, we would like to share our  view for the requirements.

Discussion

Table 1 WF on LP-WUS from RAN4#114bis[1]

Issue 2-1-1: SNR for OOK-based LP-WUS - Proposals - Proposal 1: Conclude SNR for OOK-based LR. (Moderator) FR1 OOK-based receiver; SNR; RM coding; Length of information bits; ADC; CRC; Note; REFSENS. Huawei; -4dB for both; 8/16/32; 5bit?; 4-bit; No; SNR: -2.15 dB (8 bit), -4.63 dB (16 bit), -5.94 dB (32 bit). Sony; -7 to -3dB. Spread trum; -3; SNR last meeting. vivo; -6.5dB; 16/20; 5bit; 4-bit; No; SNR: -6.5dB for 16bit RM coding without Te or 20bit RM coding with Te. CATT; -3.3dB; SNR last meeting; -96.3/-93.3. CMCC; -2.5dB; SNR last meeting. ZTE; -1.5dB; in TS38.212 Clause 5.3.3.3; 5bit; 4-bit; No; SNR -1.5dB (without ML), -7.5dB (with ML); 9dB higher than MR. Nokia; >RAN1 SNR; SNR last meeting. E///; -1.5 dB; SNR last meeting. QC; -4.0 dB; SNR as side condition. Assume RAN1: NF + IM = 17.0 dB, and 14.0 dB?. LGE; -7 to 8 dB; 4bit; SNR last meeting. Agreements: - Companies are encouraged to provide the simulation results with the assumption of the RM coding of 8bit, 16bit and 32bit to align simulations in RAN4. It is targeted to conclude SNR value for envelope detection and OFDM sequence detection next RAN4 meeting.

Based on the agreements made in RAN4#114bis, we have conducted the LLS for OOK based LP-WUS, and the results are Presented in Table 2.

Table 2 BLER 1% for RM coding scenario

| Size payload, bits | Size coded bits | OOK-1 AWGN | OOK-4 AWGN |
| --- | --- | --- | --- |
| Payload: 3 bits with RM, w/o MC | 8 | -12.7 | -7.3 |
|  | 16 | -13.3 | -7.8 |
|  | 32 | -13.1 | -7.8 |
| Payload: 3 bits with RM, with MC | 16 | -12.4 | -7.2 |
|  | 32 | -12.8 | -7.6 |
|  | 64 | -12.9 | -7.6 |
| Payload: 4 bits with RM, w/o MC | 8 | -11.7 | -7.3 |
|  | 16 | -13 | -7.9 |
|  | 32 | -13.2 | -7.8 |
| Payload: 4 bits with RM, with MC | 16 | -11.6 | -6.2 |
|  | 32 | -12.4 | -7.4 |
|  | 64 | -12.9 | -7.7 |
| Payload: 5 bits with RM, w/o MC | 8 | -12.2 | -5.9 |
|  | 16 | -12.9 | -7 |
|  | 32 | -13.3 | -7.8 |
| Payload: 5 bits with RM, with MC | 16 | -12 | -7 |
|  | 32 | -12.7 | -7.4 |
|  | 64 | -13 | -7.7 |

Observation: The worst case SNR for the 5/8 OOK-4 is -5.9 dB, therefore SNR value for OOK-based LP-WUR should be above -5.9 dB, according to the simulation results.

Proposal 1: Proposed SNR value for OOK-based LP-WUR is -4 dB.

- REFSENS calculation

  - NF 15.5 dB

  - IM 2.5 dB

  - SNR -4 dB

  - REFSENS: -174 + 10*LOG(3.96*10^6)+15.5-4+2.5 = -94 dBm

Proposal 2: Proposed REFSENS value for OOK-based LP-WUR is -94 dBm

Conclusion

Observation: The worst case SNR for the 5/8 OOK-4 is -5.9 dB, therefore SNR value for OOK-based LP-WUR should be above -5.9 dB, according to the simulation results.

Proposal 1: Proposed SNR value for OOK-based LP-WUR is -4 dB.

- REFSENS calculation

  - NF 15.5 dB

  - IM 2.5 dB

  - SNR -4 dB

  - REFSENS: -174 + 10*LOG(3.96*10^6)+15.5-4+2.5 = -94 dBm

Proposal 2: Proposed REFSENS value for OOK-based LP-WUR is -94 dBm

- References

# [1] R4-2505234, WF for [114bis][131] NR_LPWUS_UERF  (Vivo)

- Appendix

Simulation results for OOK-based LP-WUS