# 3GPP TSG-RAN WG4 Meeting #115	R4-2506344

# Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.27.4

Source:	China Telecom

Title:	Discussion on RRM performance requirements of NES for NR

Document for:	Discussion

- Introduction

In this paper, we present our initial discussion on RRM performance requirements of NES for NR.

- Discussion

For performance requirements of NES part 1, based on the core part discussion in RAN4 previous meetings, the requirements on OD-SSB based deactivated SCell measurement with FMW, OD-SSB based SCell activation, and OD-SSB based L1 measurement are to be defined, therefore, related test cases need to be designed. Besides, two cases including Case 1 and Case 2 need to be considered for test case design.

For OD-SSB based deactivated SCell measurement with FMW, OD-SSB based NR intra-frequency measurements requirements are to be defined in the core part, Case 1 and Case 2 are also considered for OD-SSB based NR intra-frequency measurements.

For OD-SSB based SCell activation, scenarios including OD-SSB based normal SCell Activation, OD-SSB based direct SCell Activation at SCell addition, OD-SSB based PUCCH SCell Activation, Fast SCell Activation with OD-SSB, and OD-SSB based SCell Activation with the L3 reporting during activation are considered in core part, therefore, it’s proposed to design test cases for these scenarios. Considering the workload for these scenarios, the test cases for Case 1 and Case 2 for each scenario can be designed both two or one of them, which can be further discussed.

For OD-SSB based L1 measurement, it’s proposed to design test case for L1-RSRP measurements based on OD-SSB.

Proposal 1: For OD-SSB based deactivated SCell measurement with FMW, it’s proposed to define test cases as following:

- OD-SSB based NR intra-frequency measurements for Case 1;

- OD-SSB based NR intra-frequency measurements for Case 2.

Proposal 2: For OD-SSB based SCell activation, it’s proposed to define test cases as following:

- OD-SSB based normal SCell Activation;

- OD-SSB based direct SCell Activation at SCell addition;

- OD-SSB based PUCCH SCell Activation;

- Fast SCell Activation with OD-SSB;

- OD-SSB based SCell Activation with the L3 reporting during activation.

Proposal 3: For OD-SSB based SCell activation, considering the workload, whether to design one test case or two test cases for Case 1 and Case 2 for each type of SCell activation can be further discussed.

Proposal 4: For OD-SSB based L1 measurement, it’s proposed to design test case for L1-RSRP measurements based on OD-SSB.

For performance requirements of NES part 2, SSB adaptation and On-demand SIB1 are discussed in the core part, related test cases need to be designed.

For SSB adaptation, test cases for L1 measurement with SSB adaptation and L3 measurement with SSB adaptation can be designed, moreover, test case for SCell activation with SSB adaptation need also to be designed.

For On-demand SIB1, it’s proposed to design test case for On-demand SIB1 with UL WUS mechanism.

Proposal 5: For SSB adaptation, it’s proposed to define test cases as following:

- L1 measurement with SSB adaptation;

- L3 measurement with SSB adaptation;

- SCell activation with SSB adaptation.

Proposal 6: For On-demand SIB1, it’s proposed to design test case for On-demand SIB1 with UL WUS mechanism.

- Conclusion

The contribution presents our views on RRM performance requirements of NES for NR, with the following proposals:

Proposal 1: For OD-SSB based deactivated SCell measurement with FMW, it’s proposed to define test cases as following:

- OD-SSB based NR intra-frequency measurements for Case 1;

- OD-SSB based NR intra-frequency measurements for Case 2.

Proposal 2: For OD-SSB based SCell activation, it’s proposed to define test cases as following:

- OD-SSB based normal SCell Activation;

- OD-SSB based direct SCell Activation at SCell addition;

- OD-SSB based PUCCH SCell Activation;

- Fast SCell Activation with OD-SSB;

- OD-SSB based SCell Activation with the L3 reporting during activation.

Proposal 3: For OD-SSB based SCell activation, considering the workload, whether to design one test case or two test cases for Case 1 and Case 2 for each type of SCell activation can be further discussed.

Proposal 4: For OD-SSB based L1 measurement, it’s proposed to design test case for L1-RSRP measurements based on OD-SSB.

Proposal 5: For SSB adaptation, it’s proposed to define test cases as following:

- L1 measurement with SSB adaptation;

- L3 measurement with SSB adaptation;

- SCell activation with SSB adaptation.

Proposal 6: For On-demand SIB1, it’s proposed to design test case for On-demand SIB1 with UL WUS mechanism.