# 3GPP TSG-RAN WG4 Meeting #115	R4-2506345

# Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.28.5.2

Source:	China Telecom

Title:	Discussion on RRM core requirements for LP-WUS/WUR

Document for:	Discussion

- Introduction

In RAN4 #114bis meeting, RRM core requirements for LP-WUS/WUR was discussed and the WF was approved [1].

In this paper, we present our views on RRM core requirements for LP-WUS/WUR issues captured in the WF [1] and summary [2].

- Discussion

General aspects

Status in the WF [1] and summary [2]:

Issue 1-1-9: LP-WUR status at legacy case (not at LP-WUS monitoring case/fully offloading(case 1) case/RRM relaxation (case 3) case) Recommendations: Further discuss whether LR evaluation requirements apply when LR is “ON” at legacy state. - Proposals - P1: No LP-WUR measurement and evaluation requirement apply when LR is “on” at the legacy state; (vivo ZTE Huawei Ericsson Xiaomi) Background: - Summary of entry/exit conditions based on existing RAN2’s agreements Case; Entry conditions; Exit conditions. LP-WUS monitoring; MR threshold and optional LR threshold; LR threshold. RRM measurement fully offloading (Case 1); MR threshold and optional LR threshold; LR threshold. RRM measurement relaxation (Case 3); For serving cell RRM relaxation, at least MR threshold, FFS if LR measurement is needed. FFS if the entry condition for serving cell RRM measurement relaxation is the same as neighbour cell RRM measurement relaxation; FFS. Agreement at RAN4 114bis: - At legacy state, when and how to turn on LR for serving cell measurement is up to UE implementation. Recommendations: At legacy state, when LP-WUR is “ON” for evaluating LR threshold, LP-WUR measurement and evaluation requirement will not apply.

For LP-WUR status at legacy case, it was agreed that at legacy state, when and how to turn on LR for serving cell measurement is up to UE implementation, and further discuss whether LR evaluation requirements apply when LR is “ON” at legacy state. In our views, at legacy state, MR is working without offloading or relaxation, the legacy MR evaluation requirements applied to MR is efficient and there is no need to apply LR evaluation requirements, thus we prefer that no LP-WUR measurement and evaluation requirement apply when LR is “on” at the legacy state.

Proposal 1: No LP-WUR measurement and evaluation requirement apply when LR is “on” at the legacy state.

Status in the WF [1]:

Issue 1-1-10: Considerations on higher priority frequency layer Agreement: - Support relaxed higher priority frequency layer search at case 1 Tentative agreement: - For case 1, when the serving cell is above threshold for inter-frequency or inter-RAT neighbour cell measurement, existing relaxed requirements K2*Thigher_priority_search are re-used for higher priority frequency search. - Note: K2 is 60 Recommendations: Further discussion on detail on requirements and related tentative agreement.

For higher priority requirements, we support to use existing relaxed requirements K2*Thigher_priority_search for higher priority frequency search, where K2 is 60 minutes. Besides, inter-RAT is also ok for us since there also exists inter- RAT deployment in practical network.

Proposal 2: Support the tentative agreement that for case 1, when the serving cell is above threshold for inter-frequency or inter-RAT neighbour cell measurement, existing relaxed requirements K2*Thigher_priority_search are re-used for higher priority frequency search, where K2 is 60.

Detail LP-WUR requirements at RRC_IDLE/INACTIVE state

Status in the WF [1] and summary [2]:

Issue 1-2-2: Periodicity for LP-WUR RRM requirements Agreement: Regardless of whether LP-SS is configured or not, periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity. Issue 1-2-2-1: Lower bound on LP-SS measurement periodicity Recommendations: Continue discussion - Proposals - P1: In case the periodicity of LP-SS can be configured to small value like 80ms or 160ms (xiaomi) - Option 1: define the lower limit for measurement delay requirements; (Huawei) - Option 2: define the measurement delay requirements based on max(LP-SS periodicity, LO periodicity). - P2: No need for lower bound. (Nokia) - P3: Wait for more RAN1 progress before determining whether to have lower bound. (vivo oppo) Background: RAN4 112bis meeting agreement: Measurement delay requirements for LP-WUR based on LP-SS in IDLE/Inactive mode shall be defined based on LP-SS periodicity. Issue 1-2-2-2: Upper bound on SSB-based LP-WUR measurement periodicity Recommendations: Continue discussion - Proposals - P1: No need to define upper bound for SSB-based LP-WUR measurement periodicity. (xiaomi oppo CATT CT Nokia) - P2: Prefer the periodicity for measurement and evaluation delay requirements for LP-WUR based on SSB in IDLE/Inactive mode is 1.28s if the LO periodicity is 2.56s. (vivo ZTE) - P3: Postpone the discussion for the case where LP-SS periodicity > LO periodicity until RAN1 has more agreements for LP-SS periodicity (Huawei) Background: RAN4 113 agreement: If LP-SS is not configured, periodicity for SSB based LP-WUR measurement delay requirements - based on LO periodicity - FFS whether to define an upper bound - Note: DRX cycle and LO periodicity are the same based on RAN1 agreement. When LP-SS is configured, apply the same agreement as for when LP-SS is not configured - The above agreement applies if LO periodicity is equal to or larger than LP-SS periodicity, FFS if LO periodicity is smaller than LP-SS periodicity.

For upper bound on SSB-based LP-WUR measurement periodicity, since DRX cycle and LO periodicity are the same based on RAN1 agreement, and periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity, in our views, it’s preferred not to define upper bound for SSB-based LP-WUR measurement periodicity.

Proposal 3: Do not define upper bound for SSB-based LP-WUR measurement periodicity.

Status in the WF [1] and summary [2]:

Issue 1-2-4: On LR measurement/evaluation requirements Issue 1-2-4-1: On LR measurement filtering requirement Agreement: - The UE shall filter the [SS-RSRP and SS-RSRQ] measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by [LO periodicity /2] for SSB based LR. Recommendations: Continue discuss on whether to define and how to define LP-SS based LR filtering requirements - Proposals - P1-1: The UE shall filter the LP-RSRP and LP-RSRQ measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by LP-SS periodicity/2 for LP-SS based LR. (vivo CT Ericsson) - P1-2: For LP-SS-RSRP and LP-SS-RSRQ measurement, no need to define filtering requirement (CMCC) - P2:The UE shall filter the [SS-RSRP and SS-RSRQ] measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by [LO periodicity /2] for SSB based LR. (vivo CMCC CT Ericsson Nokia) - P3: UE shall filter the LR measurements using at least 2 measurements with enough separation. (Huawei) Background: Recommendations: Check whether P2 is agreeable. Further discuss P1-1 and P1-2.

On LR measurement filtering requirement, whether to define and how to define LP-SS based LR filtering requirements are further discussed, in our views, when the LP-SS periodicity is 320ms, it’s preferred and reasonable to follow existing DRX case that each measurement should be based on filtering of at least 2 samples.

Proposal 4: For LP-SS based LR filtering requirements, it’s preferred to define same as existing DRX requirements, i.e., the UE shall filter the LP-RSRP and LP-RSRQ measurements of the serving cell using at least 2 measurements.

- Conclusion

The contribution presents our views on Rel-19 RRM core requirements for LP-WUS/WUR, with the following proposals:

Proposal 1: No LP-WUR measurement and evaluation requirement apply when LR is “on” at the legacy state.

Proposal 2: Support the tentative agreement that for case 1, when the serving cell is above threshold for inter-frequency or inter-RAT neighbour cell measurement, existing relaxed requirements K2*Thigher_priority_search are re-used for higher priority frequency search, where K2 is 60.

Proposal 3: Do not define upper bound for SSB-based LP-WUR measurement periodicity.

Proposal 4: For LP-SS based LR filtering requirements, it’s preferred to define same as existing DRX requirements, i.e., the UE shall filter the LP-RSRP and LP-RSRQ measurements of the serving cell using at least 2 measurements.

- References

- R4-2504908, WF on RRM requirements for NR_LPWUS, vivo, RAN WG4 Meeting #114, 7 – 11 April, 2025.

- R4-2503627, Topic summary for [114bis][217] NR_LPWUS, vivo, RAN WG4 Meeting #114, 7 – 11 April, 2025.