3GPP TSG-RAN4 Meeting # 115	R4-2506378

Malta, May.19th – 23th, 2025

# Title: 	TP to TR38.755 on UE RF requirements impact for fragmented carriers

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.5.4

# Document for:	Approval

- Introduction

This contribution provides a text proposal to TR38.755[1] on UE RF requirement impact for fragmented carriers based on our previous contributions.

- Reference

[1] TR38.755, Study on NR FR1 DL fragmented carriers, v0.2

- Text Proposal

----- Start of TP -----

## 7.x	RF requirements evaluation from company C

For FDD band intra-band non-contiguous CA, the UL Tx noise caused by Tx chain/PA leakage and Tx spectrum spreading into DL Rx path shall be taken into account since the duplexer gap for some FDD bands are small, and such small duplexer gap cannot provide enough attenuation for the filters design. When the separated Rx RF chain is replaced with sharing Rx RF chain, the shared filter cannot filter out each carrier anymore. the interference appeared in the gap is located in the pass band of the filter, and the filter cannot provide the attenuation in the gap between the two DL carriers. Thus the ΔRIBNC requirements for FDD intra-band NC CA would become larger which are needed to be re-evaluated for both PCC and SCC.

Unlike FDD, there is no duplex gap for TDD band, and there are no ΔRIBNC requirements defined for TDD intra-band non-contiguous CA. Thus no matter separated Rx RF chain or sharing Rx RF chain is used, there are need to no re-evaluated or define ΔRIBNC requirements for TDD intra-band non-contiguous CA.

Below we provide the link budget calculation for self-interference/ACS/IBB based on the template for CA_n3(2A), CA_n7(2A) and CA_n25(2A).

Analysis on CA_n3(2A):

The link budget of Self-interference for 5+5MHz on band n3 can be found in table 7.x.1-1.

Table 7.x.1-1: Link budget of Self-interference for 5+5 MHz on band n3

|  | Tx leakage | Wanted Carrier (PCC) | Wanted Carrier (SCC) | Noise floor | Tx leakage mapping to -12dBFS (PAPR is considered) |
| --- | --- | --- | --- | --- | --- |
| PA out | 27.0 | - | - | - | - |
| Duplexer isolation | 50.0 | - | - | - | - |
| Rx Antenna input | - | -97.0 | -92.3 | - | - |
| FE loss | - | 4.0 | 4.0 | - | - |
| Duplexer output | -23.0 | -101.0 | -96.3 | - | - |
| LPF attenuation | 12.1 | 1.0 | 1.0 | - | - |
| LPF output (dBm) | -35.1 | -102.0 | -97.3 | - | - |
| Noise floor for basic REFSENS | - | - | - | -101.0 | - |
| NF increase due to AGC adjustment | - | - | - | 11.0 | - |
| Noise floor | - | - | - | -90.0 | - |
| In-band noise (Derived based on MSD and SNR = -1dB) | - | - | -96.3 | - | - |
| ADC output (dBm) | - | - | - | - | -91.1 |
| SNR after ADC (dB) | - | - | - | - | -14.5(PCC) |
|  |  |  |  |  | -10.3(SCC) |

The Link budget of ACS case 1 and case 2 with and without Rx image on band n3 can be found in table 7.x.1-2, respectively. 25dB rejection Rx image is used for the calculation.

ACS: with and without Rx image

Table 7.x.1-2. Link budget of ACS case with and without Rx image on band n3

| LPF attenuation (dB) | 12.1 |  |  |  |
| --- | --- | --- | --- | --- |
| NF increase due to AGC adjustment(dB) | 11 |  |  |  |
|  | Without Rx image |  | With Rx image |  |
|  | Case 1 | Case 2 | Case 1 | Case 2 |
| SNR after ADC (dB) for PCC | -3.8 | -1 | -3.8 | -1 |
| SNR after ADC (dB) for SCC | -2.1 | -1 | -5 | -7.6 |

The Link budget of IBB case 1 and case 2 with and without Rx image on band n3 can be found in table 7.x.1-3. 25dB rejection Rx image is used for the calculation.

IBB: with and without Rx image

Table 7.x.1-3. Link budget of IBB case with and without Rx image on band n3

| LPF attenuation (dB) | 12.1 |  |  |  |
| --- | --- | --- | --- | --- |
| NF increase due to AGC adjustment(dB) | 11 |  |  |  |
|  | Without Rx image |  | With Rx image (25dB rejection) |  |
|  | Case 1 | Case 2 | Case 1 | Case 2 |
| SNR after ADC (dB) for PCC | -9.2 | -9.2 | -9.2 | -9.2 |
| SNR after ADC (dB) for SCC | -5.6 | -5.6 | -8.5 | -17.6 |

Analysis on CA_n7(2A):

The link budget of Self-interference for 10+5MHz on band n7 can be found in table 7.x.1-4, respectively.

Table 7.x.1-4: Link budget of Self-interference for 10+5 MHz on band n7

|  | Tx leakage | Wanted Carrier (PCC) | Wanted Carrier (SCC) | Noise floor (PCC) | Noise floor (SCC) | Tx leakage mapping to -12dBFS (PAPR is considered) |
| --- | --- | --- | --- | --- | --- | --- |
| PA out | 27.0 | - | - | - |  | - |
| Duplexer isolation | 50.0 | - | - | - |  | - |
| Rx Antenna input | - | -94.8 | -98.0 | - |  | - |
| FE loss | - | 4.0 | 4.0 | - |  | - |
| Duplexer output | -23.0 | -98.8 | -102.0 | - |  | - |
| LPF attenuation | 17.3 | 1.0 | 1.0 | - |  | - |
| LPF output (dBm) | -40.3 | -99.8 | -103.0 | - |  | - |
| Noise floor for basic REFSENS | - | - | - | -98.8 | -102.0 | - |
| NF increase due to AGC adjustment | - | - | - | 6.0 | 6.0 | - |
| Noise floor | - | - | - | -92.8 | -96.0 | - |
| In-band noise (Derived based on MSD and SNR = -1dB) | - | - | -102.0 | - |  | - |
| ADC output (dBm) | - | - | - | - |  | -96.3 |
| SNR after ADC (dB) | - | - | - | - |  | -8.6(PCC) |
|  |  |  |  |  |  | -10.4(SCC) |

The Link budget of ACS case 1 and case 2 with and without Rx image on band n7 can be found in table 7.x.1-5. 25dB rejection Rx image is used for the calculation.

ACS: with and without Rx image

Table 7.x.1-5. Link budget of ACS case with and without Rx image on band n7

| LPF attenuation (dB) | 17.3 |  |  |  |
| --- | --- | --- | --- | --- |
| NF increase due to AGC adjustment(dB) | 6 |  |  |  |
|  | Without Rx image |  | With Rx image |  |
|  | Case 1 | Case 2 | Case 1 | Case 2 |
| SNR after ADC (dB) for PCC | -1.9 | -1 | -1.9 | -1 |
| SNR after ADC (dB) for SCC | -2.2 | -1 | -10.4 | -7.6 |

The Link budget of IBB case 1 and case 2 with and without Rx image on band n7 can be found in table 7.x.1-6. 25dB rejection Rx image is used for the calculation.

IBB: with and without Rx image

Table 7.x.1-6. Link budget of IBB case with and without Rx image on band n7

| LPF attenuation (dB) | 17.3 |  |  |  |
| --- | --- | --- | --- | --- |
| NF increase due to AGC adjustment(dB) | 6 |  |  |  |
|  | Without Rx image |  | With Rx image (25dB rejection) |  |
|  | Case 1 | Case 2 | Case 1 | Case 2 |
| SNR after ADC (dB) for PCC | -4.9 | -4.9 | -4.9 | -4.9 |
| SNR after ADC (dB) for SCC | -5.7 | -5.7 | -12.1 | -23.1 |

Analysis on CA_n25(2A):

The link budget of Self-interference for 5+5MHz on band n25 can be found in table 7.x.1-7 and 7.x.1-10, respectively.

Table 7.x.1-7: Link budget of Self-interference for 5+5 MHz on band n25

|  | Tx leakage | Wanted Carrier (PCC) | Wanted Carrier (SCC) | Noise floor | Tx leakage mapping to -12dBFS (PAPR is considered) |
| --- | --- | --- | --- | --- | --- |
| PA out | 27.0 | - | - | - | - |
| Duplexer isolation | 50.0 | - | - | - | - |
| Rx Antenna input | - | -96.5 | -91.5 | - | - |
| FE loss | - | 4.0 | 4.0 | - | - |
| Duplexer output | -23.0 | -100.5 | -95.5 | - | - |
| LPF attenuation | 12.6 | 1.0 | 1.0 | - | - |
| LPF output (dBm) | -35.6 | -101.5 | -96.5 | - | - |
| Noise floor for basic REFSENS | - | - | - | -100.5 | - |
| NF increase due to AGC adjustment | - | - | - | 10.5 | - |
| Noise floor | - | - | - | -90.0 | - |
| In-band noise (Derived based on MSD and SNR = -1dB) | - | - | -95.5 | - | - |
| ADC output (dBm) | - | - | - | - | -91.6 |
| SNR after ADC (dB) | - | - | - | - | -13.8(PCC) |
|  |  |  |  |  | -9.5(SCC) |

The Link budget of ACS case 1 and case 2 with and without Rx image on band n25 can be found in table 7.x.1-8. 25dB rejection Rx image is used for the calculation.

ACS: with and without Rx image

Table 7.x.1-8. Link budget of ACS case with and without Rx image on band n25

| LPF attenuation (dB) | 12.6 |  |  |  |
| --- | --- | --- | --- | --- |
| NF increase due to AGC adjustment(dB) | 10.5 |  |  |  |
|  | Without Rx image |  | With Rx image |  |
|  | Case 1 | Case 2 | Case 1 | Case 2 |
| SNR after ADC (dB) for PCC | -3.4 | -1 | -3.4 | -1 |
| SNR after ADC (dB) for SCC | -1.9 | -1 | -4.7 | -7.6 |

The Link budget of IBB case 1 and case 2 with and without Rx image on band n25 can be found in table 7.x.1-9. 25dB rejection Rx image is used for the calculation.

IBB: with and without Rx image

Table 7.x.1-9. Link budget of IBB case with and without Rx image on band n25

| LPF attenuation (dB) | 12.6 |  |  |  |
| --- | --- | --- | --- | --- |
| NF increase due to AGC adjustment(dB) | 10.5 |  |  |  |
|  | Without Rx image |  | With Rx image (25dB rejection) |  |
|  | Case 1 | Case 2 | Case 1 | Case 2 |
| SNR after ADC (dB) for PCC | -8.6 | -8.6 | -8.6 | -8.6 |
| SNR after ADC (dB) for SCC | -5 | -5 | -7.8 | -16.8 |

----- End of TP -----