3GPP TSG-RAN4 Meeting # 115	R4-2506379

Malta, May.19th – 23th, 2025

# Title: 	On release independent for 6Rx RF requirement

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.1.2.3.1

# Document for:	Approval

# 1 Introduction

In last meeting, some agreements were achieved for 6Rx in the WF [1], where for release independent, there was no consensus due to lack of the time.

Issue 1-1-1: Release independence of 6Rx RF requirements Agreement: FFS whether to define 6Rx RF requirements in a release independent manner

In this contribution, we continue discussing the release independent on top of [2].

# 2	Discussion

When RAN4 discussed the 4Rx and 8Rx feature in the previous releases, it was agreed that 4Rx RF requirements are release independent from R15 while 8Rx RF requirements are release independent from R17. The release independent information are included in the TS38.307. It should be noted that both FWA and handheld form factor UE are applicable for 4Rx, and only FWA/CPE/Vehicle/Industrial devices form factor are applicable for 8Rx.

For 6Rx, it includes handheld UE and FWA UE, and the ΔRIB,6R values for handheld UE and FWA are not the same. In addition to that, the supportive of 6 layer for handheld UE and FWA undergo a very long controversial discussion in demodulation. Consequently, different performance requirements are defined for 6Rx handheld UE and 6Rx FWA, which are:

-	6-layer performance requirements will be only defined for FWA.

-	4-layer performance requirements will be defined and will apply to FWA and handheld UE devices.

It should noted that whether 4-layer or 6-layer is not considered when RAN4 discuss the RF requirements.

In our understanding, the purpose for the release independent for a feature is to allow this feature can be implemented in an earlier release (even in a earlier frozen release) even though this feature is introduced in later release. The definition for the release independent can be found in TS38.306, which is shown as below:

release independent: applicable to some frozen releases, starting from a certain release Rel-M NOTE 1: Normally, a feature is introduced only in the latest open release Rel-N and future releases are based on the previous one so that future releases inherit the requirements of this feature. Introducing a feature "in a release independent way from Rel-M onwards" (M<N) means it was decided by TSG RAN that this feature would be also beneficial in previous, already frozen releases starting with Rel-M until Rel-(N-1). In order to avoid touching TS 38.101 [2-5] or TS 38.133 [6] of these frozen releases, the corresponding requirements are captured in TS 38.307 via pointers to [2-5] or [6] of the release in which the feature was introduced. NOTE 2: Release independent does not mean applicable to all releases.

For 6Rx, similar as 8Rx, where RF requirements for 8Rx UE is release independent from Rel-17, we think it will be no harmful that this 6Rx feature for both handheld UE and FWA can be release independent from Rel-17, and only 4-layer is applicable for handheld UE performance requirements.

Regarding the SRS antenna switching, currently t1r6, t2r6 are already included in srs-AntennaSwitchingBeyond4RX-r17 capability, and capability for t3r6 is under discussing. Due to the srs-AntennaSwitchingBeyond4RX-r17 capability is an optional capability, so we think it should be decoupled with 6Rx feature although ΔTSRSRx requirements for t3r6 are defined. Of course, the release independent for t3r6 can not be earlier than the release which 6Rx feature can be release independent from.

Proposal: Release independent for 6Rx RF requirement for both handheld UE and FWA can be from Rel-17.

# 3 Conclusion

In this contribution, we continue discussing on the release independent for 6Rx RF requirements.

Proposal: Release independent for 6Rx RF requirement for both handheld UE and FWA can be from Rel-17.

# 4	Reference

[1] R4-2505101, WF on 6Rx RF requirements and SRS IL imbalance, Intel

[2] R4-2503965 On release independent for 6Rx RF requirement, ZTE Corporation, Sanechips, Ericsson