3GPP TSG-RAN4 Meeting # 115	R4-2506391

Malta, May.19th – 23th, 2025

Title:	Reply LS on UE capability for FDD-FDD inter-band CA simultaneous RxTx

Response to:	R2-2501572

Release:	Rel-19

Work Item:	NR_CADC_SUL_R19-Core

Source:	TSG RAN WG4

To:	TSG RAN WG2

Cc:	TSG RAN WG1

Contact Person:

Name:  		Wubin Zhou

E-mail Address:  	zhou.wubin@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

- Overall Description:

RAN4 would like to thank RAN2 for the LS on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx. RAN4 would like to respectfully provide the following responses for RAN2 consideration. According to the discussion during RAN4#114-bis, the responses are:

- Whether it’s possible for a UE to support simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 without any scheduling restriction, or whether there may be a case where a UE can support simultaneous Rx/Tx in some part(s) of the overlapping FDD bands? If possible, from which release version?

- Answer:

- In Rel-19, for 2UL/2DL CA_n5-n8, full band frequency range of each band is supported, and full band n5 and n8 RF filters implementation are assumed in RAN4 discussion. RAN4 has already agreed that uplink is only in n5 for one uplink CA_n5-n8 and 2UL/2DL CA_n5-n8 is supported using non-concurrent n5 DL and n8 UL.

- In RAN4 previous discussion, simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 without scheduling restriction can only be applied for partial frequency range of each band with dedicated RF filters implementation. However, dedicated RF filters implementation with partial frequency range is precluded in Rel-19.

- Therefore, for full band n5 and n8 RF filters implementation with full band frequency range, it is impossible to implement simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 without any scheduling restriction, and also it is impossible for a UE to support simultaneous Rx/Tx in some part(s) of the overlapping FDD bands.

- If a UE may be able to support simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 (or some other inter-band FDD-FDD CA combinations) in some form, whether it requires additional UE capabilities for the support/requirement of other features, e.g., full-duplex, switching time, or some frequency gap/guard band?

- Answer:

For inter-band FDD-FDD band combination, RAN4 have already agreed that:

- For FDD-FDD band combination, there is no capability signalling designed for simultaneous Rx-Tx. There is no scheduling restriction for FDD-FDD band combination from network side. Both the network and UE assume simultaneous Rx-Tx is mandatorily supported without signalling, by default.

- Currently CA_n5-n8 is the only one band combination where UL and DL from different FDD bands partially overlap. UE can not support simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 with full band n5 and n8 RF filters implementation to support full band frequency range of each band.

- In addition, RAN4 has already agreed to introduce a new NOTE (i.e. NOTE 15 as below) to pre-define 2UL/2DL CA_n5-n8 in the specification:

NOTE 15: Uplink is only in n5 for one uplink CA_n5-n8. 2UL/2DL CA_n5A-8A is supported using non-concurrent n5 DL and n8 UL.

# 2. Actions:

To RAN2:

ACTION: RAN4 respectfully ask RAN2 to take above RAN4 responses into consideration.

# 3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN WG4 Meeting #115            19 - 23 May 2025             					Malta, MT

TSG-RAN WG4 Meeting #116             25- 29 Aug 2025             					India, IN