3GPP TSG-RAN4 Meeting # 115	R4-2506395

Malta, May.19th – 23th, 2025

# Source:	ZTE Corporation, Sanechips

# Title:	TP for TR38.719-02-01_Correct the harmonic MSD test points for CA_n3-n104 and n20-n41

# Agenda Item:	6.3.3

# Document for:	Approval

- Introduction

The Lcrb value for harmonic/harmonic mixing MSD test point have already been implemented in the specification by following the rules (15kHz/30kHz) below:

-          For the UL1/DLx test point, Lcrb=25RB / 12RB,

-          For the UL2/DLx test point, Lcrb=12RB / 6RB,

-          For the UL3/DLx test point, Lcrb=8RB / 4RB,

-          For the UL4/DLx test point, Lcrb=6RB / 3RB,

-          For the UL5/DLx test point, Lcrb=5RB / 2RB.

However, in current TS38.101-1 specification, for CA_n3-n104 and n20-n41 band combination, the MSD test point (i.e. UL RB allocation) are not compliance to the above rules.

Table 7.3A.4-1: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC3 aggressor NR UL band for NR DL CA FR1 (Extracted from TS38.101-1)

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n1 | n77 | 5 | 15 | 12 | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n1 | n77 | 5 | 15 | 12 | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n2 | n48 | 5 | 15 | 25 | 5 | 8.1 | NOTE 6 | UL2/DL1 near-miss |
| n2 | n77 | 5 | 15 | 12 | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n2 | n77 | 5 | 15 | 12 | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n2 | n78 | 5 | 15 | 12 | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n2 | n78 | 5 | 15 | 12 | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n3 | n77 | 5 | 15 | 12 | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n3 | n77 | 5 | 15 | 12 | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n3 | n78 | 5 | 15 | 12 | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n3 | n78 | 5 | 15 | 12 | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n3 | n104 | 5 | 15 | 25 (RBstart=0) | 20 | 11.7 | NOTE 4 | UL4/DL1 direct-hit |
| n3 | n104 | 50 | 15 | 100 (RBstart=0) | 100 | 6.1 | NOTE 4 | UL4/DL1 direct-hit |
| ... |  |  |  |  |  |  |  |  |
| n20 | n41 | 5 | 15 | 16 (RBstart=0) | 10 | 10.3 | NOTE 3 | UL3/DL1 |
| ... |  |  |  |  |  |  |  |  |

Therefore, this contribution provides a text proposal to correct the harmonic MSD test points for CA_n3-n104 and n20-n41 in TR38.719-02-01[1].

- Reference

- [1] TR38.719-02-01,Rel-19 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2),v0.5.0

- Text Proposal

----- Start of TP -----

  - 5.1	CA_n3-n104

<unchanged text are omitted>

#### 5.1.1.5	REFSENS requirements

Considering the n3 is mid band and n104 is ultra-band, here we use the full separate antenna RF architecture assumption for the MSD derivation. The MSD for H4 caused by n3 UL falls into n104 DL are proposed in the table 5.1.1.5-1:

Table 5.1.1.5-1: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC3 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n3 | n104 | 5 | 15 | 6 | 20 | 11.7 | NOTE 4 | UL4/DL1 direct-hit |
| n3 | n104 | 5 | 15 | 6 | 100 | 6.1 | NOTE 4 | UL4/DL1 direct-hit |
| NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |

<Next change>

## 5.14	CA_n20-n41

<unchanged text are omitted>

#### 5.14.1.5	REFSENS requirements

Based on the co-existence studies n20 third harmonic uplink falls into the n41 downlink. The value is based on DC_20A_n41A, which has similar harmonic issue.

Table 5.14.1.5-1: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC3 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n20 | n41 | 5 | 15 | 8 | 10 | 10.3 | NOTE 3 | UL3/DL1 |
| NOTE 3: The requirements should be verified for UL NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with the carrier |  |  |  |  |  |  |  |  |

----- End of TP -----