3GPP TSG-RAN WG4 Meeting #115      		R4-2506415

Malta, MT, 19th - 23rd May, 2025

Agenda Item:	7.12.7

Source: 	CMCC

Title: 			 Discussion on demodulation and CSI requirements for Rel-19 ATG

Document for:	Discussion

- Introduction

In last meeting, RAN4 discussed partial test scope and test setups about DL CA, DL MIMO and UL MIMO feature. The WF has been approved in [1]. In this contribution, we further give our views on test scope and simulation assumption.

- ATG UE DL CA demodulation requirement

Issue 1-1-2: CA Duplex mode

Agreement:

- TDD 30 kHz + TDD 30 kHz

- FDD 15 kHz + TDD 30 kHz

FFS:

- FDD 15 kHz + FDD 15 kHz

Since anyway the single carrier requirement of FDD 15kHz will be specified, no additional specification work is needed for FDD 15 kHz + FDD 15 kHz CA configuration. Therefore, we prefer to also included it.

Proposal 1: Cover the CA configuration of FDD 15kHz + FDD 15kHz.

Issue 1-1-3-1: TDD pattern

Agreement:

- Use legacy TDD pattern 7DS2U as the staring point

FFS:

- new TDD pattern 30D4S6U

Since there are still some potential maintenance issues about the new TDD pattern we introduced in Rel-18, We propose to use legacy TDD pattern.

Proposal 2: Use the legacy TDD pattern as the starting point.

Issue 1-1-3-2: Rank

- Proposal 1: Rank 2

- Proposal 2: Rank 1

Since MIMO can be supported in Rel-19, and only conducted test will be applied for ATG UE, which means the UE antenna type is transparent in the test, we think the Rank 2 can be used for defining the CA requirement.

Proposal 3: For CA requirement, use Rank 2 as the starting point.

Issue 1-1-3-7: Bandwidth

Agreement:

- all bandwidth configuration for FDD 15kHz and TDD 30kHz

- Single carrier requirements for each of the considered bandwidth sizes in the bandwidth combination sets shall then be specified.

- FFS:

- RAN4 to clarify the bandwidth combination sets for the FR1 intra-band contiguous CA in CA_n79C and for FR1+FR1 inter-band CA in CA_n3A-n39A.

For n3 15kHz SCS, all FDD 15kHz bandwidths are supported.

For n39 and n79 30kHz SCS, only 5MHz can not be covered due to no request from operators. However, it is still applicable, because 30kHz SCS support the 5MHz BW configuration.

Therefore, for better forwards compatible, we propose to specify single carrier requirements for all supported bandwidth sizes for FDD 15kHz and TDD 30kHz.

Proposal 4: Specify single carrier requirements for all supported bandwidth sizes for FDD 15kHz and TDD 30kHz.

- Issue 1-1-4: CA configuration selection rules

- Proposal 1:

  - Define the CA configuration selection rules as

| CA capability | Step 1 | Step 2 | Step 3 | Step 4 |
| --- | --- | --- | --- | --- |
| CA_C | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | N/A | N/A |
| CA_AX | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | Select the CA configurations with the largest number of bands and with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 3. |

- Issue 1-1-5: Test applicability rules

- Proposal 1:

  - Define the test rules as:

| Tests | CA capability where the tests apply | CA configuration from the selected CA capability where the tests apply | CA Bandwidth combination to be tested in priority order | PCell CC configuration |
| --- | --- | --- | --- | --- |
| FDD 15 kHz + FDD 15 kHz | CA_C, CA_AX | Table TBD | Largest aggregated CA bandwidth combination | Any of CCs |
| TDD 30 kHz + TDD 30 kHz | CA_C, CA_AX | Table TBD | Largest aggregated CA bandwidth combination | Any of CCs |
| FDD 15 kHz + TDD 30 kHz | CA_AX | Table TBD | Largest aggregated CA bandwidth combination | FDD CC if supported, otherwise TDD CC |

In high level, we think the legacy Rel-16 CA configuration selection rule and test applicability rule can be reused for Rel-19 ATG CA, but some refinement need to be done with the consideration of ATG specific feature, including:

- Only intra-band contiguous CA and inter-band CA are included in WID

- For FDD 15kHz +TDD 30kHz case, (e.g., Example band combination: CA_n3A-n39A), FDD CC is configured as PCC

- Only FDD 15kHz and TDD 30kHz are supported for Rel-18 single carrier requirement

After the refinement, we propose following CA configuration selection rule and test applicability rule for Rel-19 ATG CA:

Proposal 5: Define the CA configuration selection rules as:

| CA capability | Step 1 | Step 2 | Step 3 | Step 4 |
| --- | --- | --- | --- | --- |
| CA_C | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | N/A | N/A |
| CA_AX | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | Select the CA configurations with the largest number of bands and with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 3. |

Proposal 6: Define the test rules as:

| Tests | CA capability where the tests apply | CA configuration from the selected CA capability where the tests apply | CA Bandwidth combination to be tested in priority order | PCell CC configuration |
| --- | --- | --- | --- | --- |
| FDD 15 kHz + FDD 15 kHz | CA_C, CA_AX | Table TBD | Largest aggregated CA bandwidth combination | Any of CCs |
| TDD 30 kHz + TDD 30 kHz | CA_C, CA_AX | Table TBD | Largest aggregated CA bandwidth combination | Any of CCs |
| FDD 15 kHz + TDD 30 kHz | CA_AX | Table TBD | Largest aggregated CA bandwidth combination | FDD CC if supported, otherwise TDD CC |

- DL MIMO related UE demodulation requirement

Issue 2-1-1: Antenna configuration

Agreement:

- 2T2R and 2T4R

- FFS the antenna correlation:

- Proposal 1: XP-low

- Considering CPE has higher capability compared with handheld UE, the XP-low correlation can be taken as the starting point.

Proposal 7: For antenna correlation, use XP-low model.

Issue 2-1-2: TDD pattern

Agreement:

- Use legacy TDD pattern 7DS2U as the staring point

- FFS whether the new TDD pattern 30D4S6U will be used

Since there are still some potential maintenance issues about the new TDD pattern we introduced in Rel-18, We propose to use legacy TDD pattern for both FDD and TDD.

Proposal 8: Use legacy TDD pattern as the starting point.

Issue 2-2-2: RI

- Proposal 1:

  - Investigate the feasibility of introducing the RI requirement.

  - For antenna configuration and propagation channel, reuse the configuration from DL MIMO demodulation test. For other test parameters, reuse the configuration from legacy.

  - Decide the SNR and γ by simulation results

Considering AWGN static channel was used, for dedicated SNR, the reported RI will be maintained as a fixed value. Therefore, it is meaningless to introduce the RI requirement.

Proposal 9: Don’t introduce RI requirement for ATG MIMO.

- UL MIMO related BS Demodulation requirement

Issue 3-1-1: Whether to reuse existing requirements for ATG UL MIMO

Agreement:

- The legacy PUSCH test cases in Clause 8.2.1with 2Tx 2 layer will be applied for ATG

- FFS:

- Whether other legacy test cases are needed.

In Rel-18, the legacy PUSCH test cases in Clause 8.2.1, 8.2.2 and 8.2.3 with 1Tx test cases are applied for ATG, and incremental PUSCH test cases in Clause 8.2.14 were introduced.

The legacy PUSCH test cases 8.2.2 and 8.2.3 can only be applied for single layer. Therefore, no other legacy test cases are needed.

Proposal 10: No other legacy test cases are needed.

Issue 3-1-2: Whether to introduce new test cases for ATG UL MIMO

- Proposals

  - Proposal 1:

    - incremental PUSCH 2 layer test cases need to be introduced

  - Proposal 2:

    - Do not introduce new 2Tx PUSCH requirements for ATG scenario

To verify BS demodulation performance under 2 layer, XP antenna model, and high doppler scenario, incremental PUSCH 2 layer test cases need to be introduced.

Proposal 11: Introduce incremental PUSCH 2 layer test cases for ATG UL-MIMO.

Issue 3-1-3: Antenna configuration for incremental test

- Proposals

  - Proposal 1:

    - 2T2R, 2T4R and 2T8R with X-pol low correlation

  - Proposal 2:

    - 2T2R, 2T4R and 2T8R

  - Proposal 1:

    - 2T2R

As stated in WID, it is feasible for UE with dual-polarization antenna to support UL MIMO. Therefore, 2T2R, 2T4R, and 2T8R with XP low correlation can be taken as the starting point.

Proposal 12: For incremental test case, take 2T2R, 2T4R and 2T8R with X-pol low correlation as the starting point.

Issue 3-1-4: Propagation condition for incremental test

- Proposals

  - Proposal 1:

    - For incremental test case, use AWGN+200Hz for FDD, and AWGN+500Hz for TDD. The static propagation condition model from TS38.101-4 can be taken as the reference

Same as Rel-18, AWGN+200Hz can be used for FDD, and AWGN+500Hz can be used for TDD.

For the specific channel model, we think the static propagation condition from TS38.101-4 can be taken as the reference.

Proposal 13: For incremental test case, use AWGN+200Hz for FDD, and AWGN+500Hz for TDD. The static propagation condition model from TS38.101-4 can be taken as the reference.

Issue 3-1-5: Bandwidth & SCS for incremental test

- Proposals

  - Proposal 1:

    - 5MHz for FDD 15kHz SCS and 10MHz for TDD 30kHz SCS

Same SCS and CBW from Rel-18 can be reused, which are 5MHz for FDD 15kHz and 10MHz for TDD 30kHz.

Proposal 14: For incremental test case, use 5MHz for FDD 15kHz SCS and 10MHz for TDD 30kHz SCS.

Issue 3-1-6: Rank for incremental test

- Proposals

  - Proposal 1:

    - Rank 2

Issue 3-1-7: MCS for incremental test

- Proposals

  - Proposal 1:

    - 16QAM Table 1 MCS12, 64QAM Table 1 MCS 20 and 256QAM Table 2 MCS 20 for initial simulation

  - Proposal 2:

    - consider low modulation order for high rank transmission

  - Proposal 3:

    - MCS 16

Since the spatial independence comes from antenna polarization, then only Rank 2 can be supported. Regarding MCS, we propose to consider 16QAM Table 1 MCS12, 64QAM Table 1 MCS 20 and 256QAM Table 2 MCS 20 for initial simulation.

Proposal 15: For incremental test case, consider Rank 2, 16QAM Table 1 MCS12, 64QAM Table 1 MCS 20 and 256QAM Table 2 MCS 20 for initial simulation.

Issue 3-1-8: DMRS port for incremental test

- Proposals

  - Proposal 1:

    - DMRS port 2 {0,1}

Since Rank 2 is considered here, 2 port DMRS shall also be supported.

Proposal 16: For incremental test case, DMRS port 2 {0,1} for UL MIMO.

Issue 3-1-9: TPMI for incremental test

- Proposals

  - Proposal 1:

    - TPMI index 0

TPMI index 0 shall be used for two layer and two ports transmission.

Proposal 17: For incremental test case, TPMI index 0 for UL MIMO.

Issue 3-1-10: Others

- Proposals

  - Proposal 1:

    - Type A

    - HARQ retransmission

    - 70% of maximum throughput

  - Proposal 2:

    - Others same as existing Rel-15 BS PUSCH demodulation requirement with 2Tx

For the other test configuration not mentioned before, existing configuration as R15 BS PUSCH demodulation requirement with 2Tx can be reused.

Proposal 18: For the other test configuration not specifically mentioned, existing configuration as R15 BS PUSCH demodulation requirement with 2Tx can be reused.

- Conclusion

Based on the discussion above, the following proposals are concluded.

DL CA

Proposal 1: Cover the CA configuration of FDD 15kHz + FDD 15kHz.

Proposal 2: Use the legacy TDD pattern as the starting point.

Proposal 3: For CA requirement, use Rank 2 as the starting point.

Proposal 4: Specify single carrier requirements for all supported bandwidth sizes for FDD 15kHz and TDD 30kHz.

Proposal 5: Define the CA configuration selection rules as:

| CA capability | Step 1 | Step 2 | Step 3 | Step 4 |
| --- | --- | --- | --- | --- |
| CA_C | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | N/A | N/A |
| CA_AX | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | Select the CA configurations with the largest number of bands and with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 3. |

Proposal 6: Define the test rules as:

| Tests | CA capability where the tests apply | CA configuration from the selected CA capability where the tests apply | CA Bandwidth combination to be tested in priority order | PCell CC configuration |
| --- | --- | --- | --- | --- |
| FDD 15 kHz + FDD 15 kHz | CA_C, CA_AX | Table TBD | Largest aggregated CA bandwidth combination | Any of CCs |
| TDD 30 kHz + TDD 30 kHz | CA_C, CA_AX | Table TBD | Largest aggregated CA bandwidth combination | Any of CCs |
| FDD 15 kHz + TDD 30 kHz | CA_AX | Table TBD | Largest aggregated CA bandwidth combination | FDD CC if supported, otherwise TDD CC |

DL MIMO

Proposal 7: For antenna correlation, use XP-low model.

Proposal 8: Use legacy TDD pattern as the starting point.

Proposal 9: Don’t introduce RI requirement for ATG MIMO.

UL MIMO

Proposal 10: No other legacy test cases are needed.

Proposal 11: Introduce incremental PUSCH 2 layer test cases for ATG UL-MIMO.

Proposal 12: For incremental test case, take 2T2R, 2T4R and 2T8R with X-pol low correlation as the starting point.

Proposal 13: For incremental test case, use AWGN+200Hz for FDD, and AWGN+500Hz for TDD. The static propagation condition model from TS38.101-4 can be taken as the reference.

Proposal 14: For incremental test case, use 5MHz for FDD 15kHz SCS and 10MHz for TDD 30kHz SCS.

Proposal 15: For incremental test case, consider Rank 2, 16QAM Table 1 MCS12, 64QAM Table 1 MCS 20 and 256QAM Table 2 MCS 20 for initial simulation.

Proposal 16: For incremental test case, DMRS port 2 {0,1} for UL MIMO.

Proposal 17: For incremental test case, TPMI index 0 for UL MIMO.

Proposal 18: For the other test configuration not specifically mentioned, existing configuration as R15 BS PUSCH demodulation requirement with 2Tx can be reused.

- Reference

[1] R4-2504760, WF on NR_ATG_enh demodulation requirement, CMCC