3GPP TSG-RAN WG4 Meeting #115		R4-2506428

# Malta, MT, 19th - 23rd May, 2025

Agenda Item:	7.28.5.2

Source: 	CMCC

Title: 			 Discussion on RRM impact of LP-WUR

Document for:	Discussion

- Introduction

In last meeting, RAN4 further discussed detail LP-WUR requirements, MR RRM relaxation requirements and measurement accuracy related issues. The WF has been approved in [1]. In this contribution, we further analyze the RRM impact of LP-WUR, and provide our proposals.

- Discussion

  - General aspects

Issue 1-1-10: Considerations on higher priority frequency layer

Agreement:

- Support relaxed higher priority frequency layer search at case 1

Recommendations:

- Further discussion on detail on requirements and related tentative agreement.

For Case 1, in order to guarantee UE power saving gain, we are ok to only apply the relaxed higher priority layer measurement requirement, that the UE shall search for inter-frequency and inter-RAT layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is (60 * Nlayers) and K2=60.

For Case 3, since MR is still running, the legacy logic can be reused, that

- If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search

- If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-frequency layers of higher priority at least Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter.

- Meanwhile, since the scaling factor 16 has agreed to be used in Case3, we are also fine to consider the scaling factor 16 for high priority measurement for UE power saving demand.

- Proposal 1: For Case 1, UE shall search for inter-frequency and inter-RAT layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is (60 * Nlayers) and K2=60.

Proposal 2: For Case 3,

- If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, then the UE shall search for inter-frequency layers of higher priority at least every 16 * Thigher_priority_search, where Thigher_priority_search is (60 * Nlayers)

- If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, then the UE shall search for and measure inter-frequency layers of higher priority at least Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter with scaling factor 16.

  - Detail LP-WUR requirements at RRC_IDLE/INACTIVE state

Issue 1-2-2-1: Lower bound on LP-SS measurement periodicity

Recommendations:

- Continue discussion

Agreement from RAN4#112bis

- Measurement delay requirements for LP-WUR based on LP-SS in IDLE/Inactive mode shall be defined based on LP-SS periodicity.

Agreement from RAN1#120bis

- Regarding whether to support additional sync signal to LP-SS, support the following for LP-WUS with M=1, M=2:

- Option 1A: No additional sync signal, LP-SS periodicity =320ms

    - Additionally support LP-SS periodicity of 160ms

- RAN1 has introduced additional LP-SS periodicity of 160ms for M=1 and M=2, target for sync performance.

- Since anyway UE need to detect each LP-SS for sync, it can be also for measurement usage, no additional power consumption will be caused. Therefore, we think no lower bound is needed, the previous RAN4 agreement shall be upheld.

- Proposal 3: Lower bound is not needed on top of LP-SS periodicity when define the measurement delay requirement.

Issue 1-2-4-1: On LR measurement filtering requirement

Agreement:

- The UE shall filter the [SS-RSRP and SS-RSRQ] measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by [LO periodicity /2] for SSB based LR.

Recommendations:

- Continue discuss on whether to define and how to define LP-SS based LR filtering requirements

First of all, UE shall also filter the LP-RSRP and LP-RSRQ measurement of serving cell using at least 2 measurements.

Then, in legacy, since there are multiple SMTCs in one I-DRX cycle, to guarantee the filter gain, two measurements shall be spaced by DRX cycle/2. However, for LP-SS measurement, the sampling period is LP-SS periodicity, and only one LP-SS resource in one LP-SS periodicity. Therefore, such filtering requirement is not needed.

- Proposal 4: UE shall filter the LP-RSRP and LP-RSRQ measurement of serving cell using at least 2 measurements.

- Proposal 5: As long as the measurement delay requirement is defined based on LP-SS periodicity and no lower bound, no need to further define the filtering space for LP-RSRP and LP-RSRQ measurement.

Issue 1-2-6: On MR wake up delay related requirements for LP-WUR and RAN1 LS R1-2501624

In RAN1#118bis, it was agreed to support 3 candidate values for the wake-up delay capability a UE reports for LP-WUS operation in IDLE/INACTIVE mode. In addition, the following agreement was made in RAN1#120: Agreement For UE capability report on the wake-up delay: - Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[70ms], [500ms] and [900ms]}. - The reported values assume SSB periodicity of 20ms, where [70ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization. - FFS: translation of wake-up delay for different SSB periodicities - FFS: different sets of 3 candidate value for different SSB periodicities Note that the definition of wake-up delay is the minimum gap time between LP-WUS reception and MR to start PDCCH monitoring. RAN1 respectfully asks RAN4 to check and confirm the number of SSBs currently assumed by RAN1 for MR synchronization for SSB periodicity of 20ms. RAN1 also respectfully asks RAN4 to confirm if it is reasonable to assume the same number of SSBs for MR synchronization for different SSB periodicities.

Issue 1-2-6-1: On the number of SSBs

Agreement:

- Aim to conclude the issue by Tuesday in RAN4 #115.

Recommendations:

- For the number of samples and side conditions could consider:

- 1) determine the SSB number based on minimum number for AGC, timing etc., without link to any particular side conditions

- 2) determine the SSB number based on minimum number for AGC, timing etc., considering LR side condition.

Issue 1-2-6-2: On SSBs number for different SSB periodicities

Agreement

- Same SSB number for different SSB periodicity

Agreement

- When SSB periodicity is 5 ms or 10ms, a lower bound 20 ms will be used for MR wake up delay.

- Note: This above agreement based on the assumption that the wake up duration is defined as the ramping time + SSB number * SSB periodicity

At least following two side conditions will impact the SSB number required for AGC and sync during wake-up.

- Es/Iot from MR side

- Whether LR could provide coarse time sync information for MR

For Es/Iot, using -2dB from handover side condition as worst case is reasonable, and typically, when UE fulfill the LP-WUS monitoring entry condition, the Es/Iot of MR will be higher than -2dB in our point of view. In handover requirement, 1 SSB is needed for known case, and 4 SSBs are needed for unknown case.

For the rough sync information sharing between LR and MR, we think the worst case can be considered, that LR could not provide valid rough sync information for MR.

With above consideration, for UE to finish AGC and sync, we think 2 SSB samples are sufficient for deep sleep mode which similar as handover known case, and 4 SSB samples are sufficient for ultra-deep sleep mode which similar as handover unknown case.

- Proposal 6: 2 samples for sync after UE woke up from deep sleep state and 4 samples for sync after UE woke up from ultra-deep sleep state.

- Conclusion

Based on the discussion above, the following observation and proposals are concluded.

- Proposal 1: For Case 1, UE shall search for inter-frequency and inter-RAT layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is (60 * Nlayers) and K2=60.

Proposal 2: For Case 3,

- If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, then the UE shall search for inter-frequency layers of higher priority at least every 16 * Thigher_priority_search, where Thigher_priority_search is (60 * Nlayers)

- If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, then the UE shall search for and measure inter-frequency layers of higher priority at least Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter with scaling factor 16.

- Proposal 3: Lower bound is not needed on top of LP-SS periodicity when define the measurement delay requirement.

- Proposal 4: UE shall filter the LP-RSRP and LP-RSRQ measurement of serving cell using at least 2 measurements.

- Proposal 5: As long as the measurement delay requirement is defined based on LP-SS periodicity and no lower bound, no need to further define the filtering space for LP-RSRP and LP-RSRQ measurement.

- Proposal 6: 2 samples for sync after UE woke up from deep sleep state and 4 samples for sync after UE woke up from ultra-deep sleep state.

- Reference

- R4-2504908, WF on RRM requirements for NR_LPWUS, Moderator (vivo)