3GPP TSG-RAN WG4 Meeting # 115		R4-2506430

Malta, MT, 19th - 23rd May, 2025

Agenda Item:	7.28.6

Source: 	CMCC

Title: 			 Discussion on test cases for LP-WUS

Document for:	Discussion

- Introduction

Rel-19 RRM core requirements for LP-WUS were discussed during RAN4#112 to RAN4#114bis. According to the TU allocation and work plan, RAN4 is supposed to start the performance part for the WI. In this contribution, we will provide our views on the test feasibility for R19 LP-WUS.

- Discussion

- Currently, RAN4 core part is focusing on LR measurement requirements, MR relaxation requirements, and the entry/exit criteria evaluation requirements among multiple measurement states.

- In Case#1, with MR ultra-deep sleep and LR measuring the LP-SS/SSB, except for higher frequency carrier re-selection, we believe no cell selection and cell re-selection will be happened. If relaxed higher frequency carrier cell re-selection requirement will be applied for Case#1, the testing time effort for higher frequency carrier re-selection will be too much, we are ok not to introduce test case for Case#1.

- In Case#3, with MR measuring serving and neighbour cell in relaxation mode and LR measuring the LP-SS/SSB, cell re-selection can be happened and tested. From the test coverage perspective, all intra-frequency cell re-selection, inter-frequency cell re-selection including lower and equal priority re-selection and higher priority re-selection, inter-RAT cell re-selection shall be covered. To reduce the testing time effort, DRX= 320ms or 640ms shall be configured as test setup.

Proposal 1: For Case#1, if relaxed higher frequency carrier cell re-selection requirement will be applied, we are ok not to introduce test case considering the test feasibility and testing time effort.

Proposal 2: For Case#3, at least to introduce following tests to guarantee the test coverage

- intra-frequency cell re-selection

- inter-frequency cell re-selection including lower and equal priority re-selection and higher priority re-selection

- inter-RAT cell re-selection

Proposal 3: Due to the large scaling factor for MR relaxation requirement, DRX 320ms or 640ms shall be configured as test setup to minimize the testing time effort.

- Conclusion

Based on the discussion above, the following observation and proposals are concluded.

Proposal 1: For Case#1, if relaxed higher frequency carrier cell re-selection requirement will be applied, we are ok not to introduce test case considering the test feasibility and testing time effort.

Proposal 2: For Case#3, at least to introduce following tests to guarantee the test coverage

- intra-frequency cell re-selection

- inter-frequency cell re-selection including lower and equal priority re-selection and higher priority re-selection

- inter-RAT cell re-selection

Proposal 3: Due to the large scaling factor for MR relaxation requirement, DRX 320ms or 640ms shall be configured as test setup to minimize the testing time effort.