3GPP TSG-RAN WG4 Meeting # 115		R4-2506431

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.31.5.1

Source: 	CMCC

Title: 			   Discussion on the RRM requirement of (e)Redcap UE for NTN phase3

Document for:	Discussion

- Introduction

In last meeting, RAN4 discussed (e)RedCap UEs over NR FR1-NTN. The WF has been approved in [1].

In this contribution, we further discuss the RRM impact and specific requirements.

- General consideration on RRM requirements

  - What RRM requirements are defined for (e)RedCap UE with FR1-NTN

Issue 1-1-3: MDT requirements

Agreement:

- For MDT requirements in RRC_IDLE and RRC_INACTIVE state,

- Deprioritize to define Minimization of Drive Tests (MDT) requirements in RRC_IDLE state and RRC_INACTIVE state for (e)RedCap UE with FR1-NTN.

We think it is beneficial to support MDT capability for Redcap UE over NTN.

Firstly, for NTN network with large/moving cell coverage, it is barely impossible to use the traditional drive test, which requires support from both operation and testing equipment, the difficulty and cost of network optimization will increase a lot. While MDT could help network to optimize the scheduling with low cost.

Secondly, for large cell coverage, especially for lonely area, it is hard to assume that network can acquire MDT report from handheld UE, the Redcap UE can serve as a supplement in this case.

Thirdly, the link performance seen between handheld UE and Redcap UE may be different, especially for 1Rx Redcap and HD-FDD Redcap. Therefore, although NTN network has already have MDT report from handheld UE, the MDT report from Redcap UE can still provide more information to network.

Lastly, we understand there is power consumption concern for Redcap UE. However, considering it is up to UE whether to support this capability or not, and UE who support this feature will stop the memory when the cached data over the threshold, and MDT will be started only after receiving the network configuration, we think all these approach can guarantee that the feature will be applied for UE without power consumption issue and only when necessary from network side.

- Proposal 1: Define the MDT requirements in RRC_IDLE and RRC_INACTIVE state for Redcap over NTN. The legacy requirement from NTN can be reused.

  - Features to be considered for (e)RedCap UE with FR1-NTN

Issue 1-2-1-1a: The proximity distance between two SMTCs

<Way Forward>:

When (e)RedCap UE is configured with more than one SMTC on a SAN carrier, two SMTC occasions outside measurement gap are considered colliding if at least one of the following conditions is met:

- the two SMTC occasions are fully or partially overlapping in time domain, or

- the magnitude of the distance between two SMTC occasions in time domain is less than or equals to Y ms.

  - Y=5 ms

For normal UE, the proximity distance between two SMTCs is 4ms. Compared with normal UE, the baseband processing capability of (e)RedCap UE may be lower, thus, the relaxed proximity distance can be considered, 5ms is preferred.

- Proposal 2: For (e)RedCap UE, the proximity distance between two SMTCs is 5ms.

- The impact of HD-FDD

  - General HD-FDD related impact

Issue 3-1-1: General HD-FDD related impact

Agreement:

- The legacy requirements and applicable conditions can be reused as baseline:

  - Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state

  - Handover interruption time

  - Random access

  - SA: RRC Connection Release with Redirection

  - [Minimum requirement for L1 indication for RLM and LR]

  - Scheduling availability of UE performing intra/inter measurements

RAN4 to further discuss:

- The legacy requirements and applicable conditions can be reused as baseline for defining requirements of RedCap UE in NTN scenario, and discuss whether there is spec impact for the following requirements by HD-FDD:

  - FFS: Scheduling availability of UE during L1-RSRP measurement

  - Other requirement is not precluded

- Candidate options for the potential spec impact for each of the above requirements (if needed):

  - Option a: the requirement is extended, e.g., the requirement does not count resources, channels, SSB, etc if they overlap with the other direction as specified a valid case in RAN1 specification.

  - Option b: Clarify that the requirements for CSI-RS and PRS apply provided that no CSI-RS and PRS resource is dropped due to collision with UL transmission during the measurement period.

  - Option c: The requirement applies when there is at least one RS within each measurement interval.

  - FFS the same or different option for DRX and non-DRX

Firstly, for minimum requirement for L1 indication for RLM and LR, the legacy applicable condition can be reused, that for each RLM-RS configuration, at least one RLM-RS sample must fall with DL occasion within an indication period.

Proposal 3: For minimum requirement for L1 indication for RLM and LR, reuse the legacy applicable condition, that for each RLM-RS configuration, at least one RLM-RS sample must fall with DL occasion within an indication period.

Then, for scheduling availability of UE during L1-RSRP measurement, the main rule is that scheduling restrictions apply for transmission on PUCCH/PUSCH/SRS during the CBD evaluation period, as CBD evaluation is prioritized over UL transmission. However, with RAN1’s design, the CBD evaluation may not always prioritized over UL transmission. Therefore, we propose to update the side condition as: scheduling restrictions apply for transmission on PUCCH/PUSCH/SRS during the CBD evaluation period, when CBD evaluation is prioritized over UL transmission.

Proposal 4: For scheduling availability of UE during L1-RSRP measurement, update the principle as scheduling restrictions apply for transmission on PUCCH/PUSCH/SRS during the CBD evaluation period, when CBD evaluation is prioritized over UL transmission.

Lastly, for CSI-RS and PRS based measurement requirements, if network indicates UL overriding DL, the CSI-RS and PRS measurement samples may be punctured. Therefore, the applicable condition shall be defined, the requirement applies provided that there is at least one RS within each measurement interval.

Proposal 5: For CSI-RS and PRS based measurement requirements, the requirements can be applied provided that there is at least one RS within each measurement interval.

- Others

  - UE capabilities for (e)RedCap UE with FR1-NTN bands

Issue 4-2-1: How to declare the capabilities for (e)RedCap UE with FR1-NTN bands?

<Way Forward>:

- Proposal 1: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17).

- Proposal 2: Introduce new capability to declare support over RedCap over NTN.

- Proposal 3: RAN4 to not discuss or conclude on UE capability signaling and structure for RedCap NTN, unless specific requests are received from other working groups via LSs.

For (e)Redcap UE capability declaring, both Proposal 1 and Proposal 2 are workable. Considering the principle of no need to introduce new capability if existing capability can be used, we slightly prefer the implicit way to save the workload of RAN2.

Proposal 6: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17).

- Conclusion

Based on the discussion above, the following proposals are concluded.

- Proposal 1: Define the MDT requirements in RRC_IDLE and RRC_INACTIVE state for Redcap over NTN. The legacy requirement from NTN can be reused.

- Proposal 2: For (e)RedCap UE, the proximity distance between two SMTCs is 5ms.

Proposal 3: For minimum requirement for L1 indication for RLM and LR, reuse the legacy applicable condition, that for each RLM-RS configuration, at least one RLM-RS sample must fall with DL occasion within an indication period.

Proposal 4: For scheduling availability of UE during L1-RSRP measurement, update the principle as scheduling restrictions apply for transmission on PUCCH/PUSCH/SRS during the CBD evaluation period, when CBD evaluation is prioritized over UL transmission.

Proposal 5: For CSI-RS and PRS based measurement requirements, the requirements can be applied provided that there is at least one RS within each measurement interval.

Proposal 6: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17).

- Reference

- R4-2504912, WF on RRM requirements for NR_NTN_Ph3_Part1, Moderator (CATT)