3GPP TSG-RAN WG4 Meeting #115                                     	            R4-2506446

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:   	7.31.3.2

Source:   	LG Electronics

Title: 	Discussion of requirements for maintaining phase continuity and power consistency for OCC with PUSCH

Document for:   	Discussion

- Introduction

In this contribution, we will provide our view on the application of phase continuity requirements for DMRS bundling to OCC with PUSCH.

- Discussion

The following question was asked for RAN4 to provide an answer in LS [1]:

RAN1 agreed the following: RAN1 assumes that the UE is required to maintain phase continuity and power consistency for the duration of one OCC group with PUSCH. - FFS: under which conditions the above applies, e.g. under the same conditions that phase continuity applies for DMRS bundling.. RAN1 respectfully ask RAN4 whether the same phase continuity requirements as for DMRS bundling can be applied for OCC length 2 and/or OCC length 4 under the same conditions as for DMRS bundling, or if new requirements are needed.

Since the following factors in TS 38.101-5 that cause phase discontinuity for DMRS bundling are also valid for OCC with PUSCH, we believe that the existing phase continuity requirements for DMRS bundling can be applied to OCC with PUSCH.

6.4.2.2 Phase continuity requirements for DMRS bundling For bands that NTN UE indicates the support of DMRS bundling, when the NTN UE is configured with DMRS bundling, the maximum allowable difference between the measured phase value in any slot p-1 and slot p, or slot 0 and any slot p for each antenna connector shall satisfy the requirements as listed in Table 6.4.2.5-1 of TS 38.101-1 [5] for the measurement conditions defined in Table 6.4.2.5-2 of TS 38.101-1 [5], within a measurement time window limited by the UE capability of maximum duration for DMRS bundling [maxDurationDMRS-Bundling-r17] for GSO scenario and [maxDurationDMRS-Bundling-NTN-NGSO-r18] for NGSO scenario , and defined for each frequency band separately. The phase value for each slot is measured as shown in Annex F.9 of TS 38.101-1 [5]. These requirements apply to PUCCH and PUSCH transmissions with DFT-s-OFDM and CP-OFDM waveforms. The above requirements are applicable when all the following conditions are met within the measurement time window: - RB allocation in terms of length and frequency position does not change, and intra-slot and inter-slot frequency hopping is not activated. - Modulation order does not change. - No network commanded TA takes effect. - The TPMI precoder does not change. - There is no change in UE transmission power level, and no change in the level of P-MPR applied by the UE. - UE is not scheduled with uplink transmission of other physical channel/signal in-between the PUSCH or PUCCH transmissions. - Doppler conditions are set to zero and delay conditions are set to constant.

On the other hand, in the NR-NTN scenario, there are several operations that allow the UE to autonomously adjust the execution time under its control, such as UE specific TA update, common TA update, and UE pre-compensation update (e.g., GNSS position and/or ephemeris updates). If these operations are performed by the UE within an OCC group, it is difficult to maintain the orthogonality of the OCC due to the phase discontinuity problem. Therefore, for OCC with PUSCH, it is necessary to additionally include “No UE specific and common TA updates” as conditions that should be met for applying the phase continuity requirements.

Proposal: RAN4 provides an answer to the question on RAN1 LS [1] as follows:

- The phase continuity requirements for DMRS bundling can be applied to OCC length 2 and/or OCC length 4, with the addition of “No UE specific and common TA updates” as conditions that should be met to apply the phase continuity requirements.

- Conclusion

In this contribution, we discussed how RAN4 provides feedback on RAN1’s question in LS [1]. The following proposal is given.

Proposal: RAN4 provides an answer to the question on RAN1 LS [1] as follows:

- The phase continuity requirements for DMRS bundling can be applied to OCC length 2 and/or OCC length 4, with the addition of “No UE specific and common TA updates” as conditions that should be met to apply the phase continuity requirements.

- Reference

[1] R4-2505305, “LS on requirements for the phase continuity and power consistency for OCC with PUSCH in NR NTN Ph3,” RAN1.