3GPP TSG-RAN WG4 Meeting#115		R4-2506448

Malta, MT, May 19th -23rd, 2025

Agenda item:	7.7.3

Source: 	Spreadtrum, UNISOC

Title: 	Discussion on UE RF requirements for LB-LB CA switching

Document for:	Discussion

Introduction

In RAN4#114bis meeting, a WF [1] was approved. In this contribution, we want to provide some views on UE RF requirements for LB-LB CA switching.

- Way Forward Issue 1: Switching period capability - The switching period capability can be defined per band pair per band combination. Issue 2: Switching period location - Await outcome of RAN1 design Issue 3: Time mask for switching - Define the Tx on-off time mask requirements - FFS on how to define the Rx switching requirements to reflect where the switching period occurs - RAN4 has understanding that RAN1 will take switching period and timing advance into account for RAN1 scheduling pattern for low band CA via switching Issue 4: Considerations related to the Tx on-off time mask - Option 1: Specify transmit on/off time mask for FDD UL carrier/Pcell for switching between LB(FDD)-LB(SDL) - Option 2: Specify on->off time mask for switching from FDD to SDL carrier and specify off->on time mask for switching from SDL to FDD carrier - Other options are not precluded Issue 5: Test methodology - Companies are encouraged to contribute proposals related to the testability of the requirement on the time mask for switching

Discussion

- Time mask for switching

Regarding time mask for switching, according to the WF, it is necessary to discuss how to define the Rx switching requirements to reflect where the switching period occurs. In our understanding, there are no Rx switching requirements in the core part. Rx switching requirements can be moved to the RRM part to discuss Rx switching requirements.

Proposal 1: Rx switching requirements can be moved to RRM part to define.

- Considerations related to the Tx on-off time mask

According to the RAN1’s agreement, for switching from SDL carrier to FDD carrier, the switching gap is always assumed at the “switch from” carrier, i.e., SDL carrier. And there is still under discussion for RAN1 for switching from FDD carrier to SDL carrier.

Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap, - For switching from SDL carrier to FDD carrier, the switching gap is always assumed at the “switch from” carrier, i.e., SDL carrier. - For switching from FDD carrier to SDL carrier, downselect one of the following solutions - Alt 1: Switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier - Alt 2: NW configures whether the switching gap is at the FDD carrier or SDL carrier - Alt 3: Switching gap is always assumed at the SDL carrier

In our understanding, we can specify specify on->off time mask for switching from FDD to SDL carrier and specify off->on time mask for switching from SDL to FDD carrier as follow, the switching gap can wait for RAN1’s conclusion.

Proposal 2: Specify on->off time mask for switching from FDD to SDL carrier and specify off->on time mask for switching from SDL to FDD carrier. On->off time mask for switching from FDD to SDL carrier can be in Figure 1. Off->on time mask for switching from SDL to FDD carrier can wait for RAN1’s conclusion.

Conclusions

In this paper, we provide our views on UE RF requirements for LB-LB CA, we have made the following proposals.

Proposal 1: Rx switching requirements can be moved to RRM part to define.

Proposal 2: Specify on->off time mask for switching from FDD to SDL carrier and specify off->on time mask for switching from SDL to FDD carrier. On->off time mask for switching from FDD to SDL carrier can be in Figure 1. off->on time mask for switching from SDL to FDD carrier can wait for RAN1’s conclusion.

- Reference

- R4-2505223, WF on low-low band switching, Apple. TSG-RAN4#114bis meeting