3GPP TSG RAN WG4 Meeting #115	R4-2506457

St Julian’s, Malta, 19th – 23rd May, 2025

Agenda item:			7.30.2

Source:	vivo

Title:	On UE capability for XR WI

Document for:	Discussion

# Introduction

In the previous RAN4 meetings, following UE capabilities in XR WI were agreed as captured in the WF [1, 2].

## Time offset requirement for measurement gap skipping <Agreement>: - Baseline feature 5 ms - Optional capability 3 ms, FFS if any condition is needed <Agreement>: - Not define additional condition for the optional capability of 3 ms. ## UE assistance information <Agreement>: Capture the following agreement in the LS to RAN1/2. Meanwhile, it is not precluded to update the number of 1 s if any issue will be identified in the future meeting. - The information in UAI: a ratio (R) of gap occasions that is recommended for cancellation during a time period of 1 s. - R ∈ {0, 20, 40, 60}%

In this contribution, we provide our views on UE capabilities for XR WI.

# Discussion

In the last RAN1 meeting, FG 64-1 for XR WI was agreed. The details are as in the following table.

| Features | Index | Feature group | Components |
| --- | --- | --- | --- |
| 64. NR_XR_Ph3 | 64-1 | Enabling TX/RX during measurement gap scheduling restriction [by DCI format 0_1/1_1] | 1. Reception/transmission during measurement gaps/scheduling restrictions is enabled fully cancelled based on explicit indication by DCI 2. DCI format 0_1/1_1[/0_2/1_2/0_3/1_3] indication for enabling reception/transmission in gap/restriction 3. UE behaviour for indication field value of “0” - Option 1: For explicit indication by DCI, bit value equal to “0” means UE ignores the indication of this field in the DCI. - Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as of Rel-18 per legacy behaviour. 4. The minimum time offset from the end of PDCCH carrying the gap/restriction cancellation to the start of cancelled gap/restriction FFS: other component (which may be separate FG) |

For the UE capabilities introduced in RAN4, FG 64-1 should be the prerequisite feature.

Proposal 1: FG 64-1 is the prerequisite feature for UE capability of 3ms time offset and UAI of ratio indication.

For both of the UE capability of 3ms time offset and UAI of ratio indication, the granularity should be per-UE. There is no need to have further smaller granularity.

Proposal 2: For both UE capability of 3ms time offset and UAI of ratio indication, the reporting granularity is per-UE.

With the agreements made in RAN4 and above analysis, the UE feature for XR WI is proposed in Table.

Proposal 3: UE feature for Rel-19 XR WI is agreed as in Table 1.

Table 1: UE feature for XR WI

- NR_XR_Ph3

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 57.NR_XR_Ph3 | 57-1 | Time offset for enabling Tx/Rx during measurement gap | Support of 3ms time offset for enabling Tx/Rx during measurement gap indicated by DCI | 64-1 | Yes | N/A | UE does not support 3ms time offset for enabling Tx/Rx during measurement gap indicated by DCI | Per UE | N/A | No |  | Candidate value: [3ms] | Optional with capability signalling |
| 57.NR_XR_Ph3 | 57-2 | Supports indication of a ratio of gap occasions for enabling Tx/Rx | Supports of indication of a ratio of gap occasions during a time period of 1 s for enabling Tx/Rx. | 64-1 | Yes | N/A | UE cannot recommend ratio of gap occasions for enabling Tx/Rx | Per UE | N/A | No |  | Candidate value: [0, 20%, 40%, 60%] | Optional with capability signalling |

# Summary

In this contribution, we further provided views on general aspects for NR FR2 multi-Rx chain DL reception WI. Following proposals and observations are present.

Proposal 1: FG 64-1 is the prerequisite feature for UE capability of 3ms time offset and UAI of ratio indication.

Proposal 2: For both UE capability of 3ms time offset and UAI of ratio indication, the reporting granularity is per-UE.

Proposal 3: UE feature for Rel-19 XR WI is agreed as in Table 1.

- References

- R4-2420103	WF on RRM requirements for NR_XR_Ph3, Nokia

- R4-2504911	WF on RRM requirements for NR_XR_Ph3, Nokia