3GPP TSG RAN WG4 Meeting #115	R4-2506471

Malta, MT, 19th – 23rd May, 2025

Agenda item:			7.22.2

Source:	vivo

Title:	Discussion on general aspects for AI/ML for NR air interface Phase 2

Document for:	Discussion

- Introduction

On the general aspects for AI/ML, in the last meeting, RAN4 further discussed the issues on LCM discussion . In this contribution, we further provide our views for the issue on RRM requirements for LCM. The latest way forward was captured in [1].

- Discussion

Agreement: - RAN4 has the following understanding and need further check whether the understanding is aligned with RAN1 and RAN2: - AI/ML functionality activation refers to the process of enabling an applicable functionality to perform inference - functionality deactivation refers to the process of network deactivating an active functionality Agreement: - FFS on whether the deactivation requirements will be introduced - The starting point is when the UE receives the signalling to commence deactivation - FFS when the endpoint for deactivation is - FFS whether fallback delay is the same as deactivation delay or has some other components Agreement: - For beam management use case - FFS on defining the functionality activation delay requirements for the first periodic and semi-persistant reporting based on AI/ML - Define functionality activation delay requirement for the first aperiodic reporting based on AI/ML - FFS on the definition of first reporting

Regarding deactivation, there were not many conclusions in the RAN2 discussions related to the deactivation process. The most relevant description is as follows. When the UE reports that a currently applicable functionality has become non-applicable, the NW will deactivate that active functionality.

For now, RAN2 will not define terminology specific to the activation or deactivation for AI/ML models. Can come back to this discussion later. - When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable.

When a functionality becomes non-applicable and the network has already deactivated it, based on our understanding, the UE will no longer continue to perform predictive behavior or send CSI reports carrying related prediction results. Therefore, it is difficult to identify a subsequent fixed time point as the ending point.

As for fall-back, other working groups have not conducted further discussions or provided signaling support for this process. Whether to define related latency requirements and specific details would require more conclusive support before proceeding.

Proposal 1: RAN4 not to introduce deactivation/fall-back delay requirements

For the functionality activation in the beam management use case, we will analyze three types of CSI reporting:

- Periodic CSI reporting

For periodic CSI reporting, according to the following RAN2 conclusion, if the periodic CSI configuration is provided in the CSI report configuration in Step 3, the applicable functionalities can be regarded as activated upon the UE reporting the applicable functionalities via RRCReconfigurationComplete in Step 4. In other words, when the inference configuration is fully provided in Step 3, the starting point of this activation process is when the UE reports the applicable functionalities in Step 4. If the full inference configuration is not provided in Step 3, the corresponding signaling details require further discussion between RAN1 and RAN2. For this case, we can await further conclusions (if any).

- If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5).

Regarding the ending point of this activation process, we believe it is appropriate to define the UE's first reporting of inference results as the ending point. Therefore, after this functionality is activated and until the UE reports the prediction results, there are no other reporting behaviors in between. Moreover, when the UE reports the prediction results, it also indicates to some extent that the functionality is operational and capable of outputting predictions.

As for the definition of the first reporting, it can be defined as the first prediction result report after the UE reports the applicable functionalities via RRCReconfigurationComplete.

Based on the above analysis, it can be inferred that the latency duration is the same as the prediction delay discussed in beam management. The corresponding activation delay is:

Tfunctionality_activation_periodic = TL1-RSRP_Prediction_Period_CSI-RS

Where the TL1-RSRP_Prediction_Period_CSI-RS is,

| Configuration | TL1-RSRP_Prediction_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TCSI-RS+ Tprediction_processing) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TCSI-RS)+ Tprediction_processing) |
| DRX cycle > 320 ms | ceil(M*P*N)*TDRX + Tprediction_processing |
| NOTE 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Tprediction_processing is time for prediction post-processing. NOTE 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. |  |

Proposal 2: For periodic CSI reporting, the starting point of the functionality activation delay is upon the UE reporting the applicable functionalities via RRCReconfigurationComplete in Step 4 when the inference configuration is fully provided in Step 3. FFS on the case that the full inference configuration is not provided in Step 3

Proposal 3: For periodic CSI reporting, the functionality activation delay is defined as below:

Tfunctionality_activation_periodic = TL1-RSRP_Prediction_Period_CSI-RS

Where the TL1-RSRP_Prediction_Period_CSI-RS is the prediction delay and it is defined as

| Configuration | TL1-RSRP_Prediction_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TCSI-RS+ Tprediction_processing) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TCSI-RS)+ Tprediction_processing) |
| DRX cycle > 320 ms | ceil(M*P*N)*TDRX + Tprediction_processing |
| NOTE 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Tprediction_processing is time for prediction post-processing. NOTE 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. |  |

- Semi-persistent reporting and aperiodic CSI reporting

For both aperiodic CSI reporting and semi-persistent CSI reporting, regardless of whether the applicable functionality is reported after step 3 or step 5, according to the following conclusions, the activation of the applicable functionality follows the legacy CSI framework.

- Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework: Semi-persistent reporting, activated by MAC CE/DCI Aperiodic CSI reporting, activated by DCI

For semi-persistent reporting, the starting point of the activation process is the reception of the activation command triggered by MAC CE/DCI. For aperiodic CSI reporting, the starting point of the activation process is the reception of the activation command triggered by DCI.

As for the ending point, it is the same as the discussion above for periodic CSI reporting—specifically, the first predicted result reported after the UE reports the applicable functionalities via RRCReconfigurationComplete.

For Semi-persistent reporting, the corresponding activation delay is (taking MAC CE as an example)

Tfunctionality_activation_SP = THARQ + 3ms + TL1-RSRP_Prediction_Period_CSI-RS

For Aperiodic reporting, the corresponding activation delay is

Tfunctionality_activation_AP = TL1-RSRP_Prediction_Period_CSI-RS

Proposal 4: For semi-persistent reporting, the starting point of functionality activation delay is the reception of the activation command triggered by MAC CE/DCI.

Proposal 5: For semi-persistent reporting, the functionality activation delay is defined as below (For MAC CE activation):

Tfunctionality_activation_SP = THARQ + 3ms + TL1-RSRP_Prediction_Period_CSI-RS

Proposal 6: For semi-persistent reporting, the starting point of functionality activation delay is the reception of the activation command triggered by DCI

Proposal 7: For Aperiodic reporting, the functionality activation delay is defined as below:

Tfunctionality_activation_periodic = TL1-RSRP_Prediction_Period_CSI-RS

Proposal 8: For functionality activation delay, the ending point is defined as the UE's first reporting of inference results as the ending point. Specifically,the definition of the first reporting can be described as the first prediction result report after the UE reports the applicable functionalities via RRCReconfigurationComplete

- Summary

Proposal 1: RAN4 not to introduce deactivation/fall-back delay requirements

reporting the applicable functionalities via RRCReconfigurationComplete in Step 4 when the inference configuration is fully provided in Step 3. FFS on the case that the full inference configuration is not provided in Step 3

Proposal 2: For periodic CSI reporting, the starting point of the functionality activation delay is upon the UE reporting the applicable functionalities via RRCReconfigurationComplete in Step 4 when the inference configuration is fully provided in Step 3. FFS on the case that the full inference configuration is not provided in Step 3

Proposal 3: For periodic CSI reporting, the functionality activation delay is defined as below:

Tfunctionality_activation_periodic = TL1-RSRP_Prediction_Period_CSI-RS

Where the TL1-RSRP_Prediction_Period_CSI-RS is the prediction delay and it is defined as

| Configuration | TL1-RSRP_Prediction_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TCSI-RS+ Tprediction_processing) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TCSI-RS)+ Tprediction_processing) |
| DRX cycle > 320 ms | ceil(M*P*N)*TDRX + Tprediction_processing |
| NOTE 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Tprediction_processing is time for prediction post-processing. NOTE 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. |  |

Proposal 4: For semi-persistent reporting, the starting point of functionality activation delay is the reception of the activation command triggered by MAC CE/DCI.

Proposal 5: For semi-persistent reporting, the functionality activation delay is defined as below (For MAC CE activation):

Tfunctionality_activation_SP = THARQ + 3ms + TL1-RSRP_Prediction_Period_CSI-RS

Proposal 6: For semi-persistent reporting, the starting point of functionality activation delay is the reception of the activation command triggered by DCI

Proposal 7: For Aperiodic reporting, the functionality activation delay is defined as below:

Tfunctionality_activation_periodic = TL1-RSRP_Prediction_Period_CSI-RS

Proposal 8: For functionality activation delay, the ending point is defined as the UE's first reporting of inference results as the ending point. Specifically,the definition of the first reporting can be described as the first prediction result report after the UE reports the applicable functionalities via RRCReconfigurationComplete

- References

- R4-2505107, WF on AI/ML air interface study, Ericsson