3GPP TSG-RAN WG4 Meeting #115	R4-2506474

St Julian’s, Malta, 19th – 23rd May, 2025

Agenda Item:	7.17.3

Source:	Ericsson

Title:	On MMSE-IRC receiver for interference mitigation with 8Rx reception

Document for:	Discussion

# 1	Introduction

Following the objectives of the WID, companies have reached several agreements below with respect to the test scope:

Test scope - RAN4 to introduce PDSCH demodulation requirements for both TDD and FDD - Not to cover PDSCH absolute physical layer throughput requirements for 8Rx - RAN4 to introduce CQI reporting requirements under ICI for 8Rx

After several discussions in previous meetings, the test cases for both inter-cell interference and intra-cell inter-user interference scenario are finalized. In the meantime, it’s also agreed to introduce two sets of requirements to better cover different receiver implementations. In this contribution, we discussed the left open issues with the support of some evaluation results.

# 2	Inter-cell interference

In this section, we discussed the scenario of inter-cell interference cancellation with 8Rx reception.

## 2.1	CQI reporting

It has been agreed to include CQI reporting requirements for inter-cell interference scenario following legacy assumptions from Rel-17, which is to assume fading condition for the target cell and Static channel condition for the interference cell.

In order to determine the CQI receiver coverage, it’s encouraged for companies to bring simulation results for both baseline receiver and simplified receiver to see if there is performance gap. We provided our results[2] below comparing the performance of two different receiver implementations.

Figure 2.1-1 Gain comparison for baseline & simplified receiver

Based on the results above, we observed that there is almost no performance difference between two different receiver types.

Observation 1: There is almost no performance difference between two different receiver types

In this case, we propose to only define one CQI reporting requirement and apply it to both receiver types. Meanwhile, we propose to consider SINR = -2, gamma = 2.1 for the test.

Proposal 1: RAN4 to only define one CQI reporting requirement applying to both receiver types.

Proposal 2: Consider SINR = -2dB, Gamma = 2.1

# 3	Applicability rule and release independent

Test applicability to skip 2/4Rx requirements

Issue 1-5-1: Test applicability to skip 2/4Rx requirements Proposals: - Option 1: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, for both demodulation performance tests and CQI reporting tests, prefer to define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not. - Option 2: For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed. TBD: Clarification of what “corresponding 8Rx requirements” covers.

For UEs supporting both 8Rx and 2/4Rx, in Rel-18, we have already introduced test applicability rules to skip the 2/4Rx requirements if the corresponding 8Rx requirements are passed as shown below. We prefer to apply the same approach for the upcoming PDSCH requirements with 8Rx plus IRC.

Table 5.1.1.2-1: Requirements applicability Supported RX antenna ports; Test type; Test list. UE supports 8Rx, 4Rx and 2Rx; PDSCH; All tests in Clause 5.2.3 (Note 2, 3) All tests in Clause 5.2.4 (Note 2); If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.3 Table 5.2.3.1.1-4, Table 5.2.3.2.1-4 and Test 4-1 in Clause 5.2.3 Table 5.2.3.1.1-6, Table 5.2.3.2.1-6. or; PDCCH; All tests in Clause 5.3.3 (Note 2,3). UE supports both 8Rx and 4Rx; PBCH; All tests in Clause 5.4.3 (Note 1). UE supports both 8Rx and 2Rx; PDSCH; All tests in Clause 5.2.2 (Note 2, 4) All tests in Clause 5.2.4 (Note 2); If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.2 Table 5.2.2.1.1-4, Table 5.2.2.2.1-4. PDCCH; All tests in Clause 5.3.2 (Note 2,4). PBCH; All tests in Clause 5.4.2.

According to the agreed test applicability above, if a UE (supports 8Rx, 4Rx and 2Rx or both 8Rx and 4Rx) passes tests 8Rx requirements, then the following cases will be skipped.

Test 2-1 and Test 2-2 in Clause 5.2.3 Table 5.2.3.1.1-4 and Table 5.2.3.2.1-4:

Table 5.2.3.1.1-4: Minimum performance for Rank 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | R.PDSCH.1-3.1 FDD | 10 / 15 | 64QAM, 0.50 | TDLA30-10 | 2x4, ULA Low | 70 | 13.5 |
| 2-2 | R.PDSCH.2-1.1 FDD | 20 / 30 | 64QAM, 0.50 | TDLA30-10 | 2x4, ULA Low | 70 | 13.7 |

Table 5.2.3.2.1-4: Minimum performance for Rank 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | R.PDSCH.2-3.1 TDD | 40 / 30 | 64QAM, 0.50 | FR1.30-1 | TDLA30-10 | 2x4, ULA Low | 70 | 13.6 |
| 2-2 | R.PDSCH.2-9.1 TDD | 20 / 30 | 64QAM, 0.50 | FR1.30-4 | TDLA30-10 | 2x4, ULA Low | 70 | 13.7 |

Test 4-1 in Clause 5.2.3 Table 5.2.3.1.1-6 and Table 5.2.3.2.1-6:

Table 5.2.3.1.1-6: Minimum performance for Rank 4

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 4-1 | R.PDSCH.1-2.4 FDD | 10 / 15 | 16QAM, 0.48 | TDLA30-10 | 4x4, ULA Low | 70 | 15.6 |
| 4-2 | R.PDSCH.1-12.2 FDD | 10 / 15 | 16QAM, 0.48 | TDLA30-10 | 4x4, ULA Low | 70 | 15.2 |

Table 5.2.3.2.1-6: Minimum performance for Rank 4

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 4-1 | R.PDSCH.2-2.4 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 4x4, ULA Low | 70 | 15.4 |
| 4-2 | R.PDSCH.2-26.2 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 4x4, ULA Low | 70 | 15.5 |

Note that it is not every 4Rx cases will be passed. Following the same approach, it is acceptable for us to define an applicability rule that only skip ‘corresponding’ tests but keep other tests.

Observation 2: Test applicability rules have been defined in Rel-18 to skip the 2/4Rx requirements if the corresponding 8Rx requirements are passed

Proposal 3: For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed.

Release independent

After RAN4 #114bis meeting in April, it’s been agreed that the newly introduced Rel-19 requirements shall be at least release independent from Rel-18. On top of that, there are proposals discussing whether it can be release independent from Rel-17.

Release independency Agreement: - The new Rel-19 requirements shall at least be release independent from Rel-18. Remaining proposals: - Option 1: The new Rel-19 requirements shall also be release independent from Rel-17. - Option 2: The new Rel-19 requirements shall also be release independent from Rel-17. The requirements shall be mandatory for Rel-18 and optional from Rel-17.

Since the 8Rx UE requirements were agreed to be release independent from Rel-17, together with the consideration that newly introduced Rel-19 8Rx requirements are only adding IRC receiver which is a mandatory feature from Rel-17, we support that the upcoming PDSCH and CQI requirements for both inter-cell interference and intra-cell interference scenarios shall be release independent from Rel-17.

Proposal 4: The new requirements for 8Rx shall be release independent from Rel-17.

# 5	Summary

In this contribution, we shared our views and results on left open issues for introducing UE demodulation and CQI reporting requirement. Our proposals are summarized as follows:

Observation 1: There is almost no performance difference between two different receiver types

Proposal 1: RAN4 to only define one CQI reporting requirement applying to both receiver types.

Proposal 2: Consider SINR = -2dB, Gamma = 2.1

Observation 2: Test applicability rules have been defined in Rel-18 to skip the 2/4Rx requirements if the corresponding 8Rx requirements are passed

Proposal 3: For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed.

Proposal 4: The new requirements for 8Rx shall be release independent from Rel-17.

# References

[1]	R4-2504694: Way forward for [114bis][322] NR_demod_Ph5_Part2_UE, Nokia, RAN4 #114bis, Wuhan, China

[2]	R4-2506475: Simulation results for MMSE-IRC receiver for 8Rx reception, RAN4 #115, Malta, MT