3GPP TSG-RAN WG4 Meeting #115	R4-2506478

St Julian’s, Malta, 19th – 23rd May, 2025

Agenda Item:	7.1.4

Source:	Ericsson

Title:	Simulation results for 6Rx UE demodulation

Document for:	Information

# 1	Introduction

In RAN4 #114bis meeting, companies have agreed to introduce new 6Rx UE demodulation requirements which include PDSCH, SDR requirements. In this contribution, we provided our initial simulation results for the purpose of facilitate the test parameter assumptions.

# 2	4 MIMO layer cases

In this section, we mainly evaluate the performance with different candidates for MCS and propagation condition for 4 MIMO layer cases.

## 2.1	Simulation assumption

Table 2.1-1: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Codebook configuration | CodebookType |  | typeI-SinglePanel |
|  | CodebookMode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |
| CSI-RS for tracking | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of HARQ Processes |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |

## 2.2	Simulation results

Table 2.2-1 Evaluation cases

| Test num. | Rank | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | 4 | 10 / 15 | 64QAM, 0.43 Table 1 (MCS =17) | TDLA30-10 | 4x6, ULA medium | 70 | 26 |
| 2-2 | 4 | 10 / 15 | 64QAM, 0.43 Table 1 (MCS =17) | TDLA30-10 | 8x6, ULA medium | 70 | 27 |
| 2-3 | 4 | 10 / 15 | 256QAM, 0.67 Table 2 (MCS =20) | TDLA30-10 | 4x6, ULA medium | 70 | N/A |
| 2-4 | 4 | 10 / 15 | 64QAM, 0.43 Table 1 MCS = 17 | TDLA30-10 | 4x6, ULA medium | 70 | 26 |
| Note: β = 0.00718 for test num. 2-4, β = 0.005154 for other tests. |  |  |  |  |  |  |  |

# 3	6 MIMO layer cases

In this section, we mainly evaluate the performance with different candidates for MCS, DMRS for 6 MIMO layer cases.

## 3.1	Simulation assumption

Table 3.1-1: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 for case 3-1, 3-2 and 3-4 eType 1 for case 3-3 |
|  | Number of additional DMRS |  | 2 for case 3-1, and 3-4 1(eType1) for case 3-3 0 for case 3-2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 2 for case 3-1, 3-2 and 3-4 1(eType1) for case 3-3 |
| Codebook configuration | CodebookType |  | typeI-SinglePanel |
|  | CodebookMode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,1) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |
| CSI-RS for tracking | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of HARQ Processes |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |

## 3.2	Simulation results

Table 3.2-1 Evaluation cases

| Test num. | Rank | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3-1 | 6 | 10 / 15 | 64QAM, 0.43 | TDLA30-10 | 8x6, ULA Low | 70 | 17.7 |
| 3-2 | 6 | 10 / 15 | 64QAM, 0.43 | TDLA30-10 | 8x6, ULA Low | 70 | 17.9 |
| 3-3 | 6 | 10 / 15 | 64QAM, 0.43 | TDLA30-10 | 8x6, ULA Low | 70 | 17.8 |
| 3-4 | 6 | 10 / 15 | 256QAM, 0.67 | TDLA30-10 | 8x6, ULA Low | 70 | N/A |

# 4	Summary

In this contribution, we shared the simulation assumptions and initial results for PDSCH demodulation for 6Rx UE demodulation.

# References

[1]	R4-2504750: Way Forward for [114bis][325] NR_ENDC_RF_Ph4_Demod_6Rx, Huawei, HiSilicon, RAN4 #114bis, Wuhan, China