3GPP TSG-RAN WG4 Meeting #115															 R4-2506480

Malta, 19th - 23th May, 2025

Agenda item:			7.11.7

Source:	MediaTek Inc.

Title:	Discussion on RRM requirements for Ku bands for NR NTN

Document for:	Discussion

# Introduction

In last meeting, whether to specify the Ku band RRM requirements for satellite switching with re-sync was discussed in WF [1]. The agreements are as follows:

Issue 1-6: NR SAN FR2-NTN – NR SAN FR2-NTN Handover (section 6.1C.1.3)

Agreement:

- Reuse the same value of T_sat_beam as in Ka band for Ku band

- For VSAT UEs operating in FR1-NTN numerology, not specify the RRM requirements for TN-NTN mobility, inter-RAT mobility [or satellite switching with re-sync].

In this paper, the RRM requirement for Satellite switching with re-sync for Ku band is further discussed.

# Satellite switching with re-sync for Ku band

The satellite switching delay for FR1-NTN is defined in TS 38.133, clause 6.1C.3.2.2 for hard satellite switch and clause 6.1C.3.2.3 for soft satellite switch. Additionally, in the last meeting, it was agreed to reuse the same value of Tsat_beam  for the Ku band as is used in the Ka band.

To extend the requirement to cover Ku band, the switching time of receiving beam, Tsat_beam, needs to be considered during the satellite switch.

This results in the interruption time for hard satellite switch in the Ku band as follows:

Tinterrupt = Tsat_beam + Tsearch + Tprocessing + T∆ + Tmargin ms

Similarly, the satellite switch delay for soft satellite switch for in the Ku band is given by:

Tsoft_switch = max(t-service - t-serviceStart, Tsat_beam + Tsearch + T∆ + Tmargin) + Tprocessing  ms

Proposal 1: Introduce RRM requirement for satellite switching with re-sync for Ku band.

Proposal 2: Include the switching time of receiving beam, Tsat_beam, in the satellite switching delay for Ku band.

- For hard satellite switch, Tinterrupt = Tsat_beam + Tsearch + Tprocessing + T∆ + Tmargin ms

- For soft satellite switch, Tsoft_switch = max(t-service - t-serviceStart, Tsat_beam + Tsearch + T∆ + Tmargin) + Tprocessing  ms

# Summary

The observations and proposals are summarized as the following:

Proposal 1: Introduce RRM requirement for satellite switching with re-sync for Ku band.

Proposal 2: Include the switching time of receiving beam, Tsat_beam, in the satellite switching delay for Ku band.

- For hard satellite switch, Tinterrupt = Tsearch + Tprocessing + Tsat_beam + T∆ + Tmargin ms

- For soft satellite switch, Tsoft_switch = max(t-service - t-serviceStart, Tsat_beam + Tsearch + T∆ + Tmargin) + Tprocessing  ms

# Reference

- R4-2504897, WF on RRM requirements for NR_NTN_Ku_bands, Apr. 2025.