3GPP TSG-RAN WG4 Meeting #115											R4-2506488

Malta, 19th - 23th May, 2025

Agenda item:			7.33.6

Source:	MediaTek Inc.

Title:	Discussion on RRM requirements for IoT-NTN TDD mode

WID/SID:        IoT_NTN_TDD-Core

Document for:	Discussion

# 1  Introduction

## Issue 0: General aspects

Agreement (online):

For TDD NB-IoT in NTN, discuss whether to define the RRM requirements for:

- Earth moving and fixed cell, normal coverage with SNR ≥ -1dB

In RAN4#114bis, it was agreed that the side condition with SNR ≥ -1dB [1], and the corresponding RRM requirements are still under discussion. Our perspective on this matter is detailed in this paper.

# 2  RRM requirements for NTN TDD

## Issue 1: Cell reselection (4.6A.2)

Our understanding of the legacy requirement is that the UE is not required to perform more than one measurement per DRX cycle. Therefore, the existing values for Tdetect, Tmeasure, and Tevaluate in Table 4.6A.2.2-1 and Table 4.6A.2.2-2 can be applicable.

Proposal 1: The existing values for Tdetect, Tmeasure, and Tevaluate in Table 4.6A.2.2-1 and Table 4.6A.2.2-2 can be applicable.

## Issue 2: Cell measurement in connected mode (8.14A)

As a minimum requirement, this should be defined solely based on NRS and the side condition of SNR>-1dB is applicable.

Therefore, by following the conclusion of cell reselection, the existing values for Tmeasure can be applicable.

Proposal 2: For Cell measurement in connected mode, the existing values for Tmeasure can be applicable.

## Issue 5: RLM (7.23A)

This requirement is based on 2%/10% BLER, so the side condition of SNR>-1dB may not be applicable. A high SNR level could result in frequent link failures.

Therefore, RAN4 needs to decide whether to reuse the exiting requirement with relaxed the accuracy in the RLM test case or scale up the delay requirement.

Proposal 3: For RLM, RAN4 needs to choose one of the following options:

  - Option 1: Reuse the existing requirement with the relaxed accuracy in the RLM test case

  - Option 2: Scale it by a factor of [3] of the following options:AN4 needs to choose one of the following options:

# Summary

The observations and proposals are summarized as the following:

Proposal 1: The existing values for Tdetect, Tmeasure, and Tevaluate in Table 4.6A.2.2-1 and Table 4.6A.2.2-2 can be applicable.

Proposal 2: For Cell measurement in connected mode, the existing values for Tmeasure can be applicable.

Proposal 3: For RLM, RAN4 needs to choose one of the following options:

  - Option 1: Reuse the existing requirement with the relaxed accuracy in the RLM test case

  - Option 2: Scale it by a factor of [3] of

# References

- R4-2504915, WF on RRM requirements for IoT_NTN_TDD, Moderator (Qualcomm Incorporated), R4#114bis, Apr. 2025.