3GPP TSG-RAN WG4 Meeting # 115	                                                      R4-2506507

Malta, MT, May 19 – 23, 2025

Agenda item:	7.23.3.1

Source:	MediaTek Inc.

Title:	Discussion on RAN4 impacts for RRM measurement prediction

Document for:	Discussion

- Introduction

RAN#105 plenary has agreed the updated version of the SID with the following RAN4 objectives [1]:

According to the agreed work plan last meeting, this meeting RAN4 will focus on RRM measurement prediction and event prediction use-cases. In this contribution, we would like to discuss the related RRM requirements of RRM measurement prediction.

- Discussion

In RAN4#112bis, RAN4 agreed to start the discussion on scenario 2, 3, 4. In the next, we will focus on the three scenarios.

| scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |

2.1 Metrics used to evaluate the model performance

For AI/ML BM, there were a lot of discussions on the metrics used to define the model performance. Similar as AI/ML BM, the metrics should be discussed for AI/ML mobility too.

RAN4#112bis Issue 2-1-1: Performance Metrics/KPIs for Measurement Prediction use case Agreement: - RAN4 to first study the impacts on requirements for the following case: - L3 cell level RSRP (Point C) prediction - FFS L1 beam-level (Point A1) - FFS L3 beam-level (Point E) - Other cases can be considered based on RAN4 further discussion and RAN2 further progress. RAN4#114bis Issue 2-1-6: Impacts on beam/cell level measurements Agreement: - For Rel. 19 study: - RAN4 to not consider L1 beam-level (Point A1) measurement prediction.

In our view, one of the principles to choose the metrics is that UE will report corresponding results or the metric can be derived through the reported results. For cell-level RRM measurement prediction, no matter which sub-use case is adopted, the final reported results will be L3 cell level RSRP (Point C). So we propose to use cell level RSRP (Point C) as the performance metric for all the three sub-use cases.

For information Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then cell-level measurement result is generated Sub-use case 2: Cell-level measurement result(s) is predicted based on actual cell-level measurement result(s) Sub-use case 3: Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) Editor Note 1: The RRM measurement refers to L3 cell/beam level measurement and/or L1 beam level measurement

Proposal 1: For cell-level RRM measurement prediction, only use cell level RSRP (Point C) as the performance metric for all the three sub-use cases.

Last meeting, RAN4 also discussed the factors that may impact RSRP prediction accuracy for inter-frequency prediction scenario. One of the factors is the correlation coefficient between measured and predicted frequency layers.

Issue 2-1-8: Inter-frequency prediction scenario Agreement: - RAN4 to study the factors potentially impacting RSRP prediction accuracy for inter-frequency prediction scenario considering at least the following aspects: - the side condition of frequency prediction (e.g., EPRE difference) - the impact of correlation coefficient between measured and predicted frequency layers - impact of cluster approach, e.g., - When measurement from single cell in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency. - When measurement from a group of cells in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency.

After internal checking, it turns out that the correlation coefficient factor is not used in generating the channels of measured frequency layer or predicted frequency layer. It is used to explain why some companies’ simulation results show more gain, but others’ show less. In other words, it is not possible to set a fixed correlation coefficient factor and then generate the channels of measured frequency layer or predicted frequency layer fulfil the correlation coefficient. We don’t think it is feasible to study the impact of correlation coefficient unless RAN4 define a common dataset.

Observation 1: For scenario 3 (inter-frequency measurement prediction), it is not possible set a fixed correlation coefficient factor and then generating the channels of measured frequency layer or predicted frequency layer fulfil the correlation coefficient.

Proposal 2: For inter-frequency prediction scenario, it is not feasible to study the impact of correlation coefficient unless RAN4 define a common dataset.

2.2 Ground truth

In the past meetings, RAN4 reached the following progress on how to determine the ground truth. In the next, we would like to analyze more.

RAN4#114 Issue 2-1-2: Ground Truth Definition for RSRP accuracy Agreement: Take Option 1-1-FR2 as baseline, and further discuss any additional aspects need to be considered. - Option 1-1-FR2: The ground truth of L3 RPRP is the reported L3 RSRP measurement under sufficient high SNR in FR2. Issue 2-1-2: Ground Truth Definition for RSRP accuracy - Agreement: - Alt 1: Ground truth for FR1 will be based on the transmitted or reception power - Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions - Further discuss the advantages and disadvantages for Alt 1 and Alt 2 considering the aspects such as: - The information at the TE side, on the timing UE performs and/or finishes the measurement and/or prediction - Impact of L3 filtering - Number of samples for L1 filtering - Timing window, if it exists, where UE can’t measure the ground truth in Alt2 - channel condition - Avoid UE cheat in the test - S(I)NR variations in the test at different time instances (for Alt2) - Other aspects are not precluded - Companies are encouraged to provide analysis on all or some of the aspects in the list RAN4#114bis Issue 2-1-5: Ground truth definition for RSRP Accuracy for FR1 Agreement: During the SI, RAN4 to provide analysis on potential issue and possible solution to the two Alternatives. If RAN4 cannot make decision during the SI phase, RAN4 will discuss in the WI phase. - At least capture the definition of the two Alternatives in the TR.

The fact is that L3 measurement works well in the real field. In the real field, suppose there are several UEs at the same location. No matter how the UEs perform L3 measurement, no matter how they do L1 filtering and L3 filtering, the reported measurement results at the same reporting occasion should be similar. There will be some difference, but the divergence should not be too large.

Then back to the test, under the same channel realization, the reported RSRP value at the same reporting occasion of different UEs shall be similar too. There will be some difference, but the divergence should not be too large.

We totally understand other companies’ concern on using the transmission power at TE to derive the ground truth. After checking the simulation results provided by Xiaomi and QC last meeting, we also question the feasibility. As commented by other company, L3 filtering is kind of up to UE implementation. In our implementation, we will adjust L3 filtering based on NW configuration and the real measurement interval. We checked the impact on the ground truth under different measurement intervals and shifted start positions in Fig.1. The figure shows CDF of the difference between filtered L3-RSRP with different measurement intervals & shifted start positions and filtered L3-RSRP with an interval of 40ms & 0ms shifted start position. In the simulation, we use the same dataset used by RAN2 for scenario 2 (FR1 to FR1 intra-frequency temporal domain case B). Sliding window is adopted. It can be observed that the RSRP difference in the range of [5%,95%] is less than ±2dB.   We think it is acceptable and it also justifies why the whole L3 measurement mechanism works well in the real field allowing proprietary filtering.

Fig. 1

We understand that different L1 filtering and L3 filtering may be adopted by UE vendors. It may be necessary to further check whether it is workable to use the transmission power at TE to derive the ground truth. In our understanding, the simulations shall at least include the following:

- Similar simulations as shown in Xiaomi and QC’s contributions last meeting

  - The genie aided measurement results considering different L1 filtering and L3 filtering.

- The margin to add when using different L1 filtering and L3 filtering rather than a defined L1 filtering and L3 filtering.

  - It also requires simulating the performance of the model which is supposed to be done in WI phase.

As part of the simulations are supposed to be done in WI phase, we suggest not to do down-selection at SI phase.

Another alternative is to use the reported results as ground truth. If so, the test will contain two steps. In one step, TE will not transmit SSB on some occasions/frequency layers and UE will predict the measurement results at those occasions/frequency layers. In another step, TE will transmit SSB on all the occasions/frequency layers. Even it is possible to make the channel realization in the two steps the same, UE measurement behaviour will not be exactly the same. There was proposal that the ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X. We think it makes sense. In our view, it is the instantaneous fluctuation at different occasions due to fast fading which results to varied value of ground truth corresponding to different L1 filtering and L3 filtering.  Even to use the reported results as ground truth, the uncertainty introduced by “Yms” shall be considered too. So no matter which alternative finally adopted, the impact of the instantaneous fluctuation at different occasions due to fast fading shall be considered.

Proposal 3: During the SI, RAN4 not to do down-selection on the two Alternatives of ground truth definition. No matter which alternative finally adopted, the impact of the instantaneous fluctuation at different occasions due to fast fading shall be considered.

2.3 Cell identification

It is observed that there were some proposals to study the impacts on cell identification for L3 cell-level measurement prediction by AI/ML or whether the predicted cell is known or unknown. We agree that this is an important issue for further study. Due to cell identification related discussion is usually conducted in RAN4, RAN2 may not realize the issue yet and it is more appropriate to discuss the issue in RAN4.

For scenario 2 and 4, since they are intra-cell prediction, UE is supposed to detect and measure the cell before prediction. So cell identification is not an issue.

For Scenario 3, FR1 to FR1 inter-frequency (frequency domain), since measurement on target cell is not always conducted, RAN4 should discuss the impact on cell identification. As mentioned above, RAN2 will focus on frequency domain for cluster based approach. Here we also focus on cluster-based approach. As shown in the example in Fig. 2, the inputs are RSRP of 21 cells (assume Cell#0,1, …,20) on frequency#1, the outputs are RSRP of 21 cells (assume Cell#0’,1’, …,20’) on frequency#2. Cell#X and Cell#X’ are co-located.

Fig. 2

Though cell detection, UE can get Cell ID and rough timing. In collocated scenario as shown in Fig. 2, it is possible to inform UE cell ID of Cell#X’ which is co-located with Cell#X. But if extending Scenario#3 to non-collocated cases, we are not sure whether it is workable to inform UE cell ID of the neighbor cell(s) by serving cell.

Observation 2: For scenario 3 (inter-frequency measurement prediction), it is possible to inform UE cell ID of the inter-frequency neighbor cell(s) to be predicted for collocated cases while may be not possible for non-collocated cases.

Regarding timing information, it is not possible to get from serving cell unless collocated cells are well synchronized. So in scenario 3, the cells predicted but not measured recently are unknown.

Proposal 4: For scenario 3 (inter-frequency measurement prediction), the cells predicted but not measured recently are unknown.

In AI/ML BM, there is similar discussion on whether the predicted TCI state is known or unknown. And the final conclusion will have impact on TCI state switching procedures but would have little impact on model monitoring. But for AI mobility, whether the predicted cell is known or not would also have some impact on model monitoring, as UE is supposed to measure the predicted cells during model monitoring. For the cells which have been detected recently, e.g., in 5s, UE can measure the cell directly without cell search. But if the cell has not been detected yet, UE should perform PSS/SSS detection at first and then L3 measurement. So, if model monitoring is performed frequently, the gain will become less. But if UE performs model monitoring every several seconds. The time needed to finish one round of monitoring will be long.

We think it is important to study when UE performs cell detection on the predicted cells as this would have impact on model monitoring.

Proposal 5: Study when UE performs cell detection on the predicted cells, e.g., before model monitoring or during model monitoring.

2.4 Measurement error model

As guided by the progress last meeting, we would like to discuss how to add measurement error for AI mobility in FR1.

Issue 2-1-7: Impacts of measurement error on prediction accuracy WF: Companies are encouraged to bring proposals on how to capture factors to be considered for error models to consider the impact of measurement errors on prediction accuracy.

Although it is L3 measurement considered in AI mobility, UE is supposed to measure each sample and derive L3 measurement result after L1 filtering and L3 filtering. For each measurement occasion, BB error and RF error will be introduced independently. After L1 filtering, BB error will be further flattened. But from the point of adding error, BB error and RF error shall be added on each measurement occasion independently. In other words, when derive BB error distribution, one shot measurement is assumed.

The way to add measurement error is shown in Fig. 3:

- In SLS simulation, SINR of each Tx beam is calculated dependently.

  - In the calculation of SINR, noise is generated as Gaussian distribution and UE receiver noise figure is set to 9dB which is widely used in FR1. Useful signal and interferences from other cells are calculated considering Tx power, path-loss, channel gain and beamforming gains.

- After calculating SINR of each beam, we look up the table to get the corresponding variance.

  - If the SINR is not an integer or out of the range [-24:3:9] dB, we use interpolation method to obtain the mean and variance.

- A random BB error is generated following Gaussian distribution with the mean and variance corresponding to the SINR value.

- RF error is modelled as Gaussian distribution under ±3 dB RF error (μ=0,  σ2=3). A random RF error is generated for all the beams of the same frequency layer per SMTC.

Fig. 3

Proposal 6: For AI mobility, in FR1, measurement error should be added as:

- BB error:

  - Added independently per Tx beam per cell per measurement occasion according to the corresponding SINR.

  - The distribution of BB error under certain SINR is obtained through LLS assuming one shot measurement.

- RF error:

  - Modelled as Gaussian distribution under ±3 dB RF error (μ=0,  σ2=3).

  - A random RF error is generated for all the beams of the same frequency layer per SMTC.

2.5 Other RRM impacts

UE measurement and/or report behavior will be different from legacy for all the scenarios 2,3,4. RAN4 shall study whether there is any impact on measurement delay and report delay requirements. Regarding the impact on report requirements, there will be similar issue for AI/ML BM. To avoid duplicated discussion, we prefer waiting for more progress in AI/ML BM.  As this is the last second meeting and the progress in AI/ML BM is not good enough, we think it is better not to discuss the impact on measurement report delay requirements for AI mobility in SI. If there will be WI, RAN4 can further discuss how to define measurement report delay requirements for AI mobility.

Proposal 7: Regarding measurement report delay requirements for AI mobility, leverage the conclusion on AI/ML BM as much as possible.  Not to discuss the impact on measurement report delay requirements for AI mobility in SI to avoid duplicated discussions.

Next, we will focus on the impact on measurement delay requirements.

Scenario 2, FR1 to FR1 intra-frequency intra-cell temporal domain case B prediction, is to predict a subset of measurement instances in temporal domain of the same cell. This scenario is similar as BM case 2. The impact on measurement delay would take BM case 2 as a basis.

Proposal 8: Regarding the impact on measurement delay requirements for AI mobility Scenario 2, leverage the conclusion on AI/ML BM case 2 as much as possible. Not to discuss the impact on measurement delay requirements for AI mobility Scenario 2 in SI to avoid duplicated discussions.

Scenario 3, FR1 to FR1 inter-frequency inter-cell (frequency domain), in our understanding, this scenario includes “prediction on an inter-frequency layer based on measurement on an intra-frequency layer” and also “prediction on an inter-frequency layer based on measurement on a different inter-frequency layer”. As discussed in previous part, the scenario is more valuable to predict the measurement on multiple frequency layers. In our understanding, MG still needs to be configured at least for model monitoring. If there are more than one inter-frequency layers involved, RAN4 needs to discuss the gap sharing mechanism among different inter-frequency layers both for measurement and monitoring. There would be some impact on measurement delay requirements.

Proposal 9: For Scenario 3, gap sharing mechanism among different inter-frequency layers should be considered for both for measurement and monitoring when more than one inter-frequency layers are involved.

Scenario 4, FR2 to FR2 intra-frequency intra-cell temporal domain case A, is to predict measurement result(s) in prediction window based on actual measurement result(s) in observation window of the same cell without reducing any measurement in observation window. There is no impact on UE measurement behavior, so there is no impact on the measurement delay.

Proposal 10: Scenario 4 has no impact on measurement delay requirements.

- Conclusion

In this contribution, we provide our views for RRM measurement prediction. Proposals and observations are summarized as follows.

Proposal 1: For cell-level RRM measurement prediction, only use cell level RSRP (Point C) as the performance metric for all the three sub-use cases.

Observation 1: For scenario 3 (inter-frequency measurement prediction), it is not possible set a fixed correlation coefficient factor and then generating the channels of measured frequency layer or predicted frequency layer fulfil the correlation coefficient.

Proposal 2: For inter-frequency prediction scenario, it is not feasible to study the impact of correlation coefficient unless RAN4 define a common dataset.

Proposal 3: During the SI, RAN4 not to do down-selection on the two Alternatives of ground truth definition. No matter which alternative finally adopted, the impact of the instantaneous fluctuation at different occasions due to fast fading shall be considered.

Observation 2: For scenario 3 (inter-frequency measurement prediction), it is possible to inform UE cell ID of the inter-frequency neighbor cell(s) to be predicted for collocated cases while may be not possible for non-collocated cases.

Proposal 4: For scenario 3 (inter-frequency measurement prediction), the cells predicted but not measured recently are unknown.

Proposal 5: Study when UE performs cell detection on the predicted cells, e.g., before model monitoring or during model monitoring.

Proposal 6: For AI mobility, in FR1, measurement error should be added as:

- BB error:

  - Added independently per Tx beam per cell per measurement occasion according to the corresponding SINR.

  - The distribution of BB error under certain SINR is obtained through LLS assuming one shot measurement.

- RF error:

  - Modelled as Gaussian distribution under ±3 dB RF error (μ=0,  σ2=3).

  - A random RF error is generated for all the beams of the same frequency layer per SMTC.

Proposal 7: Regarding measurement report delay requirements for AI mobility, leverage the conclusion on AI/ML BM as much as possible.  Not to discuss the impact on measurement report delay requirements for AI mobility in SI to avoid duplicated discussions.

Proposal 8: Regarding the impact on measurement delay requirements for AI mobility Scenario 2, leverage the conclusion on AI/ML BM case 2 as much as possible. Not to discuss the impact on measurement delay requirements for AI mobility Scenario 2 in SI to avoid duplicated discussions.

Proposal 9: For Scenario 3, gap sharing mechanism among different inter-frequency layers should be considered for both for measurement and monitoring when more than one inter-frequency layers are involved.

Proposal 10: Scenario 4 has no impact on measurement delay requirements.

- Reference

- RP-242393, Revised SID on AIML for mobility in NR, OPPO (rapporteur), RAN#105, Melbourne, Australia, September 9-12, 2024.