3GPP TSG-RAN WG4 Meeting # 115	                                        R4-2506510

Malta, MT, May. 19 – May. 23, 2025

Agenda Item: 5.19.1

Source: MediaTek Inc.

Title: Discussion on R18 mobility

Document for: Discussion

- Introduction

Although R18 mobility has been closed, there is still some part for further clarification. We will discuss the related issues in this contribution.

- Discussion

In RAN4#114, how to handle the collision between SSB and a valid RO was discussed. The definition of RACH occasion colliding with the MG occasion still needs further discussion.

Issue 1-2-1: whether/how to handle the collision between an SSB and a valid RO for LTM Agreement on reply LS - RACH shall be prioritized over SSB, if the collision is outside MG. - If collision happens within MG, it is up to UE implementation, i.e., UE may transmit or drop RACH when the RACH occasion colliding with the MG occasion. < Agreement > - Further discuss the definition of RACH occasion colliding with the MG occasion.

In our understanding, if MG overlaps with RACH occasion and/or RF retuning time (if needed) to target cell and/or RF retuning time back to serving cell, it can be viewed as RACH occasion colliding with the MG occasion.

Fig. 1

Proposal 1: For PDCCH order RACH in LTM, RACH occasion colliding with the MG occasion can be defined as:

- MG overlaps with RACH occasion and/or RF retuning time to target cell and/or RF retuning time back to serving cell.

- Summary

In this paper, we provide some views on the open issues of R18 L1/L2 triggered mobility. We have the following proposals:

Proposal 1: For PDCCH order RACH in LTM, RACH occasion colliding with the MG occasion can be defined as:

- MG overlaps with RACH occasion and/or RF retuning time to target cell and/or RF retuning time back to serving cell.