3GPP TSG-RAN WG4 Meeting #115	R4-2506521

Malta 19th – 23rd May 2025

Source: 	Nokia

Title: 					n252 A-MPR simulations and proposal

Agenda Item: 			6.13.1

Document for:	Approval

# 1	Introduction

We provide A-MPR simulations results for n252 PC3 NS_07N. We also provide an A-MPR proposal based on our results and A-MPR regions from earlier proposals [1, 2].

# 2	Simulation assumptions and scenario

## 2.1	Simulation parameters

- Operating band n252 (UL 2000 MHz – 2020 MHz)

- 10 MHz, 15 MHz, and 20 MHz CBWs

- Power class PC3

- WOLA processing

- I/Q image -28 dBc

- Carrier leakage -28 dBc

- CIM3 -60 dBc

- Only additional spurious emission mask from NS_07N is simulated. Other gating factors (ACLR, SEM, etc.) are excluded because they are handled by MPR.

## 2.2	NS_07N mask

Table 6.5.3.3.6-1: Additional out-of-band requirements for "NS_07N"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz, 20MHz |  |  |
| 1559 ≤ f < 1605 | -50 | 700 Hz | Averaged over any 2 millisecond active transmission interval |
| 1559 ≤ f < 1605 | -40 | 1MHz |  |
| 1930 ≤ f ≤1995 | -40 | 1MHz |  |

We only consider the 1930 MHz ≤ f ≤ 1995 MHz segment because the other segments are very far from the channel.

# 3	Simulation results

We provide complete sets of the A-MPR triangles from the simulations at the relevant center frequencies in the Appendix. In this section we present some highlights of the results.

In Figs. 1 and 2 we present examples of the A-MPR triangles for 10 MHz and 20 MHz CBWs, respectively. The maximum A-MPR at 20 MHz CBW is close to the average between the two companies that have already provided results.

Fc = 2010 MHz

Fc = 2005 MHz

Figure 1: Examples of the simulated A-MPR to satisfy the NS_07N additional spurious emission limit for 10 MHz channel at Fc = 2005 MHz (upper row) and Fc = 2010 MHz (bottom row).

Figure 2: Examples of the simulated A-MPR to satisfy the NS_07N additional spurious emission limit for 20 MHz channel at Fc = 2010 MHz.

# 4	A-MPR proposal

We use the previous proposals [1, 2] as a basis, and we have mainly focused on the regions where they differ from each other. We highlight the cells that differ to either of the two proposals with yellow color. Because in our simulations the maximum A-MPR values at 10 MHz do not differ much from the corresponding ones at higher CBWs, we have changed the A2 region at 10 MHz to A3 to better optimize the remaining A2 values. If the region at 10 MHz CBW is kept as A2, then our A2 values need to be increased (see Fig. 1 and Appendix).

Proposal 1: RAN4 shall consider the tables 1 and 2 below for defining the A-MPR for n252 PC3 NS_07N.

Table 1: A-MPR regions for -40 dBm/MHz

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 10MHz | < 2010 | ≤ 1.44 | ≥ 0 | A3 |
|  |  | > 1.44 | ≥ 3.6 | A1 |
|  |  | > 7.74 | ≤ 0.72 | A1 |
|  | 2010 ≤ Fc ≤ 2015 | ≥ 0 | ≥ 4.5 | A1 |
| 15MHz | < 2012.5 | ≤ 3.06 | ≥ 0 | A3 |
|  |  | > 3.06 | ≥ 4.5, ≤ 7.2 | A1 |
|  |  | > 3.06 | > 7.2 | A2 |
|  |  | > 10.98 | ≤ 0.72 | A1 |
|  | 2012.5 | ≤ 1.44 | ≥ 0 | A2 |
|  |  | > 1.44 | ≥ 6.48 | A1 |
|  |  | > 12.6 | ≤ 0.72 | A1 |
| 20 MHz | 2010 | ≤ 4.68 | ≥ 0 | A3 |
|  |  | > 4.68 | ≥ 4.5, ≤ 9 | A1 |
|  |  | > 4.68 | > 9 | A2 |
|  |  | > 12.6 | ≤ 0.72 | A1 |

Table 2: A-MPR for -40 dBm/MHz

| Modulation/Waveform | A1 | A2 | A3 |
| --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 4 | ≤ 8 | ≤ 10.5 |
| DFT-s-OFDM QPSK | ≤ 4.5 | ≤ 8 | ≤ 11 |
| DFT-s-OFDM 16 QAM | ≤ 5 | ≤ 8 | ≤ 11.5 |
| DFT-s-OFDM 64 QAM | ≤ 5.5 | ≤ 8 | ≤ 11.5 |
| DFT-s-OFDM 256 QAM | ≤ 5.5 | ≤ 8 | ≤ 11.5 |
| CP-OFDM QPSK | ≤ 7 | ≤ 8.5 | ≤ 12.5 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 8.5 | ≤ 12.5 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 8.5 | ≤ 12.5 |
| CP-OFDM 256 QAM | ≤ 7 | ≤ 8.5 | ≤ 12.5 |

# 5	Conclusions

We have simulated A-MPR for n252 PC3 NS_07N. We have made the following proposal based on our results and A-MPR regions from earlier proposals [1, 2].

Proposal 1: RAN4 shall consider the tables 1 and 2 below for defining the A-MPR for n252 PC3 NS_07N.

Table 1: A-MPR regions for -40 dBm/MHz

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 10MHz | < 2010 | ≤ 1.44 | ≥ 0 | A3 |
|  |  | > 1.44 | ≥ 3.6 | A1 |
|  |  | > 7.74 | ≤ 0.72 | A1 |
|  | 2010 ≤ Fc ≤ 2015 | ≥ 0 | ≥ 4.5 | A1 |
| 15MHz | < 2012.5 | ≤ 3.06 | ≥ 0 | A3 |
|  |  | > 3.06 | ≥ 4.5, ≤ 7.2 | A1 |
|  |  | > 3.06 | > 7.2 | A2 |
|  |  | > 10.98 | ≤ 0.72 | A1 |
|  | 2012.5 | ≤ 1.44 | ≥ 0 | A2 |
|  |  | > 1.44 | ≥ 6.48 | A1 |
|  |  | > 12.6 | ≤ 0.72 | A1 |
| 20 MHz | 2010 | ≤ 4.68 | ≥ 0 | A3 |
|  |  | > 4.68 | ≥ 4.5, ≤ 9 | A1 |
|  |  | > 4.68 | > 9 | A2 |
|  |  | > 12.6 | ≤ 0.72 | A1 |

Table 2: A-MPR for -40 dBm/MHz

| Modulation/Waveform | A1 | A2 | A3 |
| --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 4 | ≤ 8 | ≤ 10.5 |
| DFT-s-OFDM QPSK | ≤ 4.5 | ≤ 8 | ≤ 11 |
| DFT-s-OFDM 16 QAM | ≤ 5 | ≤ 8 | ≤ 11.5 |
| DFT-s-OFDM 64 QAM | ≤ 5.5 | ≤ 8 | ≤ 11.5 |
| DFT-s-OFDM 256 QAM | ≤ 5.5 | ≤ 8 | ≤ 11.5 |
| CP-OFDM QPSK | ≤ 7 | ≤ 8.5 | ≤ 12.5 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 8.5 | ≤ 12.5 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 8.5 | ≤ 12.5 |
| CP-OFDM 256 QAM | ≤ 7 | ≤ 8.5 | ≤ 12.5 |

# 6	References

[1] R4-2504544, A-MPR and OOB blocking for n252, Qualcomm Inc.

[2] R4-2505125, Introduction of the NTN S-band to TS 38.101-5, Apple

# 7	Appendix: A-MPR triangles