3GPP TSG-RAN WG4 Meeting #115																		R4-2506523

Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.21.3

Source: 	OPPO

Title: 	CSI reporting requirement and testing framework for CSI prediction

Document for:	Approval

- Discussion

In RAN4#114bis meeting, following agreements have been made,

Codebook to be used in the tests

In Rel-19 only PMI requirements based on AI/ML CSI prediction will be defined

Use Rel-18 eType II Doppler codebook for AI/ML based CSI predictions

Requirement baseline

Requirement to be introduced for Dopler domain basis N4=1. N4>1 can be further discussed in the future if needed

Test metric

Requirements to be defined based on the ratio (same metric as for the current PMI reporting requirements in 38.101-4)

- tue_AI/ML is 90 % of the maximum throughput obtained at SNRue_AI/MLusing the precoders configured according to the UE reports,

- trnd is the throughput measured at SNRue_AI/ML with random precoding.

  - trnd is obtained with Rel-15 type I single panel codebook

- Note: SNRue_AI/ML is calculated in the same way as SNRue and SNRfollow1,follow2 in section 6.3 of 38.101-4

For γAI/ML requirement, in terms of the relationship to legacy requirements, what we define in RAN4 is the minimum performance requirement, which is not to verify how much performance gain an AI/ML function/model can bring. The assessment of performance gain should be considered when RAN1 introduces and evaluates CSI features, rather than when RAN4 defines test cases. Furthermore, under current test conditions, e.g., by TDL channel, even if we define a higher performance testing requirement, it still has little significance for the practical application of AI/ML solutions. Therefore, we can reuse the legacy PMI requirement (γ value) as a baseline test and let other options/proposals with higher performance requirements be further studied.

Proposal 1: Reuse the legacy PMI requirement (γ value) as a baseline test, and let other options/proposals with higher performance requirements be further studied.

For  γAI/ML calculation, it could be obtained within a CSI prediction window. If a CSI prediction window contains more than one CSI prediction instance, γAI/ML could be (1) calculated for each predicted CSI and then do the average within the CSI prediction window, or (2)  tue_AI/ML and trnd could be calculated within the CSI prediction window first and then get the averaged γAI/ML value.

Proposal 2: γAI/ML could be obtained within a CSI prediction window. If a CSI prediction window contains more than one CSI prediction instance,

Option1: γAI/ML is calculated for each predicted CSI and then do the average within the CSI prediction window

Option2: tue_AI/ML and trnd are calculated within the CSI prediction window first and then get the averaged γAI/ML value.

Figure 1 Reference block diagram for 1-sided model

Figure 1 provides the reference block diagram for 1-sided model as agreed previously. For CSI prediction, following steps could be considered:

- Step1: TE sends CSI-RS to DUT, within a CSI observation window (signal generating and transmitting)

- Step2: DUT conducts the CSI measurement within the CSI observation window (CSI-RS receiving)

- Step3: DUT conducts the CSI prediction within a CSI prediction window (Inference)

- Step4: DUT reports the predicted CSI (CSI reporting)

- Step5: TE calculates the γAI/ML for performance [and calculates the measurement/inference latency for LCM]

Proposal 3: For CSI prediction, following test steps could be considered:

- Step1: TE sends CSI-RS to DUT, within a CSI observation window (signal generating and transmitting)

- Step2: DUT conducts the CSI measurement within the CSI observation window (CSI-RS receiving)

- Step3: DUT conducts the CSI prediction within a CSI prediction window (Inference)

- Step4: DUT reports the predicted CSI (CSI reporting)

- Step5: TE calculates the γAI/ML for performance [and calculates the measurement/inference latency for LCM]

- Conclusions

In this contribution, we have discussed the CSI reporting requirement and testing framework for CSI prediction, and got the following proposals:

Proposal 1: Reuse the legacy PMI requirement (γ value) as a baseline test, and let other options/proposals with higher performance requirements be further studied.

Proposal 2: γAI/ML could be obtained within a CSI prediction window. If a CSI prediction window contains more than one CSI prediction instance,

Option1: γAI/ML is calculated for each predicted CSI and then do the average within the CSI prediction window

Option2: tue_AI/ML and trnd are calculated within the CSI prediction window first and then get the averaged γAI/ML value.

Proposal 3: For CSI prediction, following test steps could be considered:

- Step1: TE sends CSI-RS to DUT, within a CSI observation window (signal generating and transmitting)

- Step2: DUT conducts the CSI measurement within the CSI observation window (CSI-RS receiving)

- Step3: DUT conducts the CSI prediction within a CSI prediction window (Inference)

- Step4: DUT reports the predicted CSI (CSI reporting)

- Step5: TE calculates the γAI/ML for performance [and calculates the measurement/inference latency for LCM]

- References

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-250792, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface