3GPP TSG-RAN WG4 Meeting #115																		R4-2506527

Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.21.7

Source: 	OPPO

Title: 	Demodulation and CSI reporting requirements for CSI prediction

Document for:	Approval

- Discussion

Regarding the reference model for CSI prediction, the following agreements were reached during the RAN4#114 meeting:

Reference model for CSI prediction

Agreement:

Assume no reference AI model for CSI prediction as baseline

- If it is difficult to align the results corresponding to test metric, RAN4 will discuss the reference model.

Besides, the "Way Forward on Updated simulation assumptions for AIML CSI prediction " was agreed in RAN4 #114bis to proceed with CSI evaluation, including the following steps:

Step-1: to check the accuracy of AI/ML output CSI

For step-1, interested companies are encouraged to provide the 2nd round of CSI prediction simulation results in RAN4#115 meeting.

Step-2: to check SGCS after CSI feedback

Simulation assumptions for step-2 are updated in RAN4#114bis meeting.

Interested companies are also encouraged to provide the 1st round of CSI prediction simulation results (SGCS as KPI, SGCS between the eigenvectors of predicted PMI derived from the ideal DL channel estimation and ground-truth eigenvectors), in RAN4#115 meeting.

Step-3: to check the TPs results, practical DL channel estimation

CSI prediction simulation (TPs as KPI, with practical DL channel estimation) is postponed to RAN4#116 meeting.

According to our understanding, RAN4 needs to first assess the CSI prediction evaluation results across different companies before determining whether a reference model is necessary.

    - If RAN4 achieves aligned CSI prediction evaluation results in step-2 (as described in the agreed simulation assumptions), further evaluation (step-3 comparisons) can proceed (after RAN4#115).

    - If alignment in step-2 proves difficult, RAN4 should identify the key factors causing discrepancies and may need to initiate a second-round evaluation to reassess Step-2 alignment (after RAN4#115).

    - If alignment remains unachievable after step-3, RAN4 may then trigger discussions and evaluations on a reference model for CSI prediction (e.g., during/after RAN4#116).

    - If alignment achieves after step-3, RAN4 will trigger evaluations on the CSI reporting requirements for CSI prediction(e.g., during/after RAN4#116).

Proposal 1: Regarding the RAN4 Simulation for CSI prediction and the Reference model for CSI prediction, RAN4 should first assess the CSI prediction evaluation results across different companies before determining whether a reference model is necessary.

Proposal 2: Regarding the RAN4 Simulation for CSI prediction and the Reference model for CSI prediction,

    - If RAN4 achieves aligned CSI prediction evaluation results in step-2 (as described in the agreed simulation assumptions), further evaluation (step-3 comparisons) can proceed (after RAN4#115).

    - If alignment in step-2 proves difficult, RAN4 should identify the key factors causing discrepancies and may need to initiate a second-round evaluation to reassess Step-2 alignment (after RAN4#115).

    - If alignment remains unachievable after step-3, RAN4 may then trigger discussions and evaluations on a reference model for CSI prediction (during/after RAN4#116).

    - If alignment achieves after step-3, RAN4 will trigger evaluations on the CSI reporting requirements for CSI prediction(during/after RAN4#116).

Regarding the updated simulation for AI/ML-based CSI prediction, we conducted the step-1 and step-2 evaluations for the CSI prediction using the updated simulation assumptions(highlighted in yellow in Table 1). The simulation results are shown in Table 2 and Table3.

Table 1: Updated Simulation assumptions for AI/ML based CSI prediction

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD, TDD, OFDM |
| Carrier frequency | 2GHz for FDD 4GHz for TDD |
| Subcarrier spacing | 15kHz for FDD, 30kHz for TDD |
| Bandwidth | 10MHz for FDD, 40MHz for TDD |
| Symbol | 1 symbol |
| Number of subbands | 13 subbands, 4RB as a bundle for FDD 14 subbands, 8RB as a bundle for TDD |
| gNB TX antennas | Option1:32(N1,N2)=(4,4) as defined in Rel-15 type1 Option2:16(N1,N2)=(4,2) as defined in Rel-18 CSI prediction requirements |
| UE RX antennas | 4 |
| Channel model | TDLA30 with XP medium correlation as baseline |
| Doppler spread | 20Hz, 50Hz and 100Hz |
| Channel estimation for CSI prediction | Step-1: - Ideal DL channel estimation, for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.). Step-2: - Ideal DL channel estimation, for the purpose of calibration and/or comparing intermediate results (e.g., to check SGCS after CSI feedback). Step-3: - Practical DL channel estimation, use a common CSI-RS configuration as in demodulation test (may need to define/choose a CSI-RS configuration) |
| CSI-RS configuration | FFS: following CSI-RS configuration: - For 32Tx Number of CSI-RS ports (X); 32. CDM Type; CDM4 (FD2, TD2). Density (ρ); 1. First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3); (2, 4, 6, 8). First OFDM symbol in the PRB used for CSI-RS (l0, l1); (5, 12). - For 16Tx Number of CSI-RS ports (X); 16. CDM Type; CDM4 (FD2, TD2). Density (ρ); 1. First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3); (2, 4, 6, 8). First OFDM symbol in the PRB used for CSI-RS (l0); (5). |
| Rank per UE | Step-1: Rank 1 and 2 to do quick check Step-2 and Step-3: Rank 2 as baseline |
| CSI feedback assumption | Step1: check the CSI prediction (outputs Raw channel) performance, do not need to take CSI feedback into account |
|  | Step-2 and Step-3: take CSI feedback into account Use Rel-18 eTypeII-Doppler, N4 = 1， N4>1 can be further discussed in the future if needed |
| Test setup(timeline) | Only for step-3 FFS: following test setup (timeline): For FDD test setup with Periodic CSI-RS configuration - CSI-RS periodicity: 5 ms - CSI feedback periodicity: 5 ms - Scheduling delay (from CSI feedback to apply in scheduling): 4 slots Note: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation using a CSI-RS resource set in which the last CSI-RS resource is transmitted at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4) - Test setup for FDD test setup with Periodic CSI-RS configuration is captured in Table2 For TDD test setup with Periodic CSI-RS configuration - CSI-RS periodicity: 5 ms - CSI feedback periodicity: 5 ms - Scheduling delay (from CSI feedback to apply in scheduling): 6 slots Note : If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation using a CSI-RS resource set in which the last CSI-RS resource is transmitted at a downlink slot not later than slot#(n-6), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). - Test setup for TDD test setup with Periodic CSI-RS configuration is captured in Table3 FFS test setup for aperiodic CSI-RS configuration |
| SNR assumption | Only for step-3 [FFS, e.g., Option1: claimed by companies for step-2 first round simulation(TP as KPI) Option2: Example: [-6:2:10] dB Option3: other suggestions ] |
| CSI feedback overhead | PC7 as baseline |
| Baseline | FFS: Random PMI with Rel-15 Type I single panel codebook if needed (e.g., for Step-3 or performance requirements) |
| KPI | Step-1: check SGCS before CSI feedback (between the raw channel matrix and ground-truth at UE) - Average of SGCS for intermediate results over all subbands per layer (e.g. for rank 2), - comparing the SVD of model output with SVD of the ground truth Raw channel Step-2: - check SGCS after CSI feedback Step-3: - check TPs after CSI feedback |
| Model input type | Raw channel matrix |
| Model out type | Raw channel matrix |
| Observation window | Observation window (number/distance): 5/5ms as baseline Optional for 10/5ms |
| Prediction window | Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance): 1/5ms/5ms |
| Training data and test data | Use following combinations: Training dataset (TDLA30); Test on (TDLA30). 20Hz Doppler spreads; 20Hz Doppler spreads. 50Hz Doppler spreads; 50Hz Doppler spreads. 100Hz Doppler spreads; 100Hz Doppler spreads. Mixed Doppler spreads (20Hz, 50Hz, 100Hz); 20Hz Doppler spreads. 50Hz Doppler spreads. 100Hz Doppler spreads. |

Table 2. Simulation results(step-1) for AIML based CSI prediction

|  | TX | Train | 20Hz |  |  |  |  | 50Hz |  |  |  |  | 100Hz |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Rank1 | Rank2-ave L1&2 | Rank2-L1 | Rank2-L2 | Non-AI (Rank1) | Rank1 | Rank2-ave L1&2 | Rank2-L1 | Rank2-L2 | Non-AI (Rank1) | Rank1 | Rank2-ave L1&2 | Rank2-L1 | Rank2-L2 | Non-AI (Rank1) |
| SGCS1 | 32 | Non-mix | 0.9994 | 0.9992 | 0.9994 | 0.9990 | 0.7186 | 0.9032 | 0.8632 | 0.9032 | 0.8231 | 0.3698 | 0.2846 | 0.2289 | 0.2846 | 0.1731 | 0.2850 |
|  |  | Mix | 0.9989 | 0.9983 | 0.9989 | 0.9977 |  | 0.9017 | 0.8617 | 0.9017 | 0.8216 |  | 0.2774 | 0.2216 | 0.2774 | 0.1659 |  |

Note: SGCS1 is calculated by comparing the SVD of model output with SVD of the ground truth Raw channel

Table 3. Simulation results(step-2) for AIML based CSI prediction

|  | TX | Train | 20Hz |  |  |  |  |  | 50Hz |  |  |  |  |  | 100Hz |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Rank1 | Rank2-ave L1&2 | Rank2-L1 | Rank2-L2 | Non-AI (Rank2-L1) | Non-AI (Rank2-L2) | Rank1 | Rank2-ave L1&2 | Rank2-L1 | Rank2-L2 | Non-AI (Rank2-L1) | Non-AI (Rank2-L2) | Rank1 | Rank2-ave L1&2 | Rank2-L1 | Rank2-L2 | Non-AI (Rank2-L1) | Non-AI (Rank2-L2) |
| SGCS2-1 | 32 | Non-mix | - | 0.5820 | 0.6673 | 0.4967 | 0.5819 | 0.3832 | - | 0.5519 | 0.6470 | 0.4568 | 0.3894 | 0.2267 |  | 0.2696 | 0.3291 | 0.2101 | 0.3257 | 0.2100 |
|  |  | Mix | - | 0.5817 | 0.6668 | 0.4966 |  |  | - | 0.5511 | 0.6463 | 0.4558 |  |  |  | 0.2677 | 0.3271 | 0.2083 |  |  |
| SGCS2-2 | 32 | Non-mix |  | 0.9735 | 0.9809 | 0.9660 | 0.7272 | 0.5533 |  | 0.8105 | 0.8614 | 0.7595 | 0.4706 | 0.3147 |  | 0.3424 | 0.3982 | 0.2865 | 0.3945 | 0.2875 |
|  |  | Mix |  | 0.9660 | 0.9744 | 0.9576 |  |  |  | 0.8077 | 0.8597 | 0.7556 |  |  |  | 0.3398 | 0.3959 | 0.2837 |  |  |

Note: 	SGCS2-1 is calculated by comparing the feedbacked SVD of model output with the SVD of the ground truth Raw channel

SGCS2-2 is calculated by comparing the feedbacked SVD of model output with the feedbacked SVD of the ground truth Raw channel

After accounting for CSI feedback, for TDL channels, the SGCS2-1 (calculated by comparing the feedbacked SVD of the model output with the SVD of the ground truth raw channel) deteriorates significantly. This may due to the codebook-based CSI feedback inherently introduces quantization errors compared to the SVD eigenvectors from the ground truth channel. However, when the ground truth eigenvectors also incorporate the quantization errors from CSI feedback, the SGCS2-2 (calculated by comparing the feedbacked SVD of the model output with the feedbacked SVD of the ground truth raw channel) shows much less degradation.

Observation 1: Regarding the updated simulation for AI/ML-based CSI prediction, the updated simulation results are shown in Table 2 and Table3.

Observation 2: For TDL channels, the SGCS calculated by comparing the feedbacked SVD of the model output with the SVD of the ground truth raw channel deteriorates significantly. When the ground truth eigenvectors also incorporate the quantization errors from CSI feedback, the SGCS calculated by comparing the feedbacked SVD of the model output with the feedbacked SVD of the ground truth raw channel shows much less degradation.

For the training and test data consistency, following agreements have been made in RAN4#114 meeting:

Agreement:

- further discuss the following options

  - Option 1: Consistency between training data and test data should be ensured

    - RAN4 to study how to ensure consistency between training data(simulation) and test data (conformance test) and whether anything special should be done during the test

  - Option 2: Consistency will be implicit as test emulated channel will be very similar to the one used in RAN4 simulations

According to our understanding, the consistency between training data and test data should be ensured. The only difference between option1 and option2 is: Option 1 aims to determine whether additional information/indication is needed to explicitly guarantee the consistency. Option 2 assumes that the current test environment is sufficiently stable and capable of reproducing test results. Thus, for both of them, ensuring consistency between training and test data is a prerequisite. The unclear parts are 1) whether the test emulated channel is reproducible, 2) whether additional information/specifications are required,.

Proposal 3: Consistency between training data and test data should be ensured

    - FFS: Whether the test emulated channel will be similar to the one used in RAN4 simulations

    - FFS: Whether anything special should be done during the test

- Conclusions

In this contribution, we have discussed the demodulation and/or CSI reporting requirements for CSI prediction, and got the following proposals:

Proposal 1: Regarding the RAN4 Simulation for CSI prediction and the Reference model for CSI prediction, RAN4 should first assess the CSI prediction evaluation results across different companies before determining whether a reference model is necessary.

Proposal 2: Regarding the RAN4 Simulation for CSI prediction and the Reference model for CSI prediction,

    - If RAN4 achieves aligned CSI prediction evaluation results in step-2 (as described in the agreed simulation assumptions), further evaluation (step-3 comparisons) can proceed (after RAN4#115).

    - If alignment in step-2 proves difficult, RAN4 should identify the key factors causing discrepancies and may need to initiate a second-round evaluation to reassess Step-2 alignment (after RAN4#115).

    - If alignment remains unachievable after step-3, RAN4 may then trigger discussions and evaluations on a reference model for CSI prediction (during/after RAN4#116).

    - If alignment achieves after step-3, RAN4 will trigger evaluations on the CSI reporting requirements for CSI prediction(during/after RAN4#116).

Observation 1: Regarding the updated simulation for AI/ML-based CSI prediction, the updated simulation results are shown in Table 2 and Table3.

Table 2. Simulation results(step-1) for AIML based CSI prediction

|  | TX | Train | 20Hz |  |  |  |  | 50Hz |  |  |  |  | 100Hz |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Rank1 | Rank2-ave L1&2 | Rank2-L1 | Rank2-L2 | Non-AI (Rank1) | Rank1 | Rank2-ave L1&2 | Rank2-L1 | Rank2-L2 | Non-AI (Rank1) | Rank1 | Rank2-ave L1&2 | Rank2-L1 | Rank2-L2 | Non-AI (Rank1) |
| SGCS1 | 32 | Non-mix | 0.9994 | 0.9992 | 0.9994 | 0.9990 | 0.7186 | 0.9032 | 0.8632 | 0.9032 | 0.8231 | 0.3698 | 0.2846 | 0.2289 | 0.2846 | 0.1731 | 0.2850 |
|  |  | Mix | 0.9989 | 0.9983 | 0.9989 | 0.9977 |  | 0.9017 | 0.8617 | 0.9017 | 0.8216 |  | 0.2774 | 0.2216 | 0.2774 | 0.1659 |  |

Note: SGCS1 is calculated by comparing the SVD of model output with SVD of the ground truth Raw channel

Table 3. Simulation results(step-2) for AIML based CSI prediction

|  | TX | Train | 20Hz |  |  |  |  |  | 50Hz |  |  |  |  |  | 100Hz |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Rank1 | Rank2-ave L1&2 | Rank2-L1 | Rank2-L2 | Non-AI (Rank2-L1) | Non-AI (Rank2-L2) | Rank1 | Rank2-ave L1&2 | Rank2-L1 | Rank2-L2 | Non-AI (Rank2-L1) | Non-AI (Rank2-L2) | Rank1 | Rank2-ave L1&2 | Rank2-L1 | Rank2-L2 | Non-AI (Rank2-L1) | Non-AI (Rank2-L2) |
| SGCS2-1 | 32 | Non-mix | - | 0.5820 | 0.6673 | 0.4967 | 0.5819 | 0.3832 | - | 0.5519 | 0.6470 | 0.4568 | 0.3894 | 0.2267 |  | 0.2696 | 0.3291 | 0.2101 | 0.3257 | 0.2100 |
|  |  | Mix | - | 0.5817 | 0.6668 | 0.4966 |  |  | - | 0.5511 | 0.6463 | 0.4558 |  |  |  | 0.2677 | 0.3271 | 0.2083 |  |  |
| SGCS2-2 | 32 | Non-mix |  | 0.9735 | 0.9809 | 0.9660 | 0.7272 | 0.5533 |  | 0.8105 | 0.8614 | 0.7595 | 0.4706 | 0.3147 |  | 0.3424 | 0.3982 | 0.2865 | 0.3945 | 0.2875 |
|  |  | Mix |  | 0.9660 | 0.9744 | 0.9576 |  |  |  | 0.8077 | 0.8597 | 0.7556 |  |  |  | 0.3398 | 0.3959 | 0.2837 |  |  |

Note:

SGCS2-1 is calculated by comparing the feedbacked SVD of model output with the SVD of the ground truth Raw channel

SGCS2-2 is calculated by comparing the feedbacked SVD of model output with the feedbacked SVD of the ground truth Raw channel

Observation 2: For TDL channels, the SGCS calculated by comparing the feedbacked SVD of the model output with the SVD of the ground truth raw channel deteriorates significantly. When the ground truth eigenvectors also incorporate the quantization errors from CSI feedback, the SGCS calculated by comparing the feedbacked SVD of the model output with the feedbacked SVD of the ground truth raw channel shows much less degradation.

Proposal 3: Consistency between training data and test data should be ensured

    - FFS: Whether the test emulated channel will be similar to the one used in RAN4 simulations

    - FFS: Whether anything special should be done during the test

- References

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-250792, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface